EIS Market Integrated Marketplace

Protocols Revision Request

| PRR No. | Marketplace-PRR36 | PRR Title | Clarification of Unit Commitment Instructions and Postings |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision |  | Section No.: 4.3.1.3, 4.3.2, 4.3.2.3, 4.4.1.3 Title: DA Market Results, Day-Ahead Reliability Unit Commitment, Day-Ahead RUC Results, Intra-Day RUC Results Protocol Version: 2.0 |  |
| Impact Analysis Required |  | Yes – If yes, estimated cost: No SPP Staff will complete this section. |  |
| Requested Resolution |  | Normal Expedited Urgent Action Provide explanation if Expedited and/or Urgent Action is selected: |  |
| Revision Description |  | This MPRR clarifies unit commitment instructions and postings for Market Participants for the DA, RUC, and RTBM. |  |
| Reason for Revision |  | This language is intended to provide clarifications to the commitment instructions from SPP and the expectations of the start-up and shut-down orders from SPP to Market Participants. |  |
| Tariff Implications or Changes |  | Yes – Section No.: (Include a summary of impact and/or specific changes) No |  |
| Criteria Implications or Changes |  | Yes - Section No.: (Include a summary of impact and/or specific changes) No |  |
| Credit Implications |  | Yes (Include a summary of impact and/or specific changes) No |  |
| Date |  | 9/6/2011 |  |
| Sponsor |  |  |  |
| Name |  | Jodi Woods |  |
| E-mail Address |  | jwoods@spp.org |  |
| Company |  | Southwest Power Pool |  |
| Phone Number |  | 501.614.3282 |  |

Proposed Protocol Language Revision

#### 4.3.1.3	DA Market Results

No later than 1600 hours Day-Ahead, SPP electronically communicates the DA Market results for each hour of the Operating Day to Market Participants.  The following results are communicated to each Market Participant that relates only to that Market Participant:

- Cleared Resource Offers for Energy, Regulation-Up, Regulation-Down, Spinning Reserve and/or Supplemental Reserve, in MW;

  - Cleared Offers for Energy associated with Resource Offers represent a physical Resource commitment schedule that forms the basis for the Current Operating Plan for the upcoming Operating Day.  Market Participants should consider Resource commitment schedules resulting from SPP commitment of Resources with a Commit Status of “Market” or “Reliability as SPP start-up orders and shut-down orders.

  - Resources committed by SPP in the DA Market with a Commit Status of “Market” or “Reliability” are guaranteed to receive DA Market revenues that are at least equal to the DA Market Resource Offer costs for the associated cleared amount of Energy, Regulation-Up, Regulation-Down Spinning Reserve and/or Supplemental Reserve.  See Section 4.5.8.12 for additional details.

### 4.3.2	Day-Ahead Reliability Unit Commitment

At 1700 hours or one hour following the posting of the DA Market results, whichever is later, SPP begins the Day-Ahead RUC process to assess capacity adequacy during the Operating Day.  No later than 2000 hours or three hours following the start of the DA RUC process, whichever is later, SPP communicates certain results to affected Market Participants.  SPP then updates the Current Operating Plan and start-up and/or shutdown orders are issued which may be anytime after 2000 hours.

The Day-Ahead RUC consists of four steps: (1) process RUC inputs; (2) execute RUC algorithm; (3) evaluate and communicate RUC results; and (4) update Current Operating Plan as needed.

      - 4.3.2.3

#### Day-Ahead RUC Results

No later than 2000 hours Day-Ahead, SPP electronically communicates the following Day-Ahead RUC results for each hour of the Operating Day to Market Participants:

- Each Market Participant is notified regarding its Resources that are expected to be dispatched to Maximum Emergency Capacity Operating Limits.  This notification is for information purposes only and will not become effective until confirmed by SPP prior to the affected Operating Hour.  Market Participants will be notified at least 10 minutes prior to the beginning of the Operating Hour but not more than 30 minutes prior to the beginning of the Operating Hour that the Maximum Emergency Capacity Operating Limit will be used;

- Each Market Participant is notified regarding its Resources that are expected to be dispatched to Minimum Emergency Capacity Operating Limit.  This notification is for information purposes only and will not become effective until confirmed by SPP prior to the affected Operating Hour.  Market Participants will be notified at least 10 minutes prior to the beginning of the Operating Hour but not more than 30 minutes prior to the beginning of the Operating Hour that the Minimum Emergency Capacity Operating Limit will be used;

- Notification that a fixed Interchange Transaction is expected to be curtailed due to excess or shortage conditions;

#### 4.3.2.4	Update Current Operating Plan

  - (1)

- Using the results from the Day-Ahead RUC analysis, SPP may issue start-up orders to Resources with a Commit Status of “Market” or “Reliability” at any time following 2000 hours.   When SPP updates the Current Operating Plan, these start-up orders are issued.

  - Resources that incur one or more start-up costs within the Operating Day as a result of

SPP start-up orders are guaranteed to receive revenues that are at least equal to the RTBM Resource Offer costs for actual Energy supplied and, Regulation, Spinning Reserve and/or Supplemental Reserve cleared in the RTBM over the commitment period, subject to eligibility criteria, as described in Section 4.5.9.8.

- Using the results from the Day-Ahead RUC analysis, SPP may issue shut-down orders to SPP committed Resources that were issued start-up orders in (1) above or may issue  changes to a shut-down order for SPP committed Resources in the DA Market at any time following 2000 hours. When SPP updates the Current Operating Plan, these shut-down orders are issued.

  - SPP may only issue changes to shut-down orders issued as part of the DA Market results to address an anticipated excess supply condition as described under Section 4.3.2.2(3)(b) and/or to address any other Emergency conditions.   To the extent that a change in a shut-down order is issued by SPP to a Resource that was committed by SPP in the DA Market causes that Resource to buy back its Energy and/or Operating Reserve position at RTBM prices that exceed the DA Market prices for the comparable products, that Resource is eligible for compensation under Section 4.5.9.9.

### 4.4.1	Intra-Day Reliability Unit Commitment

Following completion of the Day-Ahead RUC process, SPP continually evaluates the need for an Intra-Day RUC for the remainder of the Day-Ahead period and the Operating Day and performs additional Intra-Day RUCs at least every four hours.  Consistent with the Day-Ahead RUC, these additional Intra-Day RUCs assess capacity adequacy during the Operating Day.

The Intra-Day RUC consists of four steps: (1) process RUC inputs; (2) execute RUC algorithm; (3) evaluate and communicate RUC results; and (4) and update Current Operating Plan as needed.

      - 4.4.1.3

      - IntraIntra-Day RUC Results

SPP electronically communicates the RUC results for each hour of the Operating Day to Market Participants as soon as practical following completion of each Intra-Day RUC execution.  These results consist of the following:

(3)	Each Market Participant is notified regarding its Resources that are expected to be dispatched to Maximum Emergency Capacity Operating Limits.  This notification is for information purposes only and will not become effective until confirmed by SPP prior to the affected Operating Hour.  Market Participants will be notified at least 10 minutes prior to the beginning of the Operating Hour but not more than 30 minutes prior to the beginning of the Operating Hour that the Maximum Emergency Capacity Operating Limit will be used;

(4)	Each Market Participant is notified regarding its Resources that are expected to be dispatched to Minimum Emergency Capacity Operating Limit.  This notification is for information purposes only and will not become effective until confirmed by SPP prior to the affected Operating Hour.  Market Participants will be notified at least 10 minutes prior to the beginning of the Operating Hour but not more than 30 minutes prior to the beginning of the Operating Hour that the Minimum Emergency Capacity Operating Limit will be used;

(5)	Notification that a fixed Interchange Transaction is expected to be curtailed due to excess or shortage conditions;

#### 4.4.1.4	Update Current Operating Plan

(1)	Using the results from the Intra-Day RUC analysis, SPP may issue start-up orders to Resources with a Commit Status of “Market” or “Reliability” at any time following Intra-Day RUC analysis.   When SPP updates the Current Operating Plan, these start-up orders are issued.

(a)	Resources that incur one or more start-up costs within the Operating Day as a result of SPP start-up orders are guaranteed to receive revenues that are at least equal to the RTBM Resource Offer costs for actual Energy supplied and, Regulation, Spinning Reserve and/or Supplemental Reserve cleared in the RTBM over the commitment period, subject to eligibility criteria, as described in Section 4.5.9.8.

(2)	Using the results from the Intra-Day RUC analysis, SPP may issue shut-down orders to SPP committed Resources that were issued start-up orders in (1) above or may issue  changes to a shut-down order for SPP committed Resources in the DA Market at any time following Intra-Day RUC analysis. When SPP updates the Current Operating Plan, these shut-down orders are issued.

(a)	SPP may only issue changes to shut-down orders issued as part of the DA Market results to address an anticipated excess supply condition as described under Section 4.3.2.2(3)(b) and/or to address any other Emergency conditions.   To the extent that a change in a shut-down order is issued by SPP to a Resource that was committed by SPP in the DA Market causes that Resource to buy back its Energy and/or Operating Reserve position at RTBM prices that exceed the DA Market prices for the comparable products, that Resource is eligible for compensation under Section 4.5.9.9.

Proposed Tariff Language Revision

N/A

Proposed Criteria Language Revision

N/A