EIS Market Integrated Marketplace

PRR Recommendation Report

| PRR No. | Marketplace-PRR36 | PRR Title | Clarification of Unit Commitment Instructions and Postings |
| --- | --- | --- | --- |
| Timeline |  | Normal Expedited Urgent Action Provide explanation if Expedited and/or Urgent Action is selected: MWG motioned on September 19, 2011 to treat all of the open MPRRs as Expedited and waive the remainder of the comment period. |  |
| Recommendation Action |  | Approve Reject Require additional information Defer Refer |  |
| Impact Analysis Required |  | Yes – If yes, estimated cost: No SPP Staff will complete this section. |  |
| Protocol Section(s) Requiring Revision |  | Section No.: 4.3.1.3, 4.3.2, 4.3.2.3, 4.4.1.3, 4.4.1.4 (new section) Title: DA Market Results, Day-Ahead Reliability Unit Commitment, Day-Ahead RUC Results, Intra-Day RUC Results, Update Current Operating Plan Protocol Version: 2.0 |  |
| Type of Revision |  | Correction/Clean-Up Clarification Design Enhancement Design Change |  |
| Revision Description |  | This MPRR clarifies unit commitment instructions and postings for Market Participants for the DA, RUC, and RTBM. |  |
| Tariff Implications or Changes |  | Yes – Section No: (Include a summary of impact and/or specific changes) No |  |
| MWG Review |  | Date of Vote: 9/30/2011—Unanimously approved All Segments present for the vote: Yes No Segment of Parties that voted No or Abstained: None |  |
| RTWG Review |  | RTWG is drafting Tariff language for Integrated Marketplace and is aware of MPRR 26 |  |
| ORWG Review |  | 10/27/2011—Approved |  |
| MOPC Review |  | 10/11/2011—Approved contingent upon ORWG approval |  |
| Board Review |  | N/A |  |

| Date |  | 9/6/2011 |
| --- | --- | --- |
| Sponsor |  |  |
| Name |  | Jodi Woods |
| E-mail Address |  | jwoods@spp.org |
| Company |  | Southwest Power Pool |
| Phone Number |  | 501.614.3282 |
| Comments Received |  |  |
| Comment Author | Carrie Simpson, SPP |  |
| Date | 9/15/2011 |  |
| Comment Description | These are comments are intended to clarify that the Reliability Unit Commitment algorithm is an advisory tool for SPP Operators and that Emergency Procedures will be performed in accordance with the Tariff and NERC standards. |  |
| Comment Status | The language is reflected in this recommendation report. |  |
| Comments Received |  |  |
| Comment Author | Jodi Woods, SPP |  |
| Date | 9/27/2011 |  |
| Comment Description | These comments provide clarifying language describing what results will be posted and how the results will be posted for both DA RUC and Intra-Day RUC studies. |  |
| Comment Status | The language is reflected in this recommendation report. |  |
| Comments Received |  |  |
| Comment Author | Carrie Simpson/Jodi Woods, SPP |  |
| Date | 9/28/2011 |  |
| Comment Description | These comments include changes recommended from the September 16th and 27th MWG conference calls. |  |
| Comment Status | The language is reflected in this recommendation report. |  |
| Comments Received |  |  |
| Comment Author | MWG |  |
| Date | 9/30/2011 |  |
| Comment Description | Clarification |  |
| Comment Status | The MPRR was approved as modified. The approved language is reflected in this recommendation report. |  |

Proposed Protocol Language Revision

#### 4.3.1.3	DA Market Results

No later than 1600 hours Day-Ahead, SPP electronically communicates the DA Market results for each hour of the Operating Day to Market Participants.  The following results are communicated to each Market Participant that relates only to that Market Participant:

- Cleared Resource Offers for Energy, Regulation-Up, Regulation-Down, Spinning Reserve and/or Supplemental Reserve, in MW;

  - Cleared Offers for Energy associated with Resource Offers represent a physical Resource commitment schedule that forms the basis for the Current Operating Plan for the upcoming Operating Day.  Market Participants should consider Resource commitment schedules resulting from SPP commitment of Resources with a Commit Status of “Market” or “Reliability as SPP start-up orders and shut-down orders.

  - Resources committed by SPP in the DA Market with a Commit Status of “Market” or “Reliability” are guaranteed to receive DA Market revenues that are at least equal to the DA Market Resource Offer costs for the associated cleared amount of Energy, Regulation-Up, Regulation-Down Spinning Reserve and/or Supplemental Reserve.  See Section 4.5.8.12 for additional details.

### 4.3.2	Day-Ahead Reliability Unit Commitment

At 1700 hours or one hour following the posting of the DA Market results, whichever is later, SPP begins the Day-Ahead RUC process to assess capacity adequacy during the Operating Day.  No later than 2000 hours or three hours following the start of the DA RUC process, whichever is later, SPP shall communicate the results described under Section 4.3.2.3.  SPP then updates the Current Operating Plan, if needed, and start-up and/or shutdown orders are issued simultaneously which may be anytime after 2000 hours.

The Day-Ahead RUC consists of four steps: (1) process RUC inputs; (2) execute RUC algorithm; (3) evaluate and communicate RUC results; and (4) update Current Operating Plan as needed.

      - 4.3.2.3

#### Day-Ahead RUC Results

No later than 2000 hours Day-Ahead, SPP electronically communicates the following Day-Ahead RUC results for each hour of the Operating Day to Market Participants:

- For any future hours in which SPP anticipates  an Emergency situation, SPP shall notify the Market Participants identifying the hours in which the emergency ranges of any Resources are expected to be required, the hours in which Resources with a Commit Status of Reliability are expected to be committed, the hours in which non-firm fixed Export Interchange Transactions are expected to be curtailed and the hours in which non-firm fixed Import Interchange Transactions are expected to be curtailed.

(a)	  In addition if necessary, affected Market Participants will be notified at least 10 minutes prior to the beginning of the Operating Hour but not more than 30 minutes prior to the beginning of the Operating Hour that the Maximum Emergency Capacity Operating Limit will be used;

(b)	In addition if necessary, affected Market Participants will be notified at least 10 minutes prior to the beginning of the Operating Hour but not more than 30 minutes prior to the beginning of the Operating Hour that the Minimum Emergency Capacity Operating Limit will be used;

#### 4.3.2.4	Update Current Operating Plan

Using the results from the Day-Ahead RUC analysis, SPP will update the Current Operating Plan and will issue start-up and shut-down orders as appropriate.  SPP can only de-commit DA Market committed Resources to address an anticipated excess supply condition as described under Section (3)(b) and/or to address any other Emergency conditions.

- If SPP de-commits an SPP committed Resource for any hour of the DA Market commitment schedule,  and causes that Resource to buy back its Energy and/or Operating Reserve position at RTBM prices that exceed the DA Market prices for the comparable products, that Resource is eligible for compensation under Section 4.5.9.9.

4.4.1	Intra-Day Reliability Unit Commitment

Following completion of the Day-Ahead RUC process, SPP continually evaluates the need for an Intra-Day RUC for the remainder of the Day-Ahead period and the Operating Day and performs additional Intra-Day RUCs at least every four hours.  Consistent with the Day-Ahead RUC, these additional Intra-Day RUCs assess capacity adequacy during the Operating Day.  As soon as practicable following completion of an Intra-Day RUC study, SPP will communicate the results described under Section 4.4.1.3.  SPP then updates the Current Operating Plan, if needed, and start-up and/or shutdown orders are issued simultaneously which may be anytime after completion of an Intra-Day RUC study.

The Intra-Day RUC consists of four steps: (1) process RUC inputs; (2) execute RUC algorithm; (3) evaluate and communicate RUC results; and (4) and update Current Operating Plan as needed.

      - 4.4.1.3

      - IntraIntra-Day RUC Results

SPP electronically communicates the RUC results for each hour of the Operating Day to Market Participants as soon as practical following completion of each Intra-Day RUC execution.  These results consist of the following:

(1)		For any future hours in which SPP anticipates  an Emergency situation, SPP shall notify the market identifying the hours in which the emergency ranges of any Resources are expected to be required, the hours in which Resources with a Commit Status of Reliability are expected to be committed, the hours in which non-firm fixed Export Interchange Transactions are expected to be curtailed and the hours in which non-firm fixed Import Interchange Transactions are expected to be curtailed.

(a)	In addition if necessary, affected Market Participants will be notified at least 10 minutes prior to the beginning of the Operating Hour but not more than 30 minutes prior to the beginning of the Operating Hour that the Maximum Emergency Capacity Operating Limit will be used;

(b)	In addition if necessary, affected Market Participants will be notified at least 10 minutes prior to the beginning of the Operating Hour but not more than 30 minutes prior to the beginning of the Operating Hour that the Minimum Emergency Capacity Operating Limit will be used.

#### 4.4.1.4	Update Current Operating Plan

(1)	Using the results from the Intra-Day RUC analysis, SPP will update the Current Operating Plan and will issue start-up and shut-down orders as appropriate.

(a)	SPP may only issue changes to shut-down orders issued as part of the DA Market results to address an anticipated excess supply condition as described under Section 4.3.2.2(3)(b) and/or to address any other Emergency conditions.   If SPP de-commits an SPP committed Resource for any hour of the DA Market commitment schedule,  and causes that Resource to buy back its Energy and/or Operating Reserve position at RTBM prices that exceed the DA Market prices for the comparable products, that Resource is eligible for compensation under Section 4.5.9.9.

Proposed Tariff Language Revision

N/A

Proposed Criteria Language Revision

N/A