EIS Market Integrated Marketplace

Protocols Revision Request

| PRR No. | Marketplace-PRR49 | PRR Title | RUC MWP Changes |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision |  | Section No.: 4.5.9.8, 4.5.9.9, 4.5.9.10 Title: RUC Make-Whole-Payment, Real-Time Out-Of-Merit Amount & RUC Make-Whole-Payment Distribution Amount Protocol Version: 6.0 |  |
| Impact Analysis Required |  | Yes – If yes, estimated cost: No SPP Staff will complete this section. |  |
| Requested Resolution |  | Normal Expedited Urgent Action Provide explanation if Expedited and/or Urgent Action is selected: This MPRR is being expedited because it is a design decision that is preventing Alstom Settlements from moving forward in the build phase. |  |
| Type of Revision |  | Correction/Clean-Up Clarification Design Enhancement Design Change |  |
| Revision Description |  | Three of the “deviations” upon which the cost allocation to a Resource is based are currently a function of the Setpoint Instruction. The idea is to isolate intervals in which the Resource is either injecting more or less than the market economics would desire in a situation where the Resource’s offer has been modified such as to limit the market’s ability to fully control the Resource. This approach is subject to inadequacies around operating in manual status or providing regulation. A better method would be to use another existing settlement determinant, the Desired Dispatch Quantity, as a measure of whether the Resource is operating within an economic range. |  |
| Reason for Revision |  | Improve the settlement of RUC MWP Cost Allocation to eliminate avoidance of or undue exposure to RUC MWP Cost Allocation. |  |
| Tariff Implications or Changes |  | Yes – Section No.: (Include a summary of impact and/or specific changes) No |  |
| Criteria Implications or Changes |  | Yes - Section No.: (Include a summary of impact and/or specific changes) No |  |
| Credit Implications |  | Yes (Include a summary of impact and/or specific changes) No |  |
| Date |  | 1/10/2012 |  |
| Sponsor |  |  |  |
| Name |  | Tony Alexander |  |
| E-mail Address |  | talexander@spp.org |  |
| Company |  | Southwest Power Pool |  |
| Phone Number |  | 501. 614.3303 |  |

Proposed Protocol Language Revision

#### RUC Make-Whole-Payment Distribution Amount

(4) The amount to each Asset Owner (AO) for each eligible Resource Settlement Location for a given RUC Make-Whole Payment Eligibility Period is calculated as follows:

#RtMwpCpAmt a, s, c =  ( CncldStartAmt a, s, c

+ Max (0, ( { ( IF ( CncldStartRatio a, s, c = 0, THEN 1, ELSE 0) }

* ( { RtStartUpElig5minFlg a, s, i, c  * RtStartUp5minAmt a, s, i, c

+ RtRucComStat5minFlg a, s, i, c * [ RtMwpCost5minAmt a, s, i, c

+ RtMwpRev5minAmt a, s, i, c

+ RtOom5minAmt a, s, i + RtRegAdj5minAmt a, s, i

–  RtURDAdj5minAmt a, s, i, c – RtStatusAdj5minAmt a, s, i, c

–  RtLimitAdj5minAmt a, s, i, c  ] } ) ) ) * (-1)

Where,

(a)	#RtMwpCost5minAmt a, s, i, c =  RtRucComStat5minFlg a, s, i, c  *

( RtIncrEn5minAmt a, s, i

+  Max ( 0, [ RtNoLoad5minAmt a, s, i, c

- IF (DaClrdHrlyQty a, s, h < 0, THEN DaNoLoadHrlyAmt a, s, h, c , ELSE 0 ) ] )

+ RtMinEn5minAmt a, s, i, c

+  RtRegUpAvail5minAmt a, s, i, c +  RtRegDnAvail5minAmt a, s, i, c

+  RtSpinAvail5minAmt a, s, i, c +  RtSuppAvail5minAmt a, s, i, c ) / 12

(a.1)	IF ABS (DaClrdHrlyQty a, s, h ) > = ABS ( RtBillMtr5minQty a, s, i )

THEN

RtIncrEn5minAmt a, s, i = 0

ELSE IF

ControlStatus5minFlg a, s, i  = “Regulating”

#RtIncrEn5minAmt a, s, i =

Where:

X = Max (ABS (DaClrdHrlyQty a, s, h , RtComMinRegCapOL5minQtya, s, i )

Y = Max ( X, Max (RtBillMtr5minQtya, s, i , 0) )

ELSE IF

ControlStatus5minFlg a, s, i  < > “Regulating”

#RtIncrEn5minAmt a, s, i =

Where:

X = Max (ABS (DaClrdHrlyQty a, s, h , RtComMinEconCapOL5minQtya, s, i )

Y = Max ( X, Max (RtBillMtr5minQtya, s, i , 0) )

(a.2)	IF ABS (DaClrdHrlyQty a, s, h ) > 0

THEN

RtMinEn5minAmt a, s, i, c = 0

ELSE IF

ControlStatus5minFlg a, s, i  = “Regulating”

# RtMinEn5minAmt a, s, i, c =

ELSE IF

ControlStatus5minFlg a, s, i  < > “Regulating”

# RtMinEn5minAmt a, s, i, c =

(a.3)	If RtRegUp5minQty a, s, i  > RtFixedRegUp5minQty a, s, i

THEN

RtRegUpAvail5minAmt a, s, i, c =

Max ( 0, [ RtRegUp5minQty a, z, s, i   - DaRegUpHrlyQty a, z, s, h] )

* RtRegUpOffer a, s, i, c

ELSE

RtRegUp5minAmt a, s, i  =0

(a.4)	If RtRegDn5minQty a, s, i  > RtFixedRegDn5minQty a, s, i

THEN

RtRegDnAvail5minAmt a, s, i, c =

Max ( 0, [ RtRegDn5minQty a, z, s, i   - DaRegDnHrlyQty a, z, s, h] )

* RtRegDnOffer a, s, i, c

ELSE

RtRegDn5minAmt a, s, i  =0

(a.5)	If RtSpin5minQty a, s, i  > RtFixedSpin5minQty a, s, i

THEN

RtSpinAvail5minAmt a, s, i, c =

Max ( 0, [ RtSpin5minQty a, z, s, i   - DaSpinHrlyQty a, z, s, h] )

* RtSpinOffer a, s, i, c

ELSE

RtSpin5minAmt a, s, i  =0

(a.6)	If RtSupp5minQty a, s, i  > RtFixedSupp5minQty a, s, i

THEN

RtSuppAvail5minAmt a, s, i, c =

Max ( 0, [ RtSupp5minQty a, z, s, i   - DaSuppHrlyQty a, z, s, h] )

* RtSuppOffer a, s, i, c

ELSE

RtSupp5minAmt a, s, i  =0

(b)	#RtMwpRev5minAmt a, s, i, c =

RtRucComStat5minFlg a, s, i, c  * [ ( ( RtLmp5minPrc s, i

* Min (0, [ RtBillMtr5minQty a, s, i - DaClrdHrlyQty a, s, h ] ) ) / 12 )

+ RtRegUpRev5minAmt a, s, i, c + RtRegDnRev5minAmt a, s, i, c

+ RtSpinRev5minAmt a, s, i, c + RtSuppRev5minAmt a, s, i, c ]

(b.1)	RtRegUpRev5minAmt a, s, i, c =

(-1) * RtRucComStat5minFlg a, s, i, c

* ( Max ( 0, [ RtRegUp5minQty a, z, s, i   - DaRegUpHrlyQty a, z, s, h] )

* RtRegUpMcp5minPrc z, i  ) / 12

(b.2)	RtRegDnRev5minAmt a, s, i, c =

(-1) * RtRucComStat5minFlg a, s, i, c

*( Max ( 0, [ RtRegDn5minQty a, z, s, i  - DaRegDnHrlyQty a, z, s, h] )

* RtRegDnMcp5minPrc z, i   ) / 12

(b.3)	RtSpinRev5minAmt a, s, i, c =

(-1) * RtRucComStat5minFlg a, s, i, c

*( Max ( 0, [ RtSpin5minQty a, z, s, i   - DaSpinHrlyQty a, z, s, h ] )

* RtSpinMcp5minPrc z, i ) / 12

(b.4)	RtSuppRev5minAmt a, s, i, c =

(-1) * RtRucComStat5minFlg a, s, i, c

*( Max ( 0, [ RtSupp5minQty a, z, s, i   - DaSuppHrlyQty a, z, s, h ] )

* RtSuppMcp5minPrc z, i   ) / 12

(c)	#CncldStartAmt a, s, c =

( RtStartUp5minAmt a, s, i, c  * RtStartUpElig5minFlg a, s, i, c )

* CncldStartRatio a, s, c

CncldStartRatio a, s, c = (ElapsedTime a, s, c  / StartUpTime a, s, c )

(d) In any Dispatch Interval in which the Resource has operated outside of its Operating Tolerance and that Resource has not been exempted from URD per Section 4.4.4.1, any incremental Energy costs associated with actual Energy output above the Resource’s Desired Dispatch is not eligible for recovery.  The URD adjustment is calculated as follows:

IF ABS (URD5minQty a, s, i ) > ResOpTol5minQty a, s, i AND

( XmptDev5minFlg a, s, i =  0 )

THEN

#RtURDAdj5minAmt a, s, i, c = RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i ) / 12

ELSE

RtURDAdj5minAmt a, s, i, c = 0

(d.1)	URD5minQty a, s, i =

( RtBillMtr5minQty a, s, i * (-1)) - RtAvgSetPoint5minQty a, s, i

(d.2)	ResOpTol5minQty a, s, i =

Min ( URDMaxTol5minQty a, s, i , Max (URDMinTol5minQty a, s, i ,

URDTol5minPct a, s, i * RtDispMaxEmerCapOL5minQty a, s, i ) )

(d.3)	IF RtDesiredEn5minQty a, s, i < ABS (DaClrdHrlyQty a, s, h )

THEN

#RtDesiredEn5minAmt a, s, i = RtIncrEn5minAmt a, s, i

ELSE IF

ControlStatus5minFlg a, s, i  = “Regulating”

#RtDesiredEn5minAmt a, s, i =

Where:

X = Max (ABS (DaClrdHrlyQty a,  s, h ) , RtComMinRegCapOL5minQtya, s, i )

Y = Max ( X, RtDesiredEn5minQtya, s, i )

ELSE IF

ControlStatus5minFlg a, s, i  < > “Regulating”

#RtDesiredEn5minAmt a, s, i =

Where:

X = Max (ABS (DaClrdHrlyQty a, s, h),  RtComMinEconCapOL5minQtya, s, i )

Y = Max ( X, RtDesiredEn5minQtya, s, i  )

(e) In any Dispatch Interval in which a Resource is in “Manual” status, any incremental Energy costs associated with actual Energy output above the Resource’s Desired Dispatch is not eligible for recovery.  The status change adjustment is calculated as follows:

IF ControlStatus5minFlg a, s, i = “Manual”

AND ABS (URD5minQty a, s, i ) <= ResOpTol5minQty a, s, i

THEN

#RtStatusAdj5minAmt a, s, i, c = RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i ) / 12

ELSE

RtStatusAdj5minAmt a, s, i, c = 0

(f)	In any Dispatch Interval in which a Resource has increased its Minimum Economic Capacity Operating Limit (or its Minimum Regulation Capacity Operating Limit if the Resource has cleared for Regulation-Up or Regulation-Down) above the Resource’s minimum limits used by SPP in the commitment decision or the minimum limits used to move from one configuration to another in the case of a Combined Cycle Resource, the Resource is not in “Manual” status and the increase in minimum limit is greater than the Resource’s Operating Tolerance, any incremental Energy costs associated with actual Energy output above the Resource’s Desired Dispatch is not eligible for recovery.  The limit change adjustment is calculated as follows:

IF ControlStatus5minFlg a, s, i < > “Regulating” AND

ControlStatus5minFlg a, s, i  < > “Manual”  AND

( RtDispMinEconCapOL5minQty a, s, i

- RtComMinEconCapOL5minQty a, s, i ) > ResOpTol5minQty a, s, i  AND

ABS (URD5minQty a, s, i ) <= ResOpTol5minQty a, s, i

THEN

#RtLimitAdj5minAmt a, s, i, c =   RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i ) / 12

ELSE IF

ControlStatus5minFlg a, s, i  = “Regulating”  AND

( RtDispMinRegCapOL5minQty a, s, i

- RtComMinRegCapOL5minQty a, s, i ) > ResOpTol5minQty a, s, i  AND

ABS (URD5minQty a, s, i ) < =ResOpTol5minQty a, s, i

THEN

#RtLimitAdj5minAmt a, s, i, c =  RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i ) / 12

ELSE

RtLimitAdj5minAmt a, s, i, c = 0

- For each Asset Owner, a daily amount is calculated at each Settlement Location.  The daily amount is calculated as follows:

RtMwpDlyAmt a, s, d =   RtMwpCpAmt a, s, c

- For each Asset Owner associated with Market Participant m, a daily amount is calculated.  The daily amount is calculated as follows:

RtMwpAoAmt a, m, d =  RtMwpDlyAmt a, s, d

- For each Market Participant, a daily amount is calculated representing the sum of Asset Owner amounts associated with that Market Participant.  The daily amount is calculated as follows:

RtMwpMpAmt m, d =  RtMwpAoAmt a, m, d

(8)	For FERC Electric Quarterly Reporting (“EQR”) purposes, SPP calculates RUC Make-Whole Payment $ per RUC Make-Whole-Payment Eligibility Period for each Asset Owner as follows:

(a)	EqrRtMwp5minPrc a, s, c = (-1) * RtMwpCpAmt a, s, c

(b)	IF EqrRtMwp5minPrc a, s, c > 0

THEN

EqrRtMwp5minQty a, s, c = 1

The above variables are defined as follows:

| Variable | Unit | Settlement Interval | Definition |
| --- | --- | --- | --- |
| RtMwpCpAmt a, s, c | $ | Eligibility Period | RUC Make-Whole-Payment Amount per AO per Settlement Location per RUC Make-Whole-Payment Eligibility Period - The amount to AO a for RUC Make-Whole-Payment Eligibility Period c at Resource Settlement Location s.. |
| DaClrdHrlyQty a, s, h | MWh | Hour | Day-Ahead Cleared Energy Quantity per AO per Settlement Location per Hour - The value described under Section 4.5.8.1 for AO a’s combined cycle resource at Settlement Location s for the Hour. |
| RtStartUp5minAmt a s, i, c | $ | Eligibility Period | Real-Time Start-Up Cost Amount per AO per Settlement Location per Dispatch Interval per RUC Make-Whole-Payment Eligibility Period - The RTBM Start-Up Offer associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. This value is calculated by dividing RtStartUpAmt a s, c by the lesser of the Resource’s (RtMinRunTime a, i, s, c *12) or (24 * 12). These interval values are carried forward into the following Operating Day, if needed, to ensure recovery of any remaining RtStartUpAmt a s, c. |
| RtStartUpAmt a s, c (Not Available on Settlement Statement) | $ | Eligibility Period | Real-Time Start-Up Cost Amount per AO per Settlement Location per RUC Make-Whole-Payment Eligibility Period - The RTBM Start-Up Offer used in the commitment decision, associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c. |
| RtStartUpElig5minFlg a, s, i, c | None | Dispatch Interval | RUC Start-Up Recovery Eligibility Flag per AO per Resource Settlement Location per Dispatch Interval per RUC Make-Whole-Payment Eligibility Period – This flag is set equal to 1 in each Dispatch Interval of a RUC Make-Whole-Payment Eligibility Period where the Resource is eligible to recover start-up costs, or 0 where the Resource is not eligible to recover start-up costs. |
| RtRucComStat5minFlg a, s, i, c | None | Dispatch Interval | RUC Commitment Status Flag per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – This flag is set equal to 1 for each Dispatch Interval of a RUC Make-Whole-Payment Eligibility Period in which a Resource’s Commitment Status was “Market” or “Reliability”, or 0 if its Commitment Status was “Self”. |
| CncldStartRatio a, s, c | None |  | Canceled Start Ratio per Resource Settlement Location in RUC Make-Whole-Payment Eligibility Period – The ratio of ElapsedTime a, s, c to StartUpTime a, s, c as calculated for each Dispatch Interval in RUC Make-Whole-Payment Eligibility Period c. |
| RtMinRunTime a, i, s, c (Not Available on Settlement Statement) | Time | Dispatch Interval | Real-Time Minimum Run Time per AO per Settlement Location Per Dispatch Interval per RUC Make-Whole-Payment Eligibility Period – The Minimum Run Time used in the commitment decision, associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c as submitted as part of the RTBM Market Offer. |
| RtNoLoad5minAmt a, i, s, c | $ | Dispatch Interval | Real-Time No-Load Cost Amount per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period - The No-Load Offer used in the commitment decision, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtMwpCost5minAmt a, s, i, c | $ | Dispatch Interval | RUC Make-Whole-Payment Cost per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period – The total Energy and Operating Reserve cost at actual Resource output, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtMwpRev5minAmt a, s, i, c | $ | Dispatch Interval | RUC Make-Whole-Payment Revenue per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period – The total Energy and Operating Reserve revenue at actual Resource output, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| CncldStartAmt a, s, c | $ | Eligibility Period | Real-Time Cancelled Start Amount per AO per Settlement Location per for the RUC Make-Whole-Payment Eligibility Period – The Start-Up Offer cost reimbursement for an SPP cancelled start-up, in dollars, associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c. |
| ElapsedTime a, s, c | Time | Eligibility Period | Elapsed Time per AO per Settlement Location per for the RUC Make-Whole-Payment Eligibility Period – The elapsed time, in minutes, between the start of a Resource’s StartUpTime a, s, c and the time SPP cancelled the start-up, in dollars, associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c. |
| StartUpTime a, s, c | Time | Eligibility Period | Start-up Time per AO per Settlement Location for the RUC Make-Whole-Payment Eligibility Period – The Start-Up Time, in minutes, used in the commitment decision associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c as specified in the RTBM Offer submitted prior to the RUC Make-Whole-Payment Eligibility Period. |
| RtURDAdj5minAmt a, s, i, c | $ | Dispatch Interval | URD Adjustment per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period – The reduction in RUC Make-Whole Payment Amount associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c when the Resource’s URD5minQty a, s, i is outside of the Resource’s ResOpTol5minQty a, s, i. |
| URD5minQty a, s, i | MW | Dispatch Interval | Uninstructed Resource Deviation per AO per Settlement Location per Dispatch Interval – The Uninstructed Resource Deviation associated with AO a’s Resource at Settlement Location s in Dispatch Interval i. |
| ResOpTol5minQty a, s, i | MW | Dispatch Interval | Resource Operating Tolerance per AO per Settlement Location per Dispatch Interval – The Resource Operating Tolerance associated with AO a’s Resource at Settlement Location s in Dispatch Interval i. |
| URDMaxTol5minQty a, s, i | MW | Dispatch Interval | Uninstructed Resource Deviation Maximum Tolerance per AO per Settlement Location per Dispatch Interval – The maximum value of ResOpTol5minQty a, s, i that is currently set at 20 MW. |
| URDMinTol5minQty a, s, i | MW | Dispatch Interval | Uninstructed Resource Deviation Minimum Tolerance per AO per Settlement Location per Dispatch Interval – The minimum value of ResOpTol5minQty a, s, i that is currently set at 5 MW. |
| URDTol5minPct a, s, i | Percent | Dispatch Interval | Uninstructed Resource Deviation Tolerance Percentage per AO per Settlement Location per Dispatch Interval – The percentage used to calculate the value of ResOpTol5minQty a, s, i that is currently set at 5%. |
| RtAvgSetPoint5minQty a, s, i | MW | Dispatch Interval | Real-Time Average Setpoint Instruction MW per AO per Settlement Location per Dispatch Interval – The average Setpoint Instruction over Dispatch Interval i for AO a’s Resource at Settlement Location s. |
| XmptDev5minFlg a, s, i | none | Dispatch Interval | URD Exemption Flag per AO per Resource Settlement Location per Dispatch Interval – A flag associated with AO a’s eligible Resource at Settlement Location s indicating that a Resource that has operated outside of its Operating Tolerance is or is not exempt from any associated penalty charges in Dispatch Interval i. If the flag is equal to zero, the Resource is not exempt. Otherwise, the flag will be set to a positive integer number which will indicate the reason of the exemption as specified under Section 4.4.4.1. |
| RtStatusAdj5minAmt a, s, i, c | $ | Dispatch Interval | Resource Status Change Adjustment per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period – The reduction in RUC Make-Whole Payment Amount associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c when the Resource’s Control Status is set to “Manual”. |
| ControlStatus5minFlg a, s, i | None | Dispatch Interval | Control Status per AO per Settlement Location per Dispatch Interval – A Resource status indicator associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i as set by SPP operators that indicates the current dispatchable status of the Resource. |
| RtDispMaxEmerCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Maximum Emergency Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The Maximum Emergency Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i. |
| RtDispMinEconCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Minimum Economic Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The Minimum Economic Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i. |
| RtDispMinRegCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Minimum Regulation Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The Minimum Regulation Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i. |
| RtLimitAdj5minAmt a, s, i, c | $ | Dispatch Interval | Resource Limit Change Adjustment per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period – The reduction in RUC Make-Whole Payment Amount associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c for a Real-Time increase in minimum limit. |
| RtComMinEconCapOL5minQty a, s, i | MW | Eligibility Period | Real-Time Minimum Economic Capacity Operating Limit Quantity per AO per Settlement Location – The Minimum Economic Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i as submitted in an RTBM Offer prior to the RUC Make-Whole-Payment Eligibility Period that was used in making the initial Resource commitment decision. |
| RtComMinRegCapOL5minQty a, s, i | MW | Eligibility Period | Real-Time Minimum Regulation Capacity Operating Limit Quantity per AO per Settlement Location– The Minimum Regulation Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i as submitted in an RTBM Offer prior to the RUC Make-Whole-Payment Eligibility Period that was used in making the initial Resource commitment decision. |
| RtIncrEn5minAmt a, s, i | $ | Dispatch Interval | Real-Time Incremental Energy Cost Amount per AO per Settlement Location per Dispatch Interval - The average incremental energy offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i from the As-Committed Minimum Capacity Limit (Economic or Regulating, as applicable) to an output level equal to RtBillMtr5minQty a, s, i. |
| RtMinEn5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Energy Cost at Minimum Limit per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period - The average incremental energy offer cost at the As-Committed Minimum Capacity Limit (Economic or Regulating, as applicable) associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c |
| RtDesiredEn5minAmt a, s, i | $ | Dispatch Interval | Real-Time Energy Cost at Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval - The average incremental energy offer cost associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i, in dollars, from the As-Committed Minimum Capacity Limit (Economic or Regulating, as applicable) to RtDesiredEn5minQty a, s, i. |
| RtDesiredEn5minQty a, s, i | MW | Dispatch Interval | Real-Time Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval – The Desired Dispatch MW for AO a’s eligible Resource for Dispatch Interval i at RtLmp5minPrc s, i as calculated from the Resource’s As Dispatched Energy Offer Curve using the As-Committed Minimum Capacity Limit (Economic or Regulating, as applicable) as an output floor and the As-Committed Maximum Capacity Limit (Economic or Regulating, as applicable) as an output ceiling. |
| RtOom5minAmt a, s, i | $ | Dispatch Interval | Real-Time Out-Of-Merit Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The value calculated under Section 4.5.9.9. |
| RtRegAdj5minAmt a, s, i | $ | Dispatch Interval | Real-Time Regulation Deployment Adjustment Amount per AO per Resource Settlement Location per Dispatch Interval - The value calculated under Section 4.5.9.19. |
| RtRegUpOffer a, s, i, c (Not Available on Settlement Statement) | $/MW | Dispatch Interval | Real-Time Regulation-Up Offer per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Regulation-Up Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtRegDnOffer a, s, i, c (Not Available on Settlement Statement) | $/MW | Dispatch Interval | Real-Time Regulation-Down Offer per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Regulation-Down Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtSpinOffer a, s, i, c (Not Available on Settlement Statement) | $/MW | Dispatch Interval | Real-Time Spinning Reserve Offer per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Spinning Reserve Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtSuppOffer a, s, i, c (Not Available on Settlement Statement) | $/MW | Dispatch Interval | Real-Time Supplemental Reserve Offer per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Supplemental Reserve Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtFixedRegUp5minQty a, s, i (Not Available on Settlement Statement) | MW | Dispatch Interval | Real-Time Fixed Regulation-Up Quantity per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Fixed Regulation-Up MW specified in the Regulation-Up Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtFixedRegDn5minQty a, s, i (Not Available on Settlement Statement) | MW | Dispatch Interval | Real-Time Fixed Regulation-Down Quantity per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Fixed Regulation-Down MW specified in the Regulation-Down Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtFixedSpin5minQty a, s, i (Not Available on Settlement Statement) | MW | Dispatch Interval | Real-Time Fixed Spinning Reserve Quantity per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Fixed Spinning Reserve MW specified in the Spinning Reserve Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtFixedSupp5minQty a, s, i (Not Available on Settlement Statement) | MW | Dispatch Interval | Real-Time Fixed Supplemental Reserve Quantity per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Fixed Supplemental Reserve MW specified in the Supplemental Reserve Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtRegUpAvail5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Regulation-Up Offer Cost Amount per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period - The Regulation-Up Offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtRegDnAvail5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Regulation-Down Offer Cost Amount per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period - The Regulation-Down Offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in DA Market Commitment Period c. |
| RtSpinAvail5minAmt a, s, i, c | c | Dispatch Interval | Real-Time Spin Offer Cost Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period - The Spinning Reserve Offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtSuppAvail5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Supplemental Offer Cost Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period - The Supplemental Reserve Offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtLmp5minPrc s, i | $/MWh | Dispatch Interval | Real-Time LMP - The value defined under Section 4.5.9.1 at Settlement Location s for Dispatch Interval i. |
| RtBillMtr5minQty a, s, i | MW | Dispatch Interval | Real-Time Actual Meter Quantity per AO per Location per Dispatch Interval - The value defined under Section 4.5.9.1 for Dispatch Interval i. |
| RtRegUpRev5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Regulation-Up Revenue Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Real-Time incremental Regulation-Up revenue associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtRegDnRev5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Regulation-Down Revenue Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Real-Time incremental Regulation-Down revenue associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtSpinRev5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Spinning Reserve Revenue Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Real-Time incremental Spinning Reserve associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtSuppRev5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Supplemental Reserve Revenue Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Real-Time incremental Supplemental Reserve revenue associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtMwpDlyAmt a, s, d | $ | Operating Day | RUC Make-Whole-Payment Amount per AO per Settlement Location per Operating Day - The RUC Make-whole amount to AO a for Operating Day d at Resource Settlement Location s. |
| RtMwpAoAmt a, m, d | $ | Operating Day | RUC Make-Whole-Payment Amount per AO per Operating Day - The RUC Make-whole amount to AO a associated with Market Participant m for Operating Day d. |
| RtMwpMpAmt m, d | $ | Operating Day | RUC Make-Whole-Payment Amount per MP per Operating Day - The RUC Make-whole amount to Market Participant m for Operating Day d. |
| EqrRtMwp5minPrc a, s, c | $ | Eligibility Period | RUC Electric Quarterly Reporting Make-Whole-Payment Amount per AO per Settlement Location per RUC Make-Whole-Payment Eligibility Period - The RUC make-whole amount to AO a for RUC Make-Whole-Payment Eligibility Period c at Resource Settlement Location s for use by AO a in reporting such make-whole-payments to FERC in accordance with FERC EQR requirements.. |
| EqrRtMwp5minQty a, s, c | $ | Eligibility Period | RUC Electric Quarterly Reporting Make-Whole-Payment Quantity per AO per Settlement Location per RUC Make-Whole-Payment Eligibility Period – This value is set equal to 1 if EqrRtMwp5minPrc a, s, c > 0 for use by AO a in reporting such make-whole-payments to FERC in accordance with FERC EQR requirements.. |
| a | none | none | An Asset Owner. |
| i | none | none | A Dispatch Interval. |
| h | none | none | An Hour. |
| d |  |  | An Operating Day. |
| s | none | none | A Settlement Location. |
| c | none | none | A RUC Make-Whole-Payment Eligibility Period. |
| m | none | none | A Market Participant. |

#### Real-Time Out-Of-Merit Amount

The amount to each Asset Owner (AO) for each eligible Resource Settlement Location for each Dispatch Interval is calculated as follows:

#RtOom5minAmt a, s, i = RtOom5minFlg a, s, i

* ( RtOomeIncr5minAmt a, s, i + RtOomeDecr5minAmt a, s, i

+ RtOomor5minAmt a, s, i ) * (-1)

Where,

(a)		RtOomeIncr5minAmt a, s, i  =

Max ( 0, RtDispIncrEn5minAmt a, s, i – RtDispDesiredEn5minAmt a, s, i

+ Min ( 0, RtBillMtr5minQty a, s, i  + RtDispDesiredEn5minQty a, s, i )

* Max( 0, RtLmp5minPrc s, i ) ) / 12

(a.1)	IF ControlStatus5minFlg a, s, i  = “Regulating”

#RtDispIncrEn5minAmt a, s, i =

Where:

X = RtDispMinRegCapOL5minQtya, s, i

Y = Max ( X, Max (RtBillMtr5minQtya, s, i , 0) )

ELSE IF ControlStatus5minFlg a, s, i  < > “Regulating”

#RtDispIncrEn5minAmt a, s, i =

Where:

X = RtDispMinEconCapOL5minQtya, s, i

Y = Max ( X, Max (RtBillMtr5minQtya, s, i , 0) )

(a.2)	IF ControlStatus5minFlg a, s, i  = “Regulating”

#RtDispDesiredEn5minAmt a, s, i =

Where:

X = RtDispMinRegCapOL5minQtya, s, i

Y = Max ( X, RtDispDesiredEn5minQtya, s, i )

ELSE IF

ControlStatus5minFlg a, s, i  < > “Regulating”

#RtDispDesiredEn5minAmt a, s, i =

Where:

X = RtDispMinEconCapOL5minQtya, s, i )

Y = Max ( X, RtDispDesiredEn5minQtya, s, i  )

(b)		RtOomeDecr5minAmt a, s, i  =

Max (0, RtBillMtr5minQty a, s, i  - DaClrdHrlyQty a, s, h  )

* Max ( 0, RtLmp5minPrc s, i  - DaLmpHrlyPrc s, h  )  / 12

(c)		RtOomor5minAmt a, s, i  =

[ ( Max (0, DaRegUpHrlyQty a, z, s, h  - RtRegUp5minQty a, z, s, i )

* Max ( 0, RtRegUpMcp5minPrc z, i  - DaRegUpMcpHrlyPrc z, h  ) )

+ ( Max (0, DaRegDnHrlyQty a, z, s, h  - RtRegDn5minQty a, z, s, i )

* Max ( 0, RtRegDnMcp5minPrc z, i  - DaRegDnMcpHrlyPrc z, h  ) )

+ ( Max (0, DaSpinHrlyQty a, z, s, h  - RtSpin5minQty a, z, s, i )

* Max ( 0, RtSpinMcp5minPrc z, i  - DaSpinMcpHrlyPrc z, h  ) )

+ ( Max (0, DaSuppHrlyQty a, z, s, h  - RtSupp5minQty a, z, s, i )

* Max ( 0, RtSuppMcp5minPrc z, i  - DaSuppMcpHrlyPrc z, h  ) ) ] / 12

- For each Asset Owner, an hourly amount is calculated at each Settlement Location.  The hourly amount is calculated as follows:

RtOomHrlyAmt a, s, h = RtOom5minAmt a, s, i

- For each Asset Owner, a daily amount is calculated at each Settlement Location.  The daily credit amount is calculated as follows:

RtOomDlyAmt a, s, d =  RtOomHrlyAmt a, s, h

- For each Asset Owner associated with Market Participant m, a daily amount is calculated.  The daily amount is calculated as follows:

RtOomAoAmt a, m, d =  RtOomDlyAmt a, s, d

- For each Market Participant, a daily amount is calculated representing the sum of Asset Owner amounts associated with that Market Participant.  The daily amount is calculated as follows:

RtOomMpAmt m, d =  RtOomAoAmt a, m, d

(6)	For FERC Electric Quarterly Reporting (“EQR”) purposes, SPP calculates Real-Time Out-of-Merit Energy and Operating Reserve $ per Dispatch Interval for each Asset Owner as follows:

(a)	EqrRtOom5minPrc a, s, i = (-1) * RtOom5minAmt a, s, i

(b)	IF EqrRtOom5minPrc a, s, i > 0

THEN

EqrRtOom5minQty a, s, i = 1

The above variables are defined as follows:

| Variable | Unit | Settlement Interval | Definition |
| --- | --- | --- | --- |
| RtOom5minAmt a, s, i | $ | Dispatch Interval | Real-Time Out-Of-Merit Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The amount to AO a for eligible Resource Settlement Location s in Dispatch Interval i for Out-Of-Merit Energy and Operating Reserve resulting from an SPP Manual Dispatch Instruction. |
| RtOomeIncr5minAmt a, s, i | $ | Dispatch Interval | Real-Time Out-Of-Merit Incremental Energy Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The portion of AO a’s RtOome5minAmt a, s, i amount for eligible Resource Settlement Location s in Dispatch Interval i for Out-Of-Merit Energy resulting from an SPP manual Dispatch Instruction in the up direction. |
| RtOomeDecr5minAmt a, s, i | $ | Dispatch Interval | Real-Time Out-Of-Merit Decremental Energy Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The portion of AO a’s RtOome5minAmt a, s, i amount for eligible Resource Settlement Location s in Dispatch Interval i for Out-Of-Merit Energy resulting from an SPP manual Dispatch Instruction in the down direction. |
| RtOom5minFlg a, s, i | None | Dispatch Interval | Real-Time Out-of-Merit Flag per AO per Settlement Location per Dispatch Interval – A flag that is set equal to 1 when SPP issues a Manual Dispatch Instruction or whenever there is a price correction event as described under Section 7, otherwise, this flag is set equal to zero. |
| RtOomor5minAmt a, s, i | $ | Dispatch Interval | Real-Time Out-Of-Merit Operating Reserve Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The portion of AO a’s RtOome5minAmt a, s, i attributable to buying back a DA Market Operating Reserve position in the RTBM at a RTBM MCP that is greater than the corresponding DA Market MCP. This should not be a normal occurrence but could happen as a result of price corrections as described under Section 7. |
| RtDispDesiredEn5minQty a, s, i | MW | Dispatch Interval | Real-Time Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval – The Desired Dispatch MW for AO a’s eligible Resource for Dispatch Interval i at RtLmp5minPrc s, i as calculated from the Resource’s As Dispatched Energy Offer Curve using the As-Dispatched Minimum Capacity Limit (Economic or Regulating, as applicable) as an output floor and the As-Dispatched Maximum Capacity Limit (Economic or Regulating, as applicable) as an output ceiling. |
| RtDispIncrEn5minAmt a, s, i | $ | Dispatch Interval | Real-Time Incremental Energy Cost Amount per AO per Settlement Location per Dispatch Interval - The average incremental energy offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i from the As-Dispatched Minimum Capacity Limit (Economic or Regulating, as applicable) to an output level equal to RtBillMtr5minQty a, s, i. |
| RtDispDesiredEn5minAmt a, s, i | $ | Dispatch Interval | Real-Time Energy Cost at Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval - The average incremental energy offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i from the As-Dispatched Minimum Capacity Limit (Economic or Regulating, as applicable) to an output level equal to RtDispDesiredEn5minQty a, s, i. |
| RtBillMtr5minQty a, s, i | MW | Dispatch Interval | Real-Time Actual Meter Quantity per AO per Location per Dispatch Interval - The value defined under Section 4.5.9.1 for Dispatch Interval i. |
| RtLmp5minPrc s, i | $/MW | Dispatch Interval | Real-Time LMP - The value defined under Section 4.5.9.1 at Settlement Location s for Dispatch Interval i. |
| DaClrdHrlyQty a, s, h | MWh | Hour | Day-Ahead Cleared Energy Quantity per AO per Settlement Location per Hour in the DA Market – The value described under Section 4.5.8.1. |
| DaRegUpHrlyQty a, z, s, h | MW | Hour | Day-Ahead Regulation-Up Quantity per AO per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.4. |
| DaRegDnHrlyQty a, z, s, h | MW | Hour | Day-Ahead Regulation-Down Quantity per AO per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.5. |
| DaSpinHrlyQty a, z, s, h | MW | Hour | Day-Ahead Spinning Reserve Quantity per AO per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.6. |
| DaSuppHrlyQty a, z, s, h | MW | Hour | Day-Ahead Spinning Reserve Quantity per AO per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.7. |
| RtRegUp5minQty a, z, s, i | MW | Dispatch Interval | Real-Time Regulation-Up Quantity per AO per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.4. |
| RtRegDn5minQty a, z, s, i | MW | Dispatch Interval | Real-Time Regulation-Down Quantity per AO per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.5. |
| RtSpin5minQty a, z, s, i | MW | Dispatch Interval | Real-Time Spinning Reserve Quantity per AO per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.6. |
| RtSupp5minQty a, z, s, i | MW | Dispatch Interval | Real-Time Supplemental Reserve Quantity per AO per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.7. |
| DaRegUpMcpHrlyPrc z, h | $/MW | Hour | Day-Ahead Regulation-Up Market Clearing Price per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.4. |
| DaRegDnMcpHrlyPrc z, h | $/MW | Hour | Day-Ahead Regulation-Down Market Clearing Price per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.5. |
| DaSpinMcpHrlyPrc z, h | $/MW | Hour | Day-Ahead Spinning Reserve Market Clearing Price per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.6. |
| DaSuppMcpHrlyPrc z, h | $/MW | Hour | Day-Ahead Spinning Reserve Market Clearing Price per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.7. |
| RtRegUpMcp5minPrc z, i | $/MW | Dispatch Interval | Real-Time Regulation-Up Market Clearing Price per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.4. |
| RtRegDnMcp5minPrc z, i | $/MW | Dispatch Interval | Real-Time Regulation-Down Market Clearing Price per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.5. |
| RtSpinMcp5minPrc z, i | $/MW | Dispatch Interval | Real-Time Spinning Reserve Market Clearing Price per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.6. |
| RtSuppMcp5minPrc z, i | $/MW | Dispatch Interval | Real-Time Supplemental Reserve Market Clearing Price per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.7. |
| RtOomHrlyAmt a, s, h | $ | Hour | Real-Time Out-Of-Merit Make-Whole-Payment Amount per AO per Settlement Location per Hour - The amount to AO a for eligible Resource Settlement Location s in Hour h for Out-Of-Merit Energy and Operating Reserve resulting from an SPP Manual Dispatch Instruction. |
| RtOomDlyAmt a, s, d | $ | Operating Day | Real-Time Out-Of-Merit Make-Whole-Payment Amount per AO per Settlement Location per Operating Day - The amount to AO a for eligible Resource Settlement Location s in Operating Day d for Out-Of-Merit Energy and Operating Reserve resulting from an SPP Manual Dispatch Instruction. |
| RtOomAoAmt a, m, d | $ | Operating Day | Real-Time Out-Of-Merit Make-Whole-Payment Amount per AO per Operating Day - The amount to AO a associated with Market Participant m in Operating Day d for Out-Of-Merit Energy and Operating Reserve resulting from an SPP Manual Dispatch Instruction. |
| RtOomMpAmt m, d | $ | Operating Day | Real-Time Out-Of-Merit Make-Whole-Payment Amount per MP per Operating Day - The amount to MP m in Operating Day d for Out-Of-Merit Energy and Operating Reserve resulting from an SPP Manual Dispatch Instruction. |
| EqrRtOom5minPrc a, s, i | $ | Dispatch Interval | Real-Time Electric Quarterly Reporting Out-of-Merit Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The Out-of-Merit make-whole amount to AO a for Dispatch Interval i at Resource Settlement Location s for use by AO a in reporting such make-whole-payments to FERC in accordance with FERC EQR requirements. |
| EqrRtOom5minQty a, s, i | $ | Dispatch Interval | Real-Time Electric Quarterly Reporting Out-of-Merit Make-Whole-Payment Quantity per AO per Settlement Location per Dispatch Interval – This value is set equal to 1 if EqrRtOom5minPrc a, s, i > 0 for use by AO a in reporting such make-whole-payments to FERC in accordance with FERC EQR requirements.. |
| a | none | none | An Asset Owner. |
| s | none | none | A Settlement Location. |
| i | none | none | A Dispatch Interval. |
| h | none | none | An Hour. |
| d | none | none | An Operating Day. |
| m | none | none | A Market Participant. |

#### 4.5.9.10	RUC Make-Whole-Payment Distribution Amount

- A RTBM charge or credit will be calculated at each Settlement Location for each Asset Owner for each hour in order to fund the payments made under Section 4.5.9.8.  The amount will be determined by multiplying the Asset Owner deviations by a daily RTBM MWP rate.

The hourly amount is calculated as follows:

#RtMwpDistHrlyAmt a, s, h =  RtMwpSppDistRate d *  RtDevHrlyQty a, s, h

Where,

(a)	RtDevHrlyQty a, s, h =  RtNetSlDevHrlyQty a, s, h + RtMinLimitDevHrlyQty a, s, h

+ RtMaxLimitDevHrlyQty a, s, h+ RtOutageDevHrlyQty a, s, h

+ RtStatusDevHrlyQty a, s, h + RtRucScDevHrlyQty a, s, h

+ RtRucCommitDevHrlyQty a, s, h + RtURDDevHrlyQty a, s, h

(a.1)	An Asset Owner’s Settlement Location deviation is calculated as the Absolute Value of the sum of (1) (RTBM actual load MWh - DA Market cleared load MWh), (2) (RTBM actual Export Interchange Transactions – DA Market cleared Export Interchange Transactions), (3) (RTBM actual Import Interchange Transactions – DA Market cleared Import Interchange Transactions), (4) (RTBM actual Through Interchange Transactions (sink only) – DA Market cleared Through Interchange Transactions (sink only)), (5) DA Market cleared Virtual Energy Bids * (-1), and (6) DA Market cleared Virtual Energy Offers * (-1).  An Asset Owner’s Settlement Location deviation is calculated as follows.

RtNetSlDevHrlyQty a, s, h = ABS RtNetSlDev5minQty a, s, i

#RtNetSlDev5minQty a, s, i =

{ Max ( 0, RtBillMtr5minQty a, s, i   )  – Max ( 0, DaClrdHrlyQty a, s, h )

+ [Max ( 0, RtImpExp5minQty a, s, i, t, dir  )

- Max ( 0, DaImpExp5minQty a, s, i, t, dir  )

+ [ IF DIR <> “THROUGH”, THEN

Min ( 0, RtImpExp5minQty a, s, i, t, dir  )

- Min ( 0, DaImpExp5minQty a, s, i, t, dir  ), ELSE 0 ] ]

* (1 – RsgCrdFlgt ) - DaClrdVHrlyQty a, s, h, t } / 12

(a.2)	For a Resource with DA Market cleared MW in an hour, if the Resource’s Minimum Economic Capacity Operating Limit (or Minimum Regulation Capacity Operating Limit if cleared for Regulation-Up or Regulation-Down) in the RTBM is (1) greater than the comparable limits used to clear the Resource in the DA Market by more than the Resource Operating Tolerance; (2) is greater than its DA Market cleared MW; and (3) the Resource’s Desired Dispatch in any Dispatch Interval within the Hour is less than or equal to the Resource’s applicable minimum limit, the difference between the Resource’s applicable minimum limit and its DA Market cleared MW is included as a deviation.  In the case where the Resource has cleared Regulation-Up or Regulation Down in the RTBM and has not cleared Regulation-Up or Regulation Down in the DA Market, the deviation is the lesser of the (1) the difference between the Resource’s RTBM regulation minimum limit and its DA Market cleared MW or (2) the difference between the Resource’s RTBM regulation minimum limit the its DA Market regulation minimum limit.

RtMinLimitDevHrlyQty a, s, h  =   RtMinLimitDev5minQty a, s, i

Where,

IF DesiredDispatchMinHrlyFlg a, s, h= “1” AND DaClrdHrlyQty a, s, h  < 0

AND RtRucComStat5minFlg a, s, i, c < > “1”

AND RtRucComStat5minFlg a, s, i, c < > “0”

THEN

** Regulation is not cleared in RTBM **

IF ControlStatus5minFlg a, s, i <> “Regulating” AND

( RtDispMinEconCapOL5minQty a, s, i - DaComMinEconCapOLHrlyQty a, s, h)

> ResOpTol5minQty a, s, i

THEN

#RtMinLimitDev5minQty a, s, i =

Max [ ( RtDispMinEconCapOL5minQty a, s, i +  DaClrdHrlyQty a, s, h ), 0 ] / 12

ELSE IF

** Regulation is cleared in both DA Market and RTBM **

IF ControlStatus5minFlg a, s, i = “Regulating” AND

DaRegUpHrlyQty a, z, s, h + DaRegDnHrlyQty a, z, s, h > 0 AND

( RtDispMinRegCapOL5minQty a, s, i - DaComMinRegCapOLHrlyQty a, s, h)

> ResOpTol5minQty a, s, i

THEN

#RtMinLimitDev5minQty a, s, i  =

Max [ ( RtDispMinRegCapOL5minQty a, s, i +  DaClrdHrlyQty a, s, h ), 0 ] / 12

ELSE IF

** Regulation is cleared in RTBM and not cleared in DA Market **

IF ControlStatus5minFlg a, s, i = “Regulating” AND

DaRegUpHrlyQty a, z, s, h + DaRegDnHrlyQty a, z, s, h = 0 AND

( RtDispMinRegCapOL5minQty a, s, i - DaComMinRegCapOLHrlyQty a, s, h)

> ResOpTol5minQty a, s, i

THEN

#RtMinLimitDev5minQty a, s, i  =

Max { RtDispMinRegCapOL5minQty a, s, i

– Max [ ABS ( DaClrdHrlyQty a, s, h ) , DaComMinRegCapOLHrlyQty a, s, h ] , 0 } / 12

ELSE

RtMinLimitDev5minQty a, s, i  = 0

(a.3)	For a Resource with DA Market cleared MW in an hour, if the Resource’s Maximum Economic Capacity Operating Limit (or Maximum Regulation Capacity Operating Limit if cleared for Regulation-Up or Regulation-Down) in the RTBM is (1) less than the comparable limits used to clear the Resource in the DA Market by more than the Resource Operating Tolerance; (2) is less than its DA Market cleared MW; and (3) the Resource’s Desired Dispatch in any Dispatch Interval within the Hour is greater than or equal to the Resource’s applicable maximum limit, the difference between the Resource’s DA Market cleared MW and its applicable maximum limit is included as a deviation.  In the case where the Resource has cleared Regulation-Up or Regulation Down in the RTBM and has not cleared Regulation-Up or Regulation Down in the DA Market, the deviation is the lesser of the (1) the difference between the Resource’s DA Market cleared MW and its RTBM regulation maximum limit or (2) the difference between the Resource’s DA Market regulation minimum limit and its RTBM regulation minimum limit.

RtMaxLimitDevHrlyQty a, s, h  =   RtMaxLimitDev5minQty a, s, i

Where,

IF DesiredDispatchMaxHrlyFlg a, s, h= “1” AND DaClrdHrlyQty a, s, h  < 0

THEN

** Regulation is not cleared in RTBM **

IF ControlStatus5minFlg a, s, i <> “Regulating” AND

( DaComMaxEconCapOLHrlyQty a, s, h - RtDispMaxEconCapOL5minQty a, s, i)

> ResOpTol5minQty a, s, i

THEN

#RtMaxLimitDev5minQty a, s, i  =

Max [ (ABS ( DaClrdHrlyQty a, s, h ) - RtDispMaxEconCapOL5minQty a, s, i), 0 ] / 12

ELSE IF

** Regulation is cleared in both DA Market and RTBM **

IF ControlStatus5minFlg a, s, i = “Regulating” AND

DaRegUpHrlyQty a, z, s, h + DaRegDnHrlyQty a, z, s, h > 0 AND

( DaComMaxRegCapOLHrlyQty a, s, h - RtDispMaxRegCapOL5minQty a, s, i )

> ResOpTol5minQty a, s, i

THEN

#RtMaxLimitDev5minQty a, s, i  =

Max [ (ABS ( DaClrdHrlyQty a, s, h ) - RtDispMaxRegCapOL5minQty a, s, i), 0 ] / 12

ELSE IF

** Regulation is cleared in RTBM and not cleared in DA Market **

IF ControlStatus5minFlg a, s, i = “Regulating” AND

DaRegUpHrlyQty a, z, s, h + DaRegDnHrlyQty a, z, s, h = 0 AND

( DaComMaxRegCapOLHrlyQty a, s, h - RtDispMaxRegCapOL5minQty a, s, i )

> ResOpTol5minQty a, s, i

THEN

#RtMaxLimitDev5minQty a, s, i  =

Max { Min [ ABS ( DaClrdHrlyQty a, s, h ) , DaComMaxRegCapOLHrlyQty a, s, h ]

- RtDispMaxRegCapOL5minQty a, s, i  , 0 } / 12

ELSE

RtMaxLimitDev5minQty a, s, i  = 0

(a.4)	 For Resources with DA Market cleared MW in an hour, if the Resource is off-line in the RTBM and it has not been de-committed by SPP the Resource DA Market cleared MW is included as a deviation.  An Asset Owner’s outage deviation is calculated as follows.

RtOutageDevHrlyQty a, s, h = RtOutageDev5minQty a, s, i

IF DaClrdHrlyQty a, s, h  < 0 AND

RtBillMtr5minQty a, s, i  >= 0 AND

ResDeCommit5minFlg a, s, i  < > “1”

THEN

#RtOutageDev5minQty a, s, i =  ABS ( DaClrdHrlyQty a, s, h ) / 12

ELSE

RtOutageDev5minQty a, s, i =  0

(a.5)	For Resources with DA Market cleared MW in an hour, for each Dispatch Interval the Resource is in “Manual” status, a deviation is calculated that is equal to one-twelfth of the difference between the Resource actual output and the Resource’s Desired Dispatch.  An Asset Owner’s status change deviation is calculated as follows.

RtStatusDevHrlyQty a, s, h =  RtStatusDev5minQty a, s, i

IF ControlStatus5minFlg a, s, i = “Manual” AND

DaClrdHrlyQty a, s, h  < 0

THEN

#RtStatusDev5minQty a, s, i =

ABS ( RtBillMtr5minQty a, s, i + RtDispDesiredEn5minQty a, s, i ) / 12

ELSE

RtStatusDev5minQty a, s, i = 0

(a.6)	For Resources that Self-Committed following the Day-Ahead Market and the Resource’s Desired Dispatch in any Dispatch Interval within the Hour is less than or equal to the Resource’s As-Dispatched Minimum Economic Capacity Operating Limit, a deviation is included in an amount equal to the Resource actual output or, in the case of a Combined Cycle Resource, a deviation is included in an amount equal to the difference between the Resource actual output and the Resource DA Market cleared amount.  Resources that were offered into the DA Market for SPP commitment and not committed in the DA Market and then Self-Committed prior to the Day-Ahead RUC are exempted from this calculation.  An Asset Owner’s Self-Commit deviation is calculated as follows.

RtRucScDevHrlyQty a, s, h =  RtRucScDev5minQty a, s, i

IF RtRucComStat5minFlg a, s, i, c = “0” AND

DesiredDispatchMinHrlyFlg a, s, h = “1” AND

RtRucScDevXmpt5minFlg a, s, i <> “1”

THEN

#RtRucScDev5minQty a, s, i =

ABS ( Min (RtBillMtr5minQty a, s, i  - DaClrdHrlyQty a, s, h  , 0 ) / 12 )

ELSE

RtRucScDev5minQty a, s, i = 0

(a.7)	For Resources that are either Self-Committed or committed by SPP following the DA Market and that are off-line in the RTBM and have not been de-committed by SPP, the greater of the Minimum Economic Capacity Operating Limit at the time of commitment or the Resource’s Desired Dispatch will be included as a deviation. An Asset Owner’s RTBM commitment outage deviation is calculated as follows.

RtRucCommitDevHrlyQty a, s, h =  RtRucCommitDev5minQty a, s, i

IF [ RtRucComStat5minFlg a, s, i, c = “0”  OR

RtRucComStat5minFlg a, s, i, c = “1” ] AND

RtBillMtr5minQty a, s, i   >= 0 AND

ResDeCommit5minFlg a, s, i  < > 1

THEN

#RtRucCommitDev5minQty a, s, i   =  RtDesiredEn5minQty a, s, i  / 12

ELSE

RtRucCommitDev5minQty a, s, i   = 0

(a.8)	In any Dispatch Interval in which a Resource operates outside of its Operating Tolerance and the Resource has not been exempted from URD per Section 4.4.4.1, one-twelfth of the Absolute Value of the Resource’s Uninstructed Resource Deviation is included as a deviation.  An Asset Owner’s URD deviation is calculated as follows.

RtURDDevHrlyQty a, s, h =  RtURDDev5minQty a, s, i

IF ABS (URD5minQty a, s, i ) > ResOpTol5minQty a, s, i  AND

( XmptDev5minFlg a, s, i =  0 )

THEN

#RtURDDev5minQty a, s, i = ABS ( URD5minQty a, s, i ) / 12

ELSE

RtURDDev5minQty a, s, i = 0

(b)	#RtMwpSppDistRate d =

( RtMwpSppDlyAmt d / RtDevSppDlyQty d ) * (-1)

(b.1)		RtMwpSppDlyAmt d =  RtMwpMpAmt m, d

(b.2)		RtDevSppDlyQty d =  RtDevHrlyQty a, s, h

- For each Asset Owner, a daily amount is calculated at each Settlement Location.  The daily amount is calculated as follows:

RtMwpDistDlyAmt a, s, d = RtMwpDistHrlyAmt a, s, h

- For each Asset Owner associated with Market Participant m, a daily amount is calculated.  The daily amount is calculated as follows:

RtMwpDistAoAmt a, m, d =  RtMwpDistDlyAmt a, s, d

- For each Market Participant, a daily amount is calculated representing the sum of Asset Owner amounts associated with that Market Participant.  The daily amount is calculated as follows:

RtMwpDistMpAmt m, d =  RtMwpDistAoAmt a, m, d

The above variables are defined as follows:

| Variable | Unit | Settlement Interval | Definition |
| --- | --- | --- | --- |
| RtMwpDistHrlyAmt a, s, h | $ | Hour | RUC Make-Whole-Payment Distribution Amount per AO per Hour per Settlement Location - The amount to AO a for Hour h and Settlement Location s for recovery of the total amount paid under Section 4.5.9.8 for Operating Day d. |
| RtDevHrlyQty a, s, h | MWh | Hour | Real-Time Deviation Quantity per AO per Hour per Settlement Location – The total deviation MWh for AO a at Settlement Location s for Hour h. |
| RtMwpSppDistRate d | $/MWh | Operating Day | RUC Make-Whole Payment SPP Distribution Rate per Operating Day – The rate applied to AO a’s RtDevHrlyQty a, s, h in each Hour h at Settlement Location s in Operating Day d. |
| RtMwpMpAmt m, d | $ | Operating Day | RUC Make-Whole-Payment Amount per MP per Operating Day - The value calculated under Section 4.5.9.8 for Operating Day d. |
| RtMwpSppDlyAmt d | $ | Operating Day | RUC Make-Whole-Payment Amount per Operating Day - The SPP total of the values calculated under Section 4.5.9.8 for Operating Day d. |
| RtDevSppDlyQty d | MWh | Operating Day | Real-Time Deviation Quantity per Operating Day - The SPP total deviation MWh for all AOs for Operating Day d. |
| RtNetSlDevHrlyQty a, s, i | MWh | Dispatch Interval | Real-Time Net Settlement Location Deviation per AO per Dispatch Interval per Settlement Location – AO a’s portion of RtNetSlDevHrlyQty a, s, h related to net of Real-Time load deviations from Day-Ahead amount, Real-Time Interchange Transaction deviations from Day-Ahead amounts and virtual transactions at Settlement Location s in Dispatch Interval i. |
| RtNetSlDevHrlyQty a, s, h | MWh | Hour | Real-Time Net Settlement Location Deviation per AO per Hour per Settlement Location – AO a’s portion of RtDevHrlyQty a, s, h related to net of Real-Time load deviations from Day-Ahead amount, Real-Time Interchange Transaction deviations from Day-Ahead amounts and virtual transactions at Settlement Location s in Hour h. |
| RtBillMtr5minQty a, s, i | MW | Dispatch Interval | Real-Time Billing Meter Quantity per AO per Settlement Location per Dispatch Interval - The quantity described under Section 4.5.9.1. |
| DaClrdHrlyQty a, s, h | MWh | Hour | Day-Ahead Cleared Energy Quantity per AO per Settlement Location per Hour in the DA Market – The quantity described under Section 4.5.8.1. |
| RtImpExp5minQty a, s, i, t, dir | MW | Dispatch Interval | Real-Time Interchange Transaction Quantity per AO per Settlement Location per Dispatch Interval - The quantity described under Section 4.5.9.2 as identified by direction dir. |
| RsgCrdFlgt | none | none | Reserve Sharing Group Contingency Reserve Deployment Flag per Event – The value described under Section 4.5.8.8. |
| DaImpExp5minQty a, s, i, t, dir | MWh | Dispatch Interval | Day-Ahead Interchange Transaction Quantity per AO per Settlement Location per Dispatch Interval - The quantity described under Section 4.5.8.2 as identified by direction dir. |
| DaClrdVHrlyQty a, s, h, t | MWh | Hour | Day-Ahead Cleared Virtual Energy Quantity per AO per Settlement Location per Transaction per Hour in the DA Market – The quantity described under Section 4.5.8.3. |
| RtMinLimitDev5minQty a, s, i | MWh | Dispatch Interval | Real-Time Minimum Limit Deviation per AO per Dispatch Interval per Settlement Location – AO a’s portion of RtDevHrlyQty a, s, h associated with Resources with cleared Day-Ahead amounts that increase their RtMinLimit a, s, h above their DaMinLimit a, s, h at Resource Settlement Location s in Hour h. |
| RtMinLimitDevHrlyQty a, s, h | MWh | Hour | Real-Time Minimum Limit Deviation per AO per Hour per Settlement Location – The sum of AO a’s RtMinLimitDev5minQty a, s, i at Resource Settlement Location s in Hour h. |
| RtMaxLimitDev5minQty a, s, i | MWh | Dispatch Interval | Real-Time Maximum Limit Deviation per AO per Dispatch Interval per Settlement Location – AO a’s portion of RtDevHrlyQty a, s, h associated with Resources with cleared Day-Ahead amounts that reduce their RtMaxLimit a, s, h below their DaMaxLimit a, s, h at Resource Settlement Location s in Hour h. |
| RtMaxLimitDevHrlyQty a, s, h | MWh | Hour | Real-Time Maximum Limit Deviation per AO per Hour per Settlement Location – The sum of AO a’s RtMaxLimitDev5minQty a, s, i at Resource Settlement Location s in Hour h. |
| RtOutageDev5minQty a, s, i | MWh | Dispatch Interval | Real-Time Outage Deviation per AO per Dispatch Interval per Settlement Location – AO a’s portion of RtDevHrlyQty a, s, h associated with Resources with cleared Day-Ahead amounts that are off-line in Real-Time and have not be de-committed by SPP at Resource Settlement Location s in Dispatch Interval i. |
| RtOutageDevHrlyQty a, s, h | MWh | Hour | Real-Time Outage Deviation per AO per Hour per Settlement Location – The sum of AO a’s RtOutageDev5minQty a, s, i at Resource Settlement Location s in Hour h. |
| RtStatusDev5minQty a, s, i | MWh | Dispatch Interval | Real-Time Resource Status Change Deviation per AO per Settlement Location per Dispatch Interval – AO a’s portion of RtDevHrlyQty a, s, h associated with Resources for which the Control Status is set to “Manual” at Settlement Location s for Dispatch Interval i. |
| RtStatusDevHrlyQty a, s, h | MWh | Dispatch Interval | Real-Time Status Deviation per AO per Hour per Settlement Location – The sum of AO a’s RtStatusDev5minQty a, s, i at Resource Settlement Location s in Hour h. |
| ControlStatus5minFlg a, s, i | none | Dispatch Interval | Control Status per AO per Settlement Location per Dispatch Interval – The value described under Section 4.5.9.8. |
| RtRucScDev5minQty a, s, i | MWh | Dispatch Interval | Real-Time RUC Self-Commit Deviation per AO per Settlement Location per Dispatch Interval – AO a’s portion of RtDevHrlyQty a, s, h associated with Resources that have Self-Committed following completion of the Day-Ahead RUC process at Settlement Location s for Dispatch Interval i. |
| RtRucScDevXmpt5minFlg a, s, i | none | Dispatch Interval | Real-Time RUC Self-Commit Deviation Exemption Flag per AO per Settlement Location per Dispatch Interval – a value of 1 for AO a’s Resources at Settlement Location s for Dispatch Interval i that were offered into the DA Market for SPP commitment and not committed in the DA Market and then Self-Committed prior to the Day-Ahead RUC, thus are exempted from Self-Commit deviation calculations. |
| RtRucScDevHrlyQty a, s, h | MWh | Hour | Real-Time RUC Self-Commit Deviation per AO per Settlement Location per Hour – The summation of AO a’s RtRucScDev5minQty a, s, i for Hour h. |
| RtRucCommitDev5minQty a, s, i | MWh | Dispatch Interval | Real-Time RUC Commit Deviation per AO per Settlement Location per Dispatch Interval – AO a’s portion of RtDevHrlyQty a, s, h associated with Resources that were committed in the Day-Ahead RUC process and fail to come on line at Settlement Location s for Dispatch Interval i. |
| RtRucCommitDevHrlyQty a, s, h | MWh | Dispatch Interval | Real-Time RUC Commit Deviation per AO per Settlement Location per Hour – The summation of AO a’s RtRucCommitDev5minQty a, s, i for Hour h. |
| RtURDDev5minQty a, s, i | MWh | Dispatch Interval | Real-Time URD Deviation per AO per Settlement Location per Dispatch Interval – AO a’s portion of RtDevHrlyQty a, s, h associated with Resources that have operated outside of their ResOpTol a, s, i at Settlement Location s for Dispatch Interval i. |
| RtURDDevHrlyQty a, s, h | MWh | Hour | Real-Time URD Deviation per AO per Hour per Settlement Location – The sum of AO a’s RtURDDev5minQty a, s, i at Resource Settlement Location s in Hour h. |
| URD5minQty a, s, i, | MW | Dispatch Interval | Uninstructed Resource Deviation per AO per Settlement Location per Dispatch Interval– The value calculated as described under Section 4.5.9.8. |
| XmptDev5minFlg a, s, i | none | Dispatch Interval | Failure-to-Follow Dispatch Exemption Flag per AO per Resource Settlement Location per Dispatch Interval – The value described under Section 4.5.9.8. |
| RtDispDesiredEn5minQty a, s, i |  | MW | Dispatch Interval |  |  | Real-Time Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval – The value described under Section 4.5.9.9. |  |
| RtDesiredEn5minQty a, s, i | MW | Dispatch Interval | Real-Time Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval – The value described under Section 4.5.9.8. |
| RtDispMinEconCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Minimum Economic Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The value described under Section 4.5.9.8 for Dispatch Interval i. |
| RtDispMinRegCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Minimum Regulation Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The value described under Section 4.5.9.8 for Dispatch Interval i. |
| DaComMinEconCapOLHrlyQty a, s, h | MW | Hour | Day-Ahead Minimum Economic Capacity Operating Limit Quantity per AO per Settlement Location per Hour – The Minimum Economic Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Hour h as submitted in the DA Market Offer used in the DA Market commitment decision. |
| DaComMinRegCapOLHrlyQty a, s, h | MW | Hour | Day-Ahead Minimum Regulation Capacity Operating Limit Quantity per AO per Settlement Location per Hour – The Minimum Regulation Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Hour h as submitted in the DA Market Offer used in the DA Market commitment decision. |
| RtDispMaxEconCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Maximum Economic Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The Maximum Economic Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i. |
| RtDispMaxRegCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Maximum Regulation Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The Maximum Regulation Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i. |
| DaComMaxEconCapOLHrlyQty a, s, h | MW | Hour | Day-Ahead Maximum Economic Capacity Operating Limit Quantity per AO per Settlement Location per Hour – The Maximum Economic Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Hour h as submitted in the DA Market Offer used in the DA Market commitment decision. |
| DaComMaxRegCapOLHrlyQty a, s, h | MW | Hour | Day-Ahead Maximum Regulation Capacity Operating Limit Quantity per AO per Settlement Location per Hour – The Maximum Regulation Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Hour h as submitted in the DA Market Offer used in the DA Market commitment decision. |
| ResOpTol5minQty a, s, i | MWh | Dispatch Interval | Resource Operating Tolerance per AO per Settlement Location per Hour – The value calculated as described under Section 4.5.9.8. |
| ResDeCommit5minFlg a, s, i | none | Dispatch Interval | Resource De-Commitment Flag per AO per Dispatch Interval per Settlement Location – A flag set by SPP indicating that AO a’s Resource has been de-committed by SPP at Resource Settlement Location s in Dispatch Interval i. |
| RtRucComStat5minFlg a, s, i, c | none | Dispatch Interval | RUC Commitment Status Flag per AO per Resource Settlement Location per Dispatch Interval – The value described under Section 4.5.9.8. |
| DesiredDispatchMinHrlyFlg a, s, h | none | Hour | Desired Dispatch Minimum Flag per AO per Hour per Settlement Location – A flag associated with AO a’s Resource that is set equal to “1” if the Resource’s Desired Dispatch is less than or equal to the Resource’s As-Dispatched Minimum Economic Capacity Operating Limit at any time in Hour h. |
| DesiredDispatchMaxHrlyFlg a, s, h | none | Hour | Desired Dispatch Maximum Flag per AO per Hour per Settlement Location – A flag associated with AO a’s Resource that is set equal to “1” if the Resource’s Desired Dispatch is greater than or equal to the Resource’s As-Dispatched Maximum Economic Capacity Operating Limit at any time in Hour h. |
| DaRegUpHrlyQty a, z, s, h | MW | Hour | Day-Ahead Regulation-Up Quantity per AO per Settlement Location per Hour – The value described under Section 4.5.8.4. |
| DaRegDnHrlyQty a, z, s, h | MW | Hour | Day-Ahead Regulation-Down Quantity per AO per Settlement Location per Hour – The value described under Section 4.5.8.5. |
| RtMwpDistDlyAmt a, s, d | $ | Operating Day | RUC Make-Whole-Payment Distribution Amount per AO per Settlement Location per Operating Day - The amount to AO a at Settlement Location s for recovery of the total amount paid under Section 4.5.9.8 for Operating Day d. |
| RtMwpDistAoAmt a, m, d | $ | Operating Day | RUC Make-Whole-Payment Distribution Amount per AO per Operating Day - The amount to AO a associated with Market Participant m for recovery of the total amounts paid under Section 4.5.9.8 for Operating Day d. |
| RtMwpDistMpAmt m, d | $ | Operating Day | RUC Make-Whole-Payment Distribution Amount per MP per Operating Day - The amount to MP m for recovery of the total amounts paid under Section 4.5.9.8 for Operating Day d. |
| i | none | none | An Dispatch Interval |
| h | none | none | An Hour. |
| d | none | none | An Operating Day. |
| a | none | none | An Asset Owner. |
| c | none | none | A RUC Make-Whole-Payment Eligibility Period. |
| s | none | None | A Settlement Location. |
| t | none | none | A single tagged Interchange Transaction, a single virtual energy transaction, a single Financial Schedule, a single contracted Operating Reserve transaction, a single TCR instrument, a single ARR award or a single Reserve Sharing Event transaction. |
| dir | none | none | Direction (Import, Export or Through). |
| m | none | none | A Market Participant. |

Proposed Tariff Language Revision

## 8.6.7	Reliability Unit Commitment Make Whole Payment Distribution Amount [Protocols Section 4.5.9.10]

An RTBM charge will be calculated at each Settlement Location for each Asset Owner for each hour in order to fund the payments made under Section 8.6.5.  The amount will be determined by multiplying An Asset Owners distribution volume by a daily RUC make whole payment rate as follows:

RUC Make Whole Payment Distribution Amount =

[(RUC SPP Make Whole Payment Distribution Rate) * (RUC Make Whole Payment Distribution Volume)]

(1)	The RUC SPP Make Whole Payment Distribution Rate is the sum of all make whole payments for the Operating Day as calculated under Section 8.6.5, divided by the sum of Asset Owners’ RUC Make Whole Payment Distribution Volumes for all Settlement Locations for the entire Operating Day.

(2)	An Asset Owner’s RUC Make Whole Payment Distribution Volume at a Settlement Location for an hour is equal to the sum of following values that are calculated for each Dispatch Interval within the hour:

(a)	The absolute value of the sum of actual Real-Time Settlement Location deviations from Day-Ahead Market cleared amounts for load, virtual transactions and interchange transactions except that, during any Dispatch Interval in which the Transmission Provider has declared an Emergency Condition, Real-Time actual load deviations from Day-Ahead Market cleared amounts shall be limited to deviations associated with actual Real-Time load in excess of amounts cleared in the Day-Ahead Market;

(b)	For Resources cleared in the Day-Ahead Market, the positive difference between RTBM Resource applicable minimum limits and Day-Ahead Market Resource minimum limits, if:

(i)	Such difference is greater than the Resource’s Operating Tolerance; and

(ii)	The applicable RTBM Resource minimum limit is greater than the Day-Ahead Market cleared Energy amount; and

(iii)	The Resource’s economic operating point as calculated from the Resource’s current Dispatch Interval Energy Offer Curve is less than or equal to the RTBM applicable minimum limit for at least one Dispatch Interval in the hour.

(c)	For Resources cleared in the Day-Ahead Market, the positive difference between the Day-Ahead Market Resource applicable maximum limits and the RTBM Resource applicable maximum limits, if:

(i)	Such difference is greater than the Resource’s Operating Tolerance; and

(ii)	The applicable RTBM Resource maximum limit is less than the Day-Ahead Market cleared Energy amount; and

(iii)	The Resource’s economic operating point as calculated from the Resource’s current Dispatch Interval Energy Offer Curve is greater than or equal to the applicable RTBM applicable maximum limit for at least one Dispatch Interval in the hour.

(d)	For Resources cleared in the Day-Ahead Market, the Resource’s Day-Ahead Market cleared amount if that Resource is off-line in the RTBM and if the Resource has not been de-committed by the Transmission Provider;

(e)	For Resources that cleared in the Day-Ahead Market that are not able to follow Dispatch Instructions, the absolute value of the difference between a Resource’s actual output and the Resource’s economic operating point.  The Resource’s economic operating point is calculated as described under Section 8.6.5(4)(d);

(f)	For Resources that were not cleared in the Day-Ahead Market and that self-committed following the close of the Day-Ahead Market, the actual Resource output if the Resource’s economic operating point as calculated from the Resource’s current Dispatch Interval Energy Offer Curve is less than or equal to the applicable RTBM applicable minimum limit for at least one Dispatch Interval in the hour;

(g)	A Resource’s economic operating point, as calculated as described under Section 8.6.5(4)(d), for Resources that were committed following the close of the Day-Ahead Market if that Resource is off-line in the RTBM and that Resource was not de-committed by the Transmission Provider; and

(h)	The absolute value of a Resource’s URD if that Resource operated outside of its Operating Tolerance and the Resource has not been exempted from URD as described under Section 6.4.1.1 of this Attachment AE.

Proposed Criteria Language Revision

N/A

Note that this charge type will almost always produce a charge.  The credit is included here for the rare occasion when a credit may be produced as a result of a data error and/or a resettlement.