EIS Market Integrated Marketplace

PRR Comments

| PRR No. | Marketplace-PRR49 | PRR Title | RUC MWP Cost Allocation Changes |
| --- | --- | --- | --- |
| Date |  | 1/19/2012 |  |
| Submitter |  |  |  |
| Name |  | Ken Donald |  |
| E-mail Address |  | kdonald@utilicast.com |  |
| Company |  | Utilicast |  |
| Phone Number |  | 727.259.3447 |  |

Comments

4.5.9.8		RUC Make-Whole-Payment Amount (section heading in the actual MPRR is mislabeled)

Integration limits in Incremental Energy Cost

It is desired to limit the calculation to only costs representing increased output from minimum output, as is the normal case when Resources are committed in a RUC process and dispatched into their operating range, as illustrated by the pink shaded area in the diagram to the right.  If the calculation were to take into account a reduction in cost when the Resource output is below the level at which it was committed, as illustrated by the red dashed line and blue shaded area in the diagram to the right, the possibility – albeit remote – that the Resource could manipulate its offer curve to diminish its revenue by an incremental amount greater than the reduction in cost, thereby attenuating its Make-Whole Payment while restricting supply.  For example, if a resource is committed on a flat offer curve of $50 / MW and its minimum output is 100 MW, the Minimum Energy component of cost is $5000.00.  If it is not being dispatched above minimum output and not setting price, say LMP = $30, its Make-Whole payment would be $2000 (assuming there are no Start-Up, No-Load or other cost / revenue components to complicate this example, cost = $5000 and revenue = $3000).  If backward integration of the As-Dispatched Offer curve is allowed, the Resource could change its status to manual and lower its offer curve to $10 to forgo some cost and revenue and increase it MWP.  If the Resource diminishes its output below the economic point on the new offer curve it will not experience any clawback in the MWP calculation.  The reduction in revenue at $30/MW and cost at $10/MW translates to an increase in MWP of $20/MW while the Resource also saves real production cost.

By defining the integration limits in such a way that integration always captures positive cost (from left to right on the offer curve) this scenario is avoided.  Since Incremental Energy Cost and Desired Energy Cost are meant to be compared to determine if the Resource’s cost at its metered output exceeds its cost at the economic point as determined by the RTBM LMP, and thus may be subject to a clawback, it is essential that the limit definition be applied to the Desired Energy Cost as well.

4.5.9.9		Real-Time Out-Of-Merit Amount

Establishing separate determinants for the measurement of Incremental Energy Cost and Desired Energy Cost for Resources given manual OOM instructions

In the previous section we saw that the integration limits for Incremental Energy Cost and Desired Energy Cost can be defined by the As-Committed Regulating or Economic, as appropriate, minimum – which is necessary because the calculation needs to pick up on the As-Dispatched Offer curve at the same quantity where the integration of Minimum Energy from the As-Committed Offer curve left off.  Resources to which the OOM charge type applies may not have been committed in a RUC process as those eligible for MWP in the previous section were – the determinant defined as the lower limit may not exist for them.  To relieve this over-constrained calculation new and separate determinants are created (by adding a ‘Disp’ segment to the names) which are based on integration limits from the As-Dispatched offer data only.  The changes may appear to be substantial, but they are mere clones of the existing Incremental Energy Cost and Desired Energy Cost formulae, but using different integration starting point.

4.5.9.10		RUC Make-Whole Payment Distribution Amount

Usage of Set-Point flags in Min / Max Limit & Self-Commit deviation sections

The existing protocols call for determinants which define when a Resource received a Set-Point instruction equal to its minimum or maximum limit as an indication that the limits are restricting economics – that the market would prefer to have more or less output from the Resource.  It has been brought to the attention of SPP that while in Start-Up or on manual control Resources - especially VERs and NDVERs - would be frequently found to be deviant by this measure simply as a result of the echo-back nature of the Set-Point instruction.  SPP proposes a slightly different measure of this uneconomic behavior to rectify any inequity caused by consideration of an echoed Set-Point instruction.  The Desired Dispatch flag measures the point as determined by the RBM LMP on the Resource’s As-Dispatched Offer curve with respect to the As-Dispatched minimum limit.  If in any 5-minute interval within the hour the economic point is below the minimum or above the maximum the hour is flagged for deviation calculation.

Change usage of Desired Energy quantity in Status deviation section

Similar to the changes in 4.5.9.8 & 4.5.9.9 to support discrete determinant based on As-Dispatched minimum and maximum boundaries… when a Resource clears in the Day-Ahead Market, but is on Manual control in Real-Time its deviation will be measured as the difference between output and an economic level bound by the As-Dispatched min and max rather than the As-Committed values.

Additional exemption flag in Self-Commit deviation section

The existing protocols do not address the exception criteria described in this section - “Resources that were offered into the DA Market for SPP commitment and not committed in the DA Market and then Self-Committed prior to the Day-Ahead RUC are exempted from this calculation.”  Adding the exemption flag provides the ability for settlement to capture this scenario.

Revised Proposed Protocol Language Revision

#### RUC Make-Whole-Payment Distribution Amount

(4) The amount to each Asset Owner (AO) for each eligible Resource Settlement Location for a given RUC Make-Whole Payment Eligibility Period is calculated as follows:

#RtMwpCpAmt a, s, c =  ( CncldStartAmt a, s, c

+ Max (0, ( { ( IF ( CncldStartRatio a, s, c = 0, THEN 1, ELSE 0) }

* ( { RtStartUpElig5minFlg a, s, i, c  * RtStartUp5minAmt a, s, i, c

+ RtRucComStat5minFlg a, s, i, c * [ RtMwpCost5minAmt a, s, i, c

+ RtMwpRev5minAmt a, s, i, c

+ RtOom5minAmt a, s, i + RtRegAdj5minAmt a, s, i

–  RtURDAdj5minAmt a, s, i, c – RtStatusAdj5minAmt a, s, i, c

–  RtLimitAdj5minAmt a, s, i, c  ] } ) ) ) * (-1)

Where,

(a)	#RtMwpCost5minAmt a, s, i, c =  RtRucComStat5minFlg a, s, i, c  *

( RtIncrEn5minAmt a, s, i

+  Max ( 0, [ RtNoLoad5minAmt a, s, i, c

- IF (DaClrdHrlyQty a, s, h < 0, THEN DaNoLoadHrlyAmt a, s, h, c , ELSE 0 ) ] )

+ RtMinEn5minAmt a, s, i, c

+  RtRegUpAvail5minAmt a, s, i, c +  RtRegDnAvail5minAmt a, s, i, c

+  RtSpinAvail5minAmt a, s, i, c +  RtSuppAvail5minAmt a, s, i, c ) / 12

(a.1)	IF ABS (DaClrdHrlyQty a, s, h ) > = ABS ( RtBillMtr5minQty a, s, i )

THEN

RtIncrEn5minAmt a, s, i = 0

ELSE

#RtIncrEn5minAmt a, s, i =

Where:

X = Max (ABS (DaClrdHrlyQty a, s, h ), RtEffMin5minQty a, s, i )

AND

IF ControlStatus5minFlg a, s, i  = “Regulating”

THEN

RtEffMin5minQty a, s, i = Min (

RtComMinRegCapOL5minQtya, s, i ,

RtDispMinRegCapOL5minQtya, s, i ,

Max (0, (-1) * RtBillMtr5minQtya, s, i )

ELSE

RtEffMin5minQty a, s, i = Min (

RtComMinEconCapOL5minQtya, s, i ,

RtDispMinEconCapOL5minQtya, s, i ,

Max (0, (-1) * RtBillMtr5minQtya, s, i )

AND

Y = Max ( (-1) * RtBillMtr5minQtya, s, i , 0)

(a.2)	IF ABS (DaClrdHrlyQty a, s, h ) > 0

THEN

RtMinEn5minAmt a, s, i, c = 0

ELSE

# RtMinEn5minAmt a, s, i, c =

(a.3)	If RtRegUp5minQty a, s, i  > RtFixedRegUp5minQty a, s, i

THEN

RtRegUpAvail5minAmt a, s, i, c =

Max ( 0, [ RtRegUp5minQty a, z, s, i   - DaRegUpHrlyQty a, z, s, h] )

* RtRegUpOffer a, s, i, c

ELSE

RtRegUp5minAmt a, s, i  =0

(a.4)	If RtRegDn5minQty a, s, i  > RtFixedRegDn5minQty a, s, i

THEN

RtRegDnAvail5minAmt a, s, i, c =

Max ( 0, [ RtRegDn5minQty a, z, s, i   - DaRegDnHrlyQty a, z, s, h] )

* RtRegDnOffer a, s, i, c

ELSE

RtRegDn5minAmt a, s, i  =0

(a.5)	If RtSpin5minQty a, s, i  > RtFixedSpin5minQty a, s, i

THEN

RtSpinAvail5minAmt a, s, i, c =

Max ( 0, [ RtSpin5minQty a, z, s, i   - DaSpinHrlyQty a, z, s, h] )

* RtSpinOffer a, s, i, c

ELSE

RtSpin5minAmt a, s, i  =0

(a.6)	If RtSupp5minQty a, s, i  > RtFixedSupp5minQty a, s, i

THEN

RtSuppAvail5minAmt a, s, i, c =

Max ( 0, [ RtSupp5minQty a, z, s, i   - DaSuppHrlyQty a, z, s, h] )

* RtSuppOffer a, s, i, c

ELSE

RtSupp5minAmt a, s, i  =0

(b)	#RtMwpRev5minAmt a, s, i, c =

RtRucComStat5minFlg a, s, i, c  * [ ( ( RtLmp5minPrc s, i

* Min (0, [ RtBillMtr5minQty a, s, i - DaClrdHrlyQty a, s, h ] ) ) / 12 )

+ RtRegUpRev5minAmt a, s, i, c + RtRegDnRev5minAmt a, s, i, c

+ RtSpinRev5minAmt a, s, i, c + RtSuppRev5minAmt a, s, i, c ]

(b.1)	RtRegUpRev5minAmt a, s, i, c =

(-1) * RtRucComStat5minFlg a, s, i, c

* ( Max ( 0, [ RtRegUp5minQty a, z, s, i   - DaRegUpHrlyQty a, z, s, h] )

* RtRegUpMcp5minPrc z, i  ) / 12

(b.2)	RtRegDnRev5minAmt a, s, i, c =

(-1) * RtRucComStat5minFlg a, s, i, c

*( Max ( 0, [ RtRegDn5minQty a, z, s, i  - DaRegDnHrlyQty a, z, s, h] )

* RtRegDnMcp5minPrc z, i   ) / 12

(b.3)	RtSpinRev5minAmt a, s, i, c =

(-1) * RtRucComStat5minFlg a, s, i, c

*( Max ( 0, [ RtSpin5minQty a, z, s, i   - DaSpinHrlyQty a, z, s, h ] )

* RtSpinMcp5minPrc z, i ) / 12

(b.4)	RtSuppRev5minAmt a, s, i, c =

(-1) * RtRucComStat5minFlg a, s, i, c

*( Max ( 0, [ RtSupp5minQty a, z, s, i   - DaSuppHrlyQty a, z, s, h ] )

* RtSuppMcp5minPrc z, i   ) / 12

(c)	#CncldStartAmt a, s, c =

( RtStartUp5minAmt a, s, i, c  * RtStartUpElig5minFlg a, s, i, c )

* CncldStartRatio a, s, c

CncldStartRatio a, s, c = (ElapsedTime a, s, c  / StartUpTime a, s, c )

(d) In any Dispatch Interval in which the Resource has operated outside of its Operating Tolerance and that Resource has not been exempted from URD per Section 4.4.4.1, any incremental Energy costs associated with actual Energy output above the Resource’s Desired Dispatch is not eligible for recovery.  The URD adjustment is calculated as follows:

IF ABS (URD5minQty a, s, i ) > ResOpTol5minQty a, s, i AND

( XmptDev5minFlg a, s, i =  0 )

THEN

#RtURDAdj5minAmt a, s, i, c = RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i ) / 12

ELSE

RtURDAdj5minAmt a, s, i, c = 0

(d.1)	URD5minQty a, s, i =

( RtBillMtr5minQty a, s, i * (-1)) - RtAvgSetPoint5minQty a, s, i

(d.2)	ResOpTol5minQty a, s, i =

Min ( URDMaxTol5minQty a, s, i , Max (URDMinTol5minQty a, s, i ,

URDTol5minPct a, s, i * RtDispMaxEmerCapOL5minQty a, s, i ) )

(d.3)	IF RtDesiredEn5minQty a, s, i < ABS (DaClrdHrlyQty a, s, h )

THEN

#RtDesiredEn5minAmt a, s, i = RtIncrEn5minAmt a, s, i

ELSE

#RtDesiredEn5minAmt a, s, i =

Where:

X = Max (ABS (DaClrdHrlyQty a,  s, h ) , RtEffMin5minQty a, s, i )

Y = RtDesiredEn5minQtya, s, i

(e) In any Dispatch Interval in which a Resource is in “Manual” status, any incremental Energy costs associated with actual Energy output above the Resource’s Desired Dispatch is not eligible for recovery.  The status change adjustment is calculated as follows:

IF ControlStatus5minFlg a, s, i = “Manual”

AND ABS (URD5minQty a, s, i ) <= ResOpTol5minQty a, s, i

THEN

#RtStatusAdj5minAmt a, s, i, c = RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i ) / 12

ELSE

RtStatusAdj5minAmt a, s, i, c = 0

(f)	In any Dispatch Interval in which a Resource has increased its Minimum Economic Capacity Operating Limit (or its Minimum Regulation Capacity Operating Limit if the Resource has cleared for Regulation-Up or Regulation-Down) above the Resource’s minimum limits used by SPP in the commitment decision or the minimum limits used to move from one configuration to another in the case of a Combined Cycle Resource, the Resource is not in “Manual” status and the increase in minimum limit is greater than the Resource’s Operating Tolerance, any incremental Energy costs associated with actual Energy output above the Resource’s Desired Dispatch is not eligible for recovery.  The limit change adjustment is calculated as follows:

IF ControlStatus5minFlg a, s, i < > “Regulating” AND

ControlStatus5minFlg a, s, i  < > “Manual”  AND

( RtDispMinEconCapOL5minQty a, s, i

- RtComMinEconCapOL5minQty a, s, i ) > ResOpTol5minQty a, s, i  AND

ABS (URD5minQty a, s, i ) <= ResOpTol5minQty a, s, i

THEN

#RtLimitAdj5minAmt a, s, i, c =   RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i ) / 12

ELSE IF

ControlStatus5minFlg a, s, i  = “Regulating”  AND

( RtDispMinRegCapOL5minQty a, s, i

- RtComMinRegCapOL5minQty a, s, i ) > ResOpTol5minQty a, s, i  AND

ABS (URD5minQty a, s, i ) < =ResOpTol5minQty a, s, i

THEN

#RtLimitAdj5minAmt a, s, i, c =  RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i ) / 12

ELSE

RtLimitAdj5minAmt a, s, i, c = 0

- For each Asset Owner, a daily amount is calculated at each Settlement Location.  The daily amount is calculated as follows:

RtMwpDlyAmt a, s, d =   RtMwpCpAmt a, s, c

- For each Asset Owner associated with Market Participant m, a daily amount is calculated.  The daily amount is calculated as follows:

RtMwpAoAmt a, m, d =  RtMwpDlyAmt a, s, d

- For each Market Participant, a daily amount is calculated representing the sum of Asset Owner amounts associated with that Market Participant.  The daily amount is calculated as follows:

RtMwpMpAmt m, d =  RtMwpAoAmt a, m, d

(8)	For FERC Electric Quarterly Reporting (“EQR”) purposes, SPP calculates RUC Make-Whole Payment $ per RUC Make-Whole-Payment Eligibility Period for each Asset Owner as follows:

(a)	EqrRtMwp5minPrc a, s, c = (-1) * RtMwpCpAmt a, s, c

(b)	IF EqrRtMwp5minPrc a, s, c > 0

THEN

EqrRtMwp5minQty a, s, c = 1

The above variables are defined as follows:

| Variable | Unit | Settlement Interval | Definition |
| --- | --- | --- | --- |
| RtMwpCpAmt a, s, c | $ | Eligibility Period | RUC Make-Whole-Payment Amount per AO per Settlement Location per RUC Make-Whole-Payment Eligibility Period - The amount to AO a for RUC Make-Whole-Payment Eligibility Period c at Resource Settlement Location s.. |
| DaClrdHrlyQty a, s, h | MWh | Hour | Day-Ahead Cleared Energy Quantity per AO per Settlement Location per Hour - The value described under Section 4.5.8.1 for AO a’s combined cycle resource at Settlement Location s for the Hour. |
| RtStartUp5minAmt a s, i, c | $ | Eligibility Period | Real-Time Start-Up Cost Amount per AO per Settlement Location per Dispatch Interval per RUC Make-Whole-Payment Eligibility Period - The RTBM Start-Up Offer associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. This value is calculated by dividing RtStartUpAmt a s, c by the lesser of the Resource’s (RtMinRunTime a, i, s, c *12) or (24 * 12). These interval values are carried forward into the following Operating Day, if needed, to ensure recovery of any remaining RtStartUpAmt a s, c. |
| RtStartUpAmt a s, c (Not Available on Settlement Statement) | $ | Eligibility Period | Real-Time Start-Up Cost Amount per AO per Settlement Location per RUC Make-Whole-Payment Eligibility Period - The RTBM Start-Up Offer used in the commitment decision, associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c. |
| RtStartUpElig5minFlg a, s, i, c | None | Dispatch Interval | RUC Start-Up Recovery Eligibility Flag per AO per Resource Settlement Location per Dispatch Interval per RUC Make-Whole-Payment Eligibility Period – This flag is set equal to 1 in each Dispatch Interval of a RUC Make-Whole-Payment Eligibility Period where the Resource is eligible to recover start-up costs, or 0 where the Resource is not eligible to recover start-up costs. |
| RtRucComStat5minFlg a, s, i, c | None | Dispatch Interval | RUC Commitment Status Flag per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – This flag is set equal to 1 for each Dispatch Interval of a RUC Make-Whole-Payment Eligibility Period in which a Resource’s Commitment Status was “Market” or “Reliability”, or 0 if its Commitment Status was “Self”. |
| CncldStartRatio a, s, c | None |  | Canceled Start Ratio per Resource Settlement Location in RUC Make-Whole-Payment Eligibility Period – The ratio of ElapsedTime a, s, c to StartUpTime a, s, c as calculated for each Dispatch Interval in RUC Make-Whole-Payment Eligibility Period c. |
| RtMinRunTime a, i, s, c (Not Available on Settlement Statement) | Time | Dispatch Interval | Real-Time Minimum Run Time per AO per Settlement Location Per Dispatch Interval per RUC Make-Whole-Payment Eligibility Period – The Minimum Run Time used in the commitment decision, associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c as submitted as part of the RTBM Market Offer. |
| RtNoLoad5minAmt a, i, s, c | $ | Dispatch Interval | Real-Time No-Load Cost Amount per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period - The No-Load Offer used in the commitment decision, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtMwpCost5minAmt a, s, i, c | $ | Dispatch Interval | RUC Make-Whole-Payment Cost per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period – The total Energy and Operating Reserve cost at actual Resource output, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtMwpRev5minAmt a, s, i, c | $ | Dispatch Interval | RUC Make-Whole-Payment Revenue per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period – The total Energy and Operating Reserve revenue at actual Resource output, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| CncldStartAmt a, s, c | $ | Eligibility Period | Real-Time Cancelled Start Amount per AO per Settlement Location per for the RUC Make-Whole-Payment Eligibility Period – The Start-Up Offer cost reimbursement for an SPP cancelled start-up, in dollars, associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c. |
| ElapsedTime a, s, c | Time | Eligibility Period | Elapsed Time per AO per Settlement Location per for the RUC Make-Whole-Payment Eligibility Period – The elapsed time, in minutes, between the start of a Resource’s StartUpTime a, s, c and the time SPP cancelled the start-up, in dollars, associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c. |
| StartUpTime a, s, c | Time | Eligibility Period | Start-up Time per AO per Settlement Location for the RUC Make-Whole-Payment Eligibility Period – The Start-Up Time, in minutes, used in the commitment decision associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c as specified in the RTBM Offer submitted prior to the RUC Make-Whole-Payment Eligibility Period. |
| RtURDAdj5minAmt a, s, i, c | $ | Dispatch Interval | URD Adjustment per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period – The reduction in RUC Make-Whole Payment Amount associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c when the Resource’s URD5minQty a, s, i is outside of the Resource’s ResOpTol5minQty a, s, i. |
| URD5minQty a, s, i | MW | Dispatch Interval | Uninstructed Resource Deviation per AO per Settlement Location per Dispatch Interval – The Uninstructed Resource Deviation associated with AO a’s Resource at Settlement Location s in Dispatch Interval i. |
| ResOpTol5minQty a, s, i | MW | Dispatch Interval | Resource Operating Tolerance per AO per Settlement Location per Dispatch Interval – The Resource Operating Tolerance associated with AO a’s Resource at Settlement Location s in Dispatch Interval i. |
| URDMaxTol5minQty a, s, i | MW | Dispatch Interval | Uninstructed Resource Deviation Maximum Tolerance per AO per Settlement Location per Dispatch Interval – The maximum value of ResOpTol5minQty a, s, i that is currently set at 20 MW. |
| URDMinTol5minQty a, s, i | MW | Dispatch Interval | Uninstructed Resource Deviation Minimum Tolerance per AO per Settlement Location per Dispatch Interval – The minimum value of ResOpTol5minQty a, s, i that is currently set at 5 MW. |
| URDTol5minPct a, s, i | Percent | Dispatch Interval | Uninstructed Resource Deviation Tolerance Percentage per AO per Settlement Location per Dispatch Interval – The percentage used to calculate the value of ResOpTol5minQty a, s, i that is currently set at 5%. |
| RtAvgSetPoint5minQty a, s, i | MW | Dispatch Interval | Real-Time Average Setpoint Instruction MW per AO per Settlement Location per Dispatch Interval – The average Setpoint Instruction over Dispatch Interval i for AO a’s Resource at Settlement Location s. |
| XmptDev5minFlg a, s, i | none | Dispatch Interval | URD Exemption Flag per AO per Resource Settlement Location per Dispatch Interval – A flag associated with AO a’s eligible Resource at Settlement Location s indicating that a Resource that has operated outside of its Operating Tolerance is or is not exempt from any associated penalty charges in Dispatch Interval i. If the flag is equal to zero, the Resource is not exempt. Otherwise, the flag will be set to a positive integer number which will indicate the reason of the exemption as specified under Section 4.4.4.1. |
| RtStatusAdj5minAmt a, s, i, c | $ | Dispatch Interval | Resource Status Change Adjustment per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period – The reduction in RUC Make-Whole Payment Amount associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c when the Resource’s Control Status is set to “Manual”. |
| ControlStatus5minFlg a, s, i | None | Dispatch Interval | Control Status per AO per Settlement Location per Dispatch Interval – A Resource status indicator associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i as set by SPP operators that indicates the current dispatchable status of the Resource. |
| RtDispMaxEmerCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Maximum Emergency Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The Maximum Emergency Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i. |
| RtEffMin5minQty a, s, i | MW | Dispatch Interval | Real-Time Effective Minimum Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The Effective Minimum Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i. |
| RtDispMinEconCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Minimum Economic Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The Minimum Economic Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i. |
| RtDispMinRegCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Minimum Regulation Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The Minimum Regulation Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i. |
| RtLimitAdj5minAmt a, s, i, c | $ | Dispatch Interval | Resource Limit Change Adjustment per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period – The reduction in RUC Make-Whole Payment Amount associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c for a Real-Time increase in minimum limit. |
| RtComMinEconCapOL5minQty a, s, i | MW | Eligibility Period | Real-Time Minimum Economic Capacity Operating Limit Quantity per AO per Settlement Location – The Minimum Economic Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i as submitted in an RTBM Offer prior to the RUC Make-Whole-Payment Eligibility Period that was used in making the initial Resource commitment decision. |
| RtComMinRegCapOL5minQty a, s, i | MW | Eligibility Period | Real-Time Minimum Regulation Capacity Operating Limit Quantity per AO per Settlement Location– The Minimum Regulation Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i as submitted in an RTBM Offer prior to the RUC Make-Whole-Payment Eligibility Period that was used in making the initial Resource commitment decision. |
| RtIncrEn5minAmt a, s, i | $ | Dispatch Interval | Real-Time Incremental Energy Cost Amount per AO per Settlement Location per Dispatch Interval - The average incremental energy offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i from the Effective Minimum Capacity Operating Limit to to RtBillMtr5minQty a, s, i. |
| RtMinEn5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Energy Cost at Minimum Limit per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period - The average incremental energy offer cost at the Effective Minimum Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c |
| RtDesiredEn5minAmt a, s, i | $ | Dispatch Interval | Real-Time Energy Cost at Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval - The average incremental energy offer cost associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i, in dollars, from the Effective Minimum Capacity Operating Limit to RtDesiredEn5minQty a, s, i. |
| RtDesiredEn5minQty a, s, i | MW | Dispatch Interval | Real-Time Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval – The Desired Dispatch MW for AO a’s eligible Resource for Dispatch Interval i at RtLmp5minPrc s, i as calculated from the Resource’s As Dispatched Energy Offer Curve using the As-Committed Minimum Capacity Limit (Economic or Regulating, as applicable) as an output floor and the As-Committed Maximum Capacity Limit (Economic or Regulating, as applicable) as an output ceiling. |
| RtOom5minAmt a, s, i | $ | Dispatch Interval | Real-Time Out-Of-Merit Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The value calculated under Section 4.5.9.9. |
| RtRegAdj5minAmt a, s, i | $ | Dispatch Interval | Real-Time Regulation Deployment Adjustment Amount per AO per Resource Settlement Location per Dispatch Interval - The value calculated under Section 4.5.9.19. |
| RtRegUpOffer a, s, i, c (Not Available on Settlement Statement) | $/MW | Dispatch Interval | Real-Time Regulation-Up Offer per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Regulation-Up Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtRegDnOffer a, s, i, c (Not Available on Settlement Statement) | $/MW | Dispatch Interval | Real-Time Regulation-Down Offer per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Regulation-Down Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtSpinOffer a, s, i, c (Not Available on Settlement Statement) | $/MW | Dispatch Interval | Real-Time Spinning Reserve Offer per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Spinning Reserve Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtSuppOffer a, s, i, c (Not Available on Settlement Statement) | $/MW | Dispatch Interval | Real-Time Supplemental Reserve Offer per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Supplemental Reserve Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtFixedRegUp5minQty a, s, i (Not Available on Settlement Statement) | MW | Dispatch Interval | Real-Time Fixed Regulation-Up Quantity per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Fixed Regulation-Up MW specified in the Regulation-Up Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtFixedRegDn5minQty a, s, i (Not Available on Settlement Statement) | MW | Dispatch Interval | Real-Time Fixed Regulation-Down Quantity per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Fixed Regulation-Down MW specified in the Regulation-Down Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtFixedSpin5minQty a, s, i (Not Available on Settlement Statement) | MW | Dispatch Interval | Real-Time Fixed Spinning Reserve Quantity per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Fixed Spinning Reserve MW specified in the Spinning Reserve Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtFixedSupp5minQty a, s, i (Not Available on Settlement Statement) | MW | Dispatch Interval | Real-Time Fixed Supplemental Reserve Quantity per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Fixed Supplemental Reserve MW specified in the Supplemental Reserve Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtRegUpAvail5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Regulation-Up Offer Cost Amount per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period - The Regulation-Up Offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtRegDnAvail5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Regulation-Down Offer Cost Amount per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period - The Regulation-Down Offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in DA Market Commitment Period c. |
| RtSpinAvail5minAmt a, s, i, c | c | Dispatch Interval | Real-Time Spin Offer Cost Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period - The Spinning Reserve Offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtSuppAvail5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Supplemental Offer Cost Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period - The Supplemental Reserve Offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtLmp5minPrc s, i | $/MWh | Dispatch Interval | Real-Time LMP - The value defined under Section 4.5.9.1 at Settlement Location s for Dispatch Interval i. |
| RtBillMtr5minQty a, s, i | MW | Dispatch Interval | Real-Time Actual Meter Quantity per AO per Location per Dispatch Interval - The value defined under Section 4.5.9.1 for Dispatch Interval i. |
| RtRegUpRev5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Regulation-Up Revenue Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Real-Time incremental Regulation-Up revenue associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtRegDnRev5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Regulation-Down Revenue Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Real-Time incremental Regulation-Down revenue associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtSpinRev5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Spinning Reserve Revenue Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Real-Time incremental Spinning Reserve associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtSuppRev5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Supplemental Reserve Revenue Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Real-Time incremental Supplemental Reserve revenue associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtMwpDlyAmt a, s, d | $ | Operating Day | RUC Make-Whole-Payment Amount per AO per Settlement Location per Operating Day - The RUC Make-whole amount to AO a for Operating Day d at Resource Settlement Location s. |
| RtMwpAoAmt a, m, d | $ | Operating Day | RUC Make-Whole-Payment Amount per AO per Operating Day - The RUC Make-whole amount to AO a associated with Market Participant m for Operating Day d. |
| RtMwpMpAmt m, d | $ | Operating Day | RUC Make-Whole-Payment Amount per MP per Operating Day - The RUC Make-whole amount to Market Participant m for Operating Day d. |
| EqrRtMwp5minPrc a, s, c | $ | Eligibility Period | RUC Electric Quarterly Reporting Make-Whole-Payment Amount per AO per Settlement Location per RUC Make-Whole-Payment Eligibility Period - The RUC make-whole amount to AO a for RUC Make-Whole-Payment Eligibility Period c at Resource Settlement Location s for use by AO a in reporting such make-whole-payments to FERC in accordance with FERC EQR requirements.. |
| EqrRtMwp5minQty a, s, c | $ | Eligibility Period | RUC Electric Quarterly Reporting Make-Whole-Payment Quantity per AO per Settlement Location per RUC Make-Whole-Payment Eligibility Period – This value is set equal to 1 if EqrRtMwp5minPrc a, s, c > 0 for use by AO a in reporting such make-whole-payments to FERC in accordance with FERC EQR requirements.. |
| a | none | none | An Asset Owner. |
| i | none | none | A Dispatch Interval. |
| h | none | none | An Hour. |
| d |  |  | An Operating Day. |
| s | none | none | A Settlement Location. |
| c | none | none | A RUC Make-Whole-Payment Eligibility Period. |
| m | none | none | A Market Participant. |

#### Real-Time Out-Of-Merit Amount

The amount to each Asset Owner (AO) for each eligible Resource Settlement Location for each Dispatch Interval is calculated as follows:

#RtOom5minAmt a, s, i = RtOom5minFlg a, s, i

* ( RtOomeIncr5minAmt a, s, i + RtOomeDecr5minAmt a, s, i

+ RtOomor5minAmt a, s, i ) * (-1)

Where,

(a)		RtOomeIncr5minAmt a, s, i  =

Max ( 0, RtDispIncrEn5minAmt a, s, i – RtDispDesiredEn5minAmt a, s, i

+ Min ( 0, RtBillMtr5minQty a, s, i  + RtDispDesiredEn5minQty a, s, i )

* Max( 0, RtLmp5minPrc s, i ) ) / 12

(a.1)	#RtDispIncrEn5minAmt a, s, i =

Where:

IF ControlStatus5minFlg a, s, i  = “Regulating”

X = RtDispMinRegCapOL5minQtya, s, i

ELSE

X = RtDispMinEconCapOL5minQtya, s, i

AND

Y = Max ( X, Max ( (-1) * RtBillMtr5minQtya, s, i , 0) )

(a.2)	#RtDispDesiredEn5minAmt a, s, i =

Where:

IF ControlStatus5minFlg a, s, i  = “Regulating”

THEN

X = RtDispMinRegCapOL5minQtya, s, i

ELSE

X = RtDispMinEconCapOL5minQtya, s, i

AND

Y = Max ( X, RtDispDesiredEn5minQtya, s, i )

(b)		RtOomeDecr5minAmt a, s, i  =

Max (0, RtBillMtr5minQty a, s, i  - DaClrdHrlyQty a, s, h  )

* Max ( 0, RtLmp5minPrc s, i  - DaLmpHrlyPrc s, h  )  / 12

(c)		RtOomor5minAmt a, s, i  =

[ ( Max (0, DaRegUpHrlyQty a, z, s, h  - RtRegUp5minQty a, z, s, i )

* Max ( 0, RtRegUpMcp5minPrc z, i  - DaRegUpMcpHrlyPrc z, h  ) )

+ ( Max (0, DaRegDnHrlyQty a, z, s, h  - RtRegDn5minQty a, z, s, i )

* Max ( 0, RtRegDnMcp5minPrc z, i  - DaRegDnMcpHrlyPrc z, h  ) )

+ ( Max (0, DaSpinHrlyQty a, z, s, h  - RtSpin5minQty a, z, s, i )

* Max ( 0, RtSpinMcp5minPrc z, i  - DaSpinMcpHrlyPrc z, h  ) )

+ ( Max (0, DaSuppHrlyQty a, z, s, h  - RtSupp5minQty a, z, s, i )

* Max ( 0, RtSuppMcp5minPrc z, i  - DaSuppMcpHrlyPrc z, h  ) ) ] / 12

- For each Asset Owner, an hourly amount is calculated at each Settlement Location.  The hourly amount is calculated as follows:

RtOomHrlyAmt a, s, h = RtOom5minAmt a, s, i

- For each Asset Owner, a daily amount is calculated at each Settlement Location.  The daily credit amount is calculated as follows:

RtOomDlyAmt a, s, d =  RtOomHrlyAmt a, s, h

- For each Asset Owner associated with Market Participant m, a daily amount is calculated.  The daily amount is calculated as follows:

RtOomAoAmt a, m, d =  RtOomDlyAmt a, s, d

- For each Market Participant, a daily amount is calculated representing the sum of Asset Owner amounts associated with that Market Participant.  The daily amount is calculated as follows:

RtOomMpAmt m, d =  RtOomAoAmt a, m, d

(6)	For FERC Electric Quarterly Reporting (“EQR”) purposes, SPP calculates Real-Time Out-of-Merit Energy and Operating Reserve $ per Dispatch Interval for each Asset Owner as follows:

(a)	EqrRtOom5minPrc a, s, i = (-1) * RtOom5minAmt a, s, i

(b)	IF EqrRtOom5minPrc a, s, i > 0

THEN

EqrRtOom5minQty a, s, i = 1

The above variables are defined as follows:

| Variable | Unit | Settlement Interval | Definition |
| --- | --- | --- | --- |
| RtOom5minAmt a, s, i | $ | Dispatch Interval | Real-Time Out-Of-Merit Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The amount to AO a for eligible Resource Settlement Location s in Dispatch Interval i for Out-Of-Merit Energy and Operating Reserve resulting from an SPP Manual Dispatch Instruction. |
| RtOomeIncr5minAmt a, s, i | $ | Dispatch Interval | Real-Time Out-Of-Merit Incremental Energy Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The portion of AO a’s RtOome5minAmt a, s, i amount for eligible Resource Settlement Location s in Dispatch Interval i for Out-Of-Merit Energy resulting from an SPP manual Dispatch Instruction in the up direction. |
| RtOomeDecr5minAmt a, s, i | $ | Dispatch Interval | Real-Time Out-Of-Merit Decremental Energy Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The portion of AO a’s RtOome5minAmt a, s, i amount for eligible Resource Settlement Location s in Dispatch Interval i for Out-Of-Merit Energy resulting from an SPP manual Dispatch Instruction in the down direction. |
| RtOom5minFlg a, s, i | None | Dispatch Interval | Real-Time Out-of-Merit Flag per AO per Settlement Location per Dispatch Interval – A flag that is set equal to 1 when SPP issues a Manual Dispatch Instruction or whenever there is a price correction event as described under Section 7, otherwise, this flag is set equal to zero. |
| RtOomor5minAmt a, s, i | $ | Dispatch Interval | Real-Time Out-Of-Merit Operating Reserve Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The portion of AO a’s RtOome5minAmt a, s, i attributable to buying back a DA Market Operating Reserve position in the RTBM at a RTBM MCP that is greater than the corresponding DA Market MCP. This should not be a normal occurrence but could happen as a result of price corrections as described under Section 7. |
| RtDispDesiredEn5minQty a, s, i | MW | Dispatch Interval | Real-Time Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval – The Desired Dispatch MW for AO a’s eligible Resource for Dispatch Interval i at RtLmp5minPrc s, i as calculated from the Resource’s As Dispatched Energy Offer Curve using the As-Dispatched Minimum Capacity Limit (Economic or Regulating, as applicable) as an output floor and the As-Dispatched Maximum Capacity Limit (Economic or Regulating, as applicable) as an output ceiling. |
| RtDispIncrEn5minAmt a, s, i | $ | Dispatch Interval | Real-Time Incremental Energy Cost Amount per AO per Settlement Location per Dispatch Interval - The average incremental energy offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i as calculated from the Resource’s As Dispatched Energy Offer Curve from the As-Dispatched Minimum Capacity Limit (Economic or Regulating, as applicable) to the maximum of the As-Dispatched Minimum Capacity Limit or RtBillMtr5minQty a, s, i. |
| RtDispDesiredEn5minAmt a, s, i | $ | Dispatch Interval | Real-Time Energy Cost at Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval - The average incremental energy offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i as calculated from the Resource’s As Dispatched Energy Offer Curve from the As-Dispatched Minimum Capacity Limit (Economic or Regulating, as applicable) to the maximum of the As-Dispatched Minimum Capacity Limit or RtDispDesiredEn5minQty a, s, i. |
| RtBillMtr5minQty a, s, i | MW | Dispatch Interval | Real-Time Actual Meter Quantity per AO per Location per Dispatch Interval - The value defined under Section 4.5.9.1 for Dispatch Interval i. |
| RtLmp5minPrc s, i | $/MW | Dispatch Interval | Real-Time LMP - The value defined under Section 4.5.9.1 at Settlement Location s for Dispatch Interval i. |
| DaClrdHrlyQty a, s, h | MWh | Hour | Day-Ahead Cleared Energy Quantity per AO per Settlement Location per Hour in the DA Market – The value described under Section 4.5.8.1. |
| DaRegUpHrlyQty a, z, s, h | MW | Hour | Day-Ahead Regulation-Up Quantity per AO per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.4. |
| DaRegDnHrlyQty a, z, s, h | MW | Hour | Day-Ahead Regulation-Down Quantity per AO per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.5. |
| DaSpinHrlyQty a, z, s, h | MW | Hour | Day-Ahead Spinning Reserve Quantity per AO per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.6. |
| DaSuppHrlyQty a, z, s, h | MW | Hour | Day-Ahead Spinning Reserve Quantity per AO per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.7. |
| RtRegUp5minQty a, z, s, i | MW | Dispatch Interval | Real-Time Regulation-Up Quantity per AO per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.4. |
| RtRegDn5minQty a, z, s, i | MW | Dispatch Interval | Real-Time Regulation-Down Quantity per AO per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.5. |
| RtSpin5minQty a, z, s, i | MW | Dispatch Interval | Real-Time Spinning Reserve Quantity per AO per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.6. |
| RtSupp5minQty a, z, s, i | MW | Dispatch Interval | Real-Time Supplemental Reserve Quantity per AO per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.7. |
| DaRegUpMcpHrlyPrc z, h | $/MW | Hour | Day-Ahead Regulation-Up Market Clearing Price per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.4. |
| DaRegDnMcpHrlyPrc z, h | $/MW | Hour | Day-Ahead Regulation-Down Market Clearing Price per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.5. |
| DaSpinMcpHrlyPrc z, h | $/MW | Hour | Day-Ahead Spinning Reserve Market Clearing Price per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.6. |
| DaSuppMcpHrlyPrc z, h | $/MW | Hour | Day-Ahead Spinning Reserve Market Clearing Price per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.7. |
| RtRegUpMcp5minPrc z, i | $/MW | Dispatch Interval | Real-Time Regulation-Up Market Clearing Price per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.4. |
| RtRegDnMcp5minPrc z, i | $/MW | Dispatch Interval | Real-Time Regulation-Down Market Clearing Price per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.5. |
| RtSpinMcp5minPrc z, i | $/MW | Dispatch Interval | Real-Time Spinning Reserve Market Clearing Price per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.6. |
| RtSuppMcp5minPrc z, i | $/MW | Dispatch Interval | Real-Time Supplemental Reserve Market Clearing Price per Settlement Location per Dispatch Interval in the RTBM– The value described under Section 4.5.9.7. |
| RtOomHrlyAmt a, s, h | $ | Hour | Real-Time Out-Of-Merit Make-Whole-Payment Amount per AO per Settlement Location per Hour - The amount to AO a for eligible Resource Settlement Location s in Hour h for Out-Of-Merit Energy and Operating Reserve resulting from an SPP Manual Dispatch Instruction. |
| RtOomDlyAmt a, s, d | $ | Operating Day | Real-Time Out-Of-Merit Make-Whole-Payment Amount per AO per Settlement Location per Operating Day - The amount to AO a for eligible Resource Settlement Location s in Operating Day d for Out-Of-Merit Energy and Operating Reserve resulting from an SPP Manual Dispatch Instruction. |
| RtOomAoAmt a, m, d | $ | Operating Day | Real-Time Out-Of-Merit Make-Whole-Payment Amount per AO per Operating Day - The amount to AO a associated with Market Participant m in Operating Day d for Out-Of-Merit Energy and Operating Reserve resulting from an SPP Manual Dispatch Instruction. |
| RtOomMpAmt m, d | $ | Operating Day | Real-Time Out-Of-Merit Make-Whole-Payment Amount per MP per Operating Day - The amount to MP m in Operating Day d for Out-Of-Merit Energy and Operating Reserve resulting from an SPP Manual Dispatch Instruction. |
| EqrRtOom5minPrc a, s, i | $ | Dispatch Interval | Real-Time Electric Quarterly Reporting Out-of-Merit Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The Out-of-Merit make-whole amount to AO a for Dispatch Interval i at Resource Settlement Location s for use by AO a in reporting such make-whole-payments to FERC in accordance with FERC EQR requirements. |
| EqrRtOom5minQty a, s, i | $ | Dispatch Interval | Real-Time Electric Quarterly Reporting Out-of-Merit Make-Whole-Payment Quantity per AO per Settlement Location per Dispatch Interval – This value is set equal to 1 if EqrRtOom5minPrc a, s, i > 0 for use by AO a in reporting such make-whole-payments to FERC in accordance with FERC EQR requirements.. |
| a | none | none | An Asset Owner. |
| s | none | none | A Settlement Location. |
| i | none | none | A Dispatch Interval. |
| h | none | none | An Hour. |
| d | none | none | An Operating Day. |
| m | none | none | A Market Participant. |

#### 4.5.9.10	RUC Make-Whole-Payment Distribution Amount

- A RTBM charge or credit will be calculated at each Settlement Location for each Asset Owner for each hour in order to fund the payments made under Section 4.5.9.8.  The amount will be determined by multiplying the Asset Owner deviations by a daily RTBM MWP rate.

The hourly amount is calculated as follows:

#RtMwpDistHrlyAmt a, s, h =  RtMwpSppDistRate d *  RtDevHrlyQty a, s, h

Where,

(a)	RtDevHrlyQty a, s, h =  RtNetSlDevHrlyQty a, s, h + RtMinLimitDevHrlyQty a, s, h

+ RtMaxLimitDevHrlyQty a, s, h+ RtOutageDevHrlyQty a, s, h

+ RtStatusDevHrlyQty a, s, h + RtRucScDevHrlyQty a, s, h

+ RtRucCommitDevHrlyQty a, s, h + RtURDDevHrlyQty a, s, h

(a.1)	An Asset Owner’s Settlement Location deviation is calculated as the Absolute Value of the sum of (1) (RTBM actual load MWh - DA Market cleared load MWh), (2) (RTBM actual Export Interchange Transactions – DA Market cleared Export Interchange Transactions), (3) (RTBM actual Import Interchange Transactions – DA Market cleared Import Interchange Transactions), (4) (RTBM actual Through Interchange Transactions (sink only) – DA Market cleared Through Interchange Transactions (sink only)), (5) DA Market cleared Virtual Energy Bids * (-1), and (6) DA Market cleared Virtual Energy Offers * (-1).  An Asset Owner’s Settlement Location deviation is calculated as follows.

RtNetSlDevHrlyQty a, s, h = ABS RtNetSlDev5minQty a, s, i

#RtNetSlDev5minQty a, s, i =

{ Max ( 0, RtBillMtr5minQty a, s, i   )  – Max ( 0, DaClrdHrlyQty a, s, h )

+ [Max ( 0, RtImpExp5minQty a, s, i, t, dir  )

- Max ( 0, DaImpExp5minQty a, s, i, t, dir  )

+ [ IF DIR <> “THROUGH”, THEN

Min ( 0, RtImpExp5minQty a, s, i, t, dir  )

- Min ( 0, DaImpExp5minQty a, s, i, t, dir  ), ELSE 0 ] ]

* (1 – RsgCrdFlgt ) - DaClrdVHrlyQty a, s, h, t } / 12

(a.2)	For a Resource with DA Market cleared MW in an hour, if the Resource’s Minimum Economic Capacity Operating Limit (or Minimum Regulation Capacity Operating Limit if cleared for Regulation-Up or Regulation-Down) in the RTBM is (1) greater than the comparable limits used to clear the Resource in the DA Market by more than the Resource Operating Tolerance; (2) is greater than its DA Market cleared MW; and (3) the Resource’s Desired Dispatch in any Dispatch Interval within the Hour is less than or equal to the Resource’s applicable minimum limit, the difference between the Resource’s applicable minimum limit and its DA Market cleared MW is included as a deviation.  In the case where the Resource has cleared Regulation-Up or Regulation Down in the RTBM and has not cleared Regulation-Up or Regulation Down in the DA Market, the deviation is the lesser of the (1) the difference between the Resource’s RTBM regulation minimum limit and its DA Market cleared MW or (2) the difference between the Resource’s RTBM regulation minimum limit the its DA Market regulation minimum limit.

RtMinLimitDevHrlyQty a, s, h  =   RtMinLimitDev5minQty a, s, i

Where,

IF DesiredDispatchMinHrlyFlg a, s, h= “1” AND DaClrdHrlyQty a, s, h  < 0

AND RtRucComStat5minFlg a, s, i, c < > “1”

AND RtRucComStat5minFlg a, s, i, c < > “0”

THEN

** Regulation is not cleared in RTBM **

IF ControlStatus5minFlg a, s, i <> “Regulating” AND

( RtDispMinEconCapOL5minQty a, s, i - DaComMinEconCapOLHrlyQty a, s, h)

> ResOpTol5minQty a, s, i

THEN

#RtMinLimitDev5minQty a, s, i =

Max [ ( RtDispMinEconCapOL5minQty a, s, i +  DaClrdHrlyQty a, s, h ), 0 ] / 12

ELSE IF

** Regulation is cleared in both DA Market and RTBM **

IF ControlStatus5minFlg a, s, i = “Regulating” AND

DaRegUpHrlyQty a, z, s, h + DaRegDnHrlyQty a, z, s, h > 0 AND

( RtDispMinRegCapOL5minQty a, s, i - DaComMinRegCapOLHrlyQty a, s, h)

> ResOpTol5minQty a, s, i

THEN

#RtMinLimitDev5minQty a, s, i  =

Max [ ( RtDispMinRegCapOL5minQty a, s, i +  DaClrdHrlyQty a, s, h ), 0 ] / 12

ELSE IF

** Regulation is cleared in RTBM and not cleared in DA Market **

IF ControlStatus5minFlg a, s, i = “Regulating” AND

DaRegUpHrlyQty a, z, s, h + DaRegDnHrlyQty a, z, s, h = 0 AND

( RtDispMinRegCapOL5minQty a, s, i - DaComMinRegCapOLHrlyQty a, s, h)

> ResOpTol5minQty a, s, i

THEN

#RtMinLimitDev5minQty a, s, i  =

Max { RtDispMinRegCapOL5minQty a, s, i

– Max [ ABS ( DaClrdHrlyQty a, s, h ) , DaComMinRegCapOLHrlyQty a, s, h ] , 0 } / 12

ELSE

RtMinLimitDev5minQty a, s, i  = 0

(a.3)	For a Resource with DA Market cleared MW in an hour, if the Resource’s Maximum Economic Capacity Operating Limit (or Maximum Regulation Capacity Operating Limit if cleared for Regulation-Up or Regulation-Down) in the RTBM is (1) less than the comparable limits used to clear the Resource in the DA Market by more than the Resource Operating Tolerance; (2) is less than its DA Market cleared MW; and (3) the Resource’s Desired Dispatch in any Dispatch Interval within the Hour is greater than or equal to the Resource’s applicable maximum limit, the difference between the Resource’s DA Market cleared MW and its applicable maximum limit is included as a deviation.  In the case where the Resource has cleared Regulation-Up or Regulation Down in the RTBM and has not cleared Regulation-Up or Regulation Down in the DA Market, the deviation is the lesser of the (1) the difference between the Resource’s DA Market cleared MW and its RTBM regulation maximum limit or (2) the difference between the Resource’s DA Market regulation minimum limit and its RTBM regulation minimum limit.

RtMaxLimitDevHrlyQty a, s, h  =   RtMaxLimitDev5minQty a, s, i

Where,

IF DesiredDispatchMaxHrlyFlg a, s, h= “1” AND DaClrdHrlyQty a, s, h  < 0

THEN

** Regulation is not cleared in RTBM **

IF ControlStatus5minFlg a, s, i <> “Regulating” AND

( DaComMaxEconCapOLHrlyQty a, s, h - RtDispMaxEconCapOL5minQty a, s, i)

> ResOpTol5minQty a, s, i

THEN

#RtMaxLimitDev5minQty a, s, i  =

Max [ (ABS ( DaClrdHrlyQty a, s, h ) - RtDispMaxEconCapOL5minQty a, s, i), 0 ] / 12

ELSE IF

** Regulation is cleared in both DA Market and RTBM **

IF ControlStatus5minFlg a, s, i = “Regulating” AND

DaRegUpHrlyQty a, z, s, h + DaRegDnHrlyQty a, z, s, h > 0 AND

( DaComMaxRegCapOLHrlyQty a, s, h - RtDispMaxRegCapOL5minQty a, s, i )

> ResOpTol5minQty a, s, i

THEN

#RtMaxLimitDev5minQty a, s, i  =

Max [ (ABS ( DaClrdHrlyQty a, s, h ) - RtDispMaxRegCapOL5minQty a, s, i), 0 ] / 12

ELSE IF

** Regulation is cleared in RTBM and not cleared in DA Market **

IF ControlStatus5minFlg a, s, i = “Regulating” AND

DaRegUpHrlyQty a, z, s, h + DaRegDnHrlyQty a, z, s, h = 0 AND

( DaComMaxRegCapOLHrlyQty a, s, h - RtDispMaxRegCapOL5minQty a, s, i )

> ResOpTol5minQty a, s, i

THEN

#RtMaxLimitDev5minQty a, s, i  =

Max { Min [ ABS ( DaClrdHrlyQty a, s, h ) , DaComMaxRegCapOLHrlyQty a, s, h ]

- RtDispMaxRegCapOL5minQty a, s, i  , 0 } / 12

ELSE

RtMaxLimitDev5minQty a, s, i  = 0

(a.4)	 For Resources with DA Market cleared MW in an hour, if the Resource is off-line in the RTBM and it has not been de-committed by SPP the Resource DA Market cleared MW is included as a deviation.  An Asset Owner’s outage deviation is calculated as follows.

RtOutageDevHrlyQty a, s, h = RtOutageDev5minQty a, s, i

IF DaClrdHrlyQty a, s, h  < 0 AND

RtBillMtr5minQty a, s, i  >= 0 AND

ResDeCommit5minFlg a, s, i  < > “1”

THEN

#RtOutageDev5minQty a, s, i =  ABS ( DaClrdHrlyQty a, s, h ) / 12

ELSE

RtOutageDev5minQty a, s, i =  0

(a.5)	For Resources with DA Market cleared MW in an hour, for each Dispatch Interval the Resource is in “Manual” status, a deviation is calculated that is equal to one-twelfth of the difference between the Resource actual output and the Resource’s Desired Dispatch.  An Asset Owner’s status change deviation is calculated as follows.

RtStatusDevHrlyQty a, s, h =  RtStatusDev5minQty a, s, i

IF ControlStatus5minFlg a, s, i = “Manual” AND

DaClrdHrlyQty a, s, h  < 0

THEN

#RtStatusDev5minQty a, s, i =

ABS ( RtBillMtr5minQty a, s, i + RtDispDesiredEn5minQty a, s, i ) / 12

ELSE

RtStatusDev5minQty a, s, i = 0

(a.6)	For Resources that Self-Committed following the Day-Ahead Market and the Resource’s Desired Dispatch in any Dispatch Interval within the Hour is less than or equal to the Resource’s As-Dispatched Minimum Economic Capacity Operating Limit, a deviation is included in an amount equal to the Resource actual output or, in the case of a Combined Cycle Resource, a deviation is included in an amount equal to the difference between the Resource actual output and the Resource DA Market cleared amount.  Resources that were offered into the DA Market for SPP commitment and not committed in the DA Market and then Self-Committed prior to the Day-Ahead RUC are exempted from this calculation.  An Asset Owner’s Self-Commit deviation is calculated as follows.

RtRucScDevHrlyQty a, s, h =  RtRucScDev5minQty a, s, i

IF RtRucComStat5minFlg a, s, i, c = “0” AND

DesiredDispatchMinHrlyFlg a, s, h = “1” AND

RtRucScDevXmpt5minFlg a, s, i <> “1”

THEN

#RtRucScDev5minQty a, s, i =

ABS ( Min (RtBillMtr5minQty a, s, i  - DaClrdHrlyQty a, s, h  , 0 ) / 12 )

ELSE

RtRucScDev5minQty a, s, i = 0

(a.7)	For Resources that are either Self-Committed or committed by SPP following the DA Market and that are off-line in the RTBM and have not been de-committed by SPP, the greater of the Minimum Economic Capacity Operating Limit at the time of commitment or the Resource’s Desired Dispatch will be included as a deviation. An Asset Owner’s RTBM commitment outage deviation is calculated as follows.

RtRucCommitDevHrlyQty a, s, h =  RtRucCommitDev5minQty a, s, i

IF [ RtRucComStat5minFlg a, s, i, c = “0”  OR

RtRucComStat5minFlg a, s, i, c = “1” ] AND

RtBillMtr5minQty a, s, i   >= 0 AND

ResDeCommit5minFlg a, s, i  < > 1

THEN

#RtRucCommitDev5minQty a, s, i   =  RtDesiredEn5minQty a, s, i  / 12

ELSE

RtRucCommitDev5minQty a, s, i   = 0

(a.8)	In any Dispatch Interval in which a Resource operates outside of its Operating Tolerance and the Resource has not been exempted from URD per Section 4.4.4.1, one-twelfth of the Absolute Value of the Resource’s Uninstructed Resource Deviation is included as a deviation.  An Asset Owner’s URD deviation is calculated as follows.

RtURDDevHrlyQty a, s, h =  RtURDDev5minQty a, s, i

IF ABS (URD5minQty a, s, i ) > ResOpTol5minQty a, s, i  AND

( XmptDev5minFlg a, s, i =  0 )

THEN

#RtURDDev5minQty a, s, i = ABS ( URD5minQty a, s, i ) / 12

ELSE

RtURDDev5minQty a, s, i = 0

(b)	#RtMwpSppDistRate d =

( RtMwpSppDlyAmt d / RtDevSppDlyQty d ) * (-1)

(b.1)		RtMwpSppDlyAmt d =  RtMwpMpAmt m, d

(b.2)		RtDevSppDlyQty d =  RtDevHrlyQty a, s, h

- For each Asset Owner, a daily amount is calculated at each Settlement Location.  The daily amount is calculated as follows:

RtMwpDistDlyAmt a, s, d = RtMwpDistHrlyAmt a, s, h

- For each Asset Owner associated with Market Participant m, a daily amount is calculated.  The daily amount is calculated as follows:

RtMwpDistAoAmt a, m, d =  RtMwpDistDlyAmt a, s, d

- For each Market Participant, a daily amount is calculated representing the sum of Asset Owner amounts associated with that Market Participant.  The daily amount is calculated as follows:

RtMwpDistMpAmt m, d =  RtMwpDistAoAmt a, m, d

The above variables are defined as follows:

| Variable | Unit | Settlement Interval | Definition |
| --- | --- | --- | --- |
| RtMwpDistHrlyAmt a, s, h | $ | Hour | RUC Make-Whole-Payment Distribution Amount per AO per Hour per Settlement Location - The amount to AO a for Hour h and Settlement Location s for recovery of the total amount paid under Section 4.5.9.8 for Operating Day d. |
| RtDevHrlyQty a, s, h | MWh | Hour | Real-Time Deviation Quantity per AO per Hour per Settlement Location – The total deviation MWh for AO a at Settlement Location s for Hour h. |
| RtMwpSppDistRate d | $/MWh | Operating Day | RUC Make-Whole Payment SPP Distribution Rate per Operating Day – The rate applied to AO a’s RtDevHrlyQty a, s, h in each Hour h at Settlement Location s in Operating Day d. |
| RtMwpMpAmt m, d | $ | Operating Day | RUC Make-Whole-Payment Amount per MP per Operating Day - The value calculated under Section 4.5.9.8 for Operating Day d. |
| RtMwpSppDlyAmt d | $ | Operating Day | RUC Make-Whole-Payment Amount per Operating Day - The SPP total of the values calculated under Section 4.5.9.8 for Operating Day d. |
| RtDevSppDlyQty d | MWh | Operating Day | Real-Time Deviation Quantity per Operating Day - The SPP total deviation MWh for all AOs for Operating Day d. |
| RtNetSlDevHrlyQty a, s, i | MWh | Dispatch Interval | Real-Time Net Settlement Location Deviation per AO per Dispatch Interval per Settlement Location – AO a’s portion of RtNetSlDevHrlyQty a, s, h related to net of Real-Time load deviations from Day-Ahead amount, Real-Time Interchange Transaction deviations from Day-Ahead amounts and virtual transactions at Settlement Location s in Dispatch Interval i. |
| RtNetSlDevHrlyQty a, s, h | MWh | Hour | Real-Time Net Settlement Location Deviation per AO per Hour per Settlement Location – AO a’s portion of RtDevHrlyQty a, s, h related to net of Real-Time load deviations from Day-Ahead amount, Real-Time Interchange Transaction deviations from Day-Ahead amounts and virtual transactions at Settlement Location s in Hour h. |
| RtBillMtr5minQty a, s, i | MW | Dispatch Interval | Real-Time Billing Meter Quantity per AO per Settlement Location per Dispatch Interval - The quantity described under Section 4.5.9.1. |
| DaClrdHrlyQty a, s, h | MWh | Hour | Day-Ahead Cleared Energy Quantity per AO per Settlement Location per Hour in the DA Market – The quantity described under Section 4.5.8.1. |
| RtImpExp5minQty a, s, i, t, dir | MW | Dispatch Interval | Real-Time Interchange Transaction Quantity per AO per Settlement Location per Dispatch Interval - The quantity described under Section 4.5.9.2 as identified by direction dir. |
| RsgCrdFlgt | none | none | Reserve Sharing Group Contingency Reserve Deployment Flag per Event – The value described under Section 4.5.8.8. |
| DaImpExp5minQty a, s, i, t, dir | MWh | Dispatch Interval | Day-Ahead Interchange Transaction Quantity per AO per Settlement Location per Dispatch Interval - The quantity described under Section 4.5.8.2 as identified by direction dir. |
| DaClrdVHrlyQty a, s, h, t | MWh | Hour | Day-Ahead Cleared Virtual Energy Quantity per AO per Settlement Location per Transaction per Hour in the DA Market – The quantity described under Section 4.5.8.3. |
| RtMinLimitDev5minQty a, s, i | MWh | Dispatch Interval | Real-Time Minimum Limit Deviation per AO per Dispatch Interval per Settlement Location – AO a’s portion of RtDevHrlyQty a, s, h associated with Resources with cleared Day-Ahead amounts that increase their applicable minimum limit in Real-Time above their applicable minimum limit from the Day-Ahead Market commitment at Resource Settlement Location s in Hour h. |
| RtMinLimitDevHrlyQty a, s, h | MWh | Hour | Real-Time Minimum Limit Deviation per AO per Hour per Settlement Location – The sum of AO a’s RtMinLimitDev5minQty a, s, i at Resource Settlement Location s in Hour h. |
| RtMaxLimitDev5minQty a, s, i | MWh | Dispatch Interval | Real-Time Maximum Limit Deviation per AO per Dispatch Interval per Settlement Location – AO a’s portion of RtDevHrlyQty a, s, h associated with Resources with cleared Day-Ahead amounts that reduce their applicable maximum limit in Real-Time below their applicable maximum limit from the Day-Ahead Market commitment at Resource Settlement Location s in Hour h. |
| RtMaxLimitDevHrlyQty a, s, h | MWh | Hour | Real-Time Maximum Limit Deviation per AO per Hour per Settlement Location – The sum of AO a’s RtMaxLimitDev5minQty a, s, i at Resource Settlement Location s in Hour h. |
| RtOutageDev5minQty a, s, i | MWh | Dispatch Interval | Real-Time Outage Deviation per AO per Dispatch Interval per Settlement Location – AO a’s portion of RtDevHrlyQty a, s, h associated with Resources with cleared Day-Ahead amounts that are off-line in Real-Time and have not be de-committed by SPP at Resource Settlement Location s in Dispatch Interval i. |
| RtOutageDevHrlyQty a, s, h | MWh | Hour | Real-Time Outage Deviation per AO per Hour per Settlement Location – The sum of AO a’s RtOutageDev5minQty a, s, i at Resource Settlement Location s in Hour h. |
| RtStatusDev5minQty a, s, i | MWh | Dispatch Interval | Real-Time Resource Status Change Deviation per AO per Settlement Location per Dispatch Interval – AO a’s portion of RtDevHrlyQty a, s, h associated with Resources for which the Control Status is set to “Manual” at Settlement Location s for Dispatch Interval i. |
| RtStatusDevHrlyQty a, s, h | MWh | Dispatch Interval | Real-Time Status Deviation per AO per Hour per Settlement Location – The sum of AO a’s RtStatusDev5minQty a, s, i at Resource Settlement Location s in Hour h. |
| ControlStatus5minFlg a, s, i | none | Dispatch Interval | Control Status per AO per Settlement Location per Dispatch Interval – The value described under Section 4.5.9.8. |
| RtRucScDev5minQty a, s, i | MWh | Dispatch Interval | Real-Time RUC Self-Commit Deviation per AO per Settlement Location per Dispatch Interval – AO a’s portion of RtDevHrlyQty a, s, h associated with Resources that have Self-Committed following completion of the Day-Ahead RUC process at Settlement Location s for Dispatch Interval i. |
| RtRucScDevXmpt5minFlg a, s, i | none | Dispatch Interval | Real-Time RUC Self-Commit Deviation Exemption Flag per AO per Settlement Location per Dispatch Interval – a value of 1 for AO a’s Resources at Settlement Location s for Dispatch Interval i that were offered into the DA Market for SPP commitment and not committed in the DA Market and then Self-Committed prior to the Day-Ahead RUC, thus are exempted from Self-Commit deviation calculations. |
| RtRucScDevHrlyQty a, s, h | MWh | Hour | Real-Time RUC Self-Commit Deviation per AO per Settlement Location per Hour – The summation of AO a’s RtRucScDev5minQty a, s, i for Hour h. |
| RtRucCommitDev5minQty a, s, i | MWh | Dispatch Interval | Real-Time RUC Commit Deviation per AO per Settlement Location per Dispatch Interval – AO a’s portion of RtDevHrlyQty a, s, h associated with Resources that were committed in the Day-Ahead RUC process and fail to come on line at Settlement Location s for Dispatch Interval i. |
| RtRucCommitDevHrlyQty a, s, h | MWh | Dispatch Interval | Real-Time RUC Commit Deviation per AO per Settlement Location per Hour – The summation of AO a’s RtRucCommitDev5minQty a, s, i for Hour h. |
| RtURDDev5minQty a, s, i | MWh | Dispatch Interval | Real-Time URD Deviation per AO per Settlement Location per Dispatch Interval – AO a’s portion of RtDevHrlyQty a, s, h associated with Resources that have operated outside of their ResOpTol a, s, i at Settlement Location s for Dispatch Interval i. |
| RtURDDevHrlyQty a, s, h | MWh | Hour | Real-Time URD Deviation per AO per Hour per Settlement Location – The sum of AO a’s RtURDDev5minQty a, s, i at Resource Settlement Location s in Hour h. |
| URD5minQty a, s, i, | MW | Dispatch Interval | Uninstructed Resource Deviation per AO per Settlement Location per Dispatch Interval– The value calculated as described under Section 4.5.9.8. |
| XmptDev5minFlg a, s, i | none | Dispatch Interval | Failure-to-Follow Dispatch Exemption Flag per AO per Resource Settlement Location per Dispatch Interval – The value described under Section 4.5.9.8. |
| RtDispDesiredEn5minQty a, s, i |  | MW | Dispatch Interval |  |  | Real-Time Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval – The value described under Section 4.5.9.9. |  |
| RtDesiredEn5minQty a, s, i | MW | Dispatch Interval | Real-Time Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval – The value described under Section 4.5.9.8. |
| RtDispMinEconCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Minimum Economic Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The value described under Section 4.5.9.8 for Dispatch Interval i. |
| RtDispMinRegCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Minimum Regulation Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The value described under Section 4.5.9.8 for Dispatch Interval i. |
| DaComMinEconCapOLHrlyQty a, s, h | MW | Hour | Day-Ahead Minimum Economic Capacity Operating Limit Quantity per AO per Settlement Location per Hour – The Minimum Economic Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Hour h as submitted in the DA Market Offer used in the DA Market commitment decision. |
| DaComMinRegCapOLHrlyQty a, s, h | MW | Hour | Day-Ahead Minimum Regulation Capacity Operating Limit Quantity per AO per Settlement Location per Hour – The Minimum Regulation Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Hour h as submitted in the DA Market Offer used in the DA Market commitment decision. |
| RtDispMaxEconCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Maximum Economic Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The Maximum Economic Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i. |
| RtDispMaxRegCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Maximum Regulation Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The Maximum Regulation Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i. |
| DaComMaxEconCapOLHrlyQty a, s, h | MW | Hour | Day-Ahead Maximum Economic Capacity Operating Limit Quantity per AO per Settlement Location per Hour – The Maximum Economic Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Hour h as submitted in the DA Market Offer used in the DA Market commitment decision. |
| DaComMaxRegCapOLHrlyQty a, s, h | MW | Hour | Day-Ahead Maximum Regulation Capacity Operating Limit Quantity per AO per Settlement Location per Hour – The Maximum Regulation Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Hour h as submitted in the DA Market Offer used in the DA Market commitment decision. |
| ResOpTol5minQty a, s, i | MWh | Dispatch Interval | Resource Operating Tolerance per AO per Settlement Location per Hour – The value calculated as described under Section 4.5.9.8. |
| ResDeCommit5minFlg a, s, i | none | Dispatch Interval | Resource De-Commitment Flag per AO per Dispatch Interval per Settlement Location – A flag set by SPP indicating that AO a’s Resource has been de-committed by SPP at Resource Settlement Location s in Dispatch Interval i. |
| RtRucComStat5minFlg a, s, i, c | none | Dispatch Interval | RUC Commitment Status Flag per AO per Resource Settlement Location per Dispatch Interval – The value described under Section 4.5.9.8. |
| DesiredDispatchMinHrlyFlg a, s, h | none | Hour | Desired Dispatch Minimum Flag per AO per Hour per Settlement Location – A flag associated with AO a’s Resource that is set equal to “1” if the Resource’s Desired Dispatch is less than or equal to the Resource’s As-Dispatched Minimum Economic Capacity Operating Limit at any time in Hour h. |
| DesiredDispatchMaxHrlyFlg a, s, h | none | Hour | Desired Dispatch Maximum Flag per AO per Hour per Settlement Location – A flag associated with AO a’s Resource that is set equal to “1” if the Resource’s Desired Dispatch is greater than or equal to the Resource’s As-Dispatched Maximum Economic Capacity Operating Limit at any time in Hour h. |
| DaRegUpHrlyQty a, z, s, h | MW | Hour | Day-Ahead Regulation-Up Quantity per AO per Settlement Location per Hour – The value described under Section 4.5.8.4. |
| DaRegDnHrlyQty a, z, s, h | MW | Hour | Day-Ahead Regulation-Down Quantity per AO per Settlement Location per Hour – The value described under Section 4.5.8.5. |
| RtMwpDistDlyAmt a, s, d | $ | Operating Day | RUC Make-Whole-Payment Distribution Amount per AO per Settlement Location per Operating Day - The amount to AO a at Settlement Location s for recovery of the total amount paid under Section 4.5.9.8 for Operating Day d. |
| RtMwpDistAoAmt a, m, d | $ | Operating Day | RUC Make-Whole-Payment Distribution Amount per AO per Operating Day - The amount to AO a associated with Market Participant m for recovery of the total amounts paid under Section 4.5.9.8 for Operating Day d. |
| RtMwpDistMpAmt m, d | $ | Operating Day | RUC Make-Whole-Payment Distribution Amount per MP per Operating Day - The amount to MP m for recovery of the total amounts paid under Section 4.5.9.8 for Operating Day d. |
| i | none | none | An Dispatch Interval |
| h | none | none | An Hour. |
| d | none | none | An Operating Day. |
| a | none | none | An Asset Owner. |
| c | none | none | A RUC Make-Whole-Payment Eligibility Period. |
| s | none | None | A Settlement Location. |
| t | none | none | A single tagged Interchange Transaction, a single virtual energy transaction, a single Financial Schedule, a single contracted Operating Reserve transaction, a single TCR instrument, a single ARR award or a single Reserve Sharing Event transaction. |
| dir | none | none | Direction (Import, Export or Through). |
| m | none | none | A Market Participant. |

Revised Proposed Tariff Language Revision

## 8.6.5	Reliability Unit Commitment Make Whole Payment Amount [Protocols Section 4.5.9.8]

(1)	A RUC make whole payment is made to the Asset Owner when the sum of a Resource’s eligible RTBM Start-Up Offer costs, No-Load Offer costs, Energy Offer Curve and Operating Reserve Offer costs associated with actual Energy and cleared RTBM Operating Reserve is greater than the Energy and Operating Reserve RTBM revenues received over the Resource’s RUC make whole payment eligibility period.

(2)	For Resources with a RUC Commitment Period that begins in one Operating Day and ends in the next Operating Day, two RUC make whole payment eligibility periods are created.  The first period begins in the first Operating Day in the Dispatch Interval associated with the Resource’s RUC Commit Time and ends at the last Dispatch Interval of the first Operating Day.  The second period begins in the first Dispatch Interval of the next Operating Day and ends in the Dispatch Interval associated with the Resource’s RUC De-Commit Time.

(3)	The following cost recovery rules apply to each RUC make whole payment eligibility period.  Offer costs are calculated using the RTBM Offer prices in effect at the time the commitment decision was made.

(a)	If the Transmission Provider cancels a Commitment Instruction prior to the start of the associated RUC make whole payment eligibility period and the Resource is not a Synchronized Resource, the Asset Owner will receive reimbursement for a time-based pro-rata share of the Resource’s RTBM Start-Up Offer.  Asset Owners may request additional compensation through submittal of actual cost documentation to the Transmission Provider.  The Transmission Provider will review the submitted documentation and confirm that the submitted information is sufficient to document actual costs and that all or a portion of the actual costs are eligible for recovery.

(b)	In order to receive the full amount of Start-Up Offer recovery within a RUC make whole payment eligibility period, the Resource must be a Synchronized Resource in at least one Dispatch Interval in the RUC make whole payment eligibility period.

(c)	In order to receive recovery of No-Load Offer costs in any Dispatch Interval in the RUC make whole payment eligibility period, the Resource must be a Synchronized Resource in that Dispatch Interval.

(d)	There may be more than one RUC make whole payment eligibility period for a Resource in a single Operating Day.  A single RUC make whole payment eligibility period is contained within a single Operating Day.

(e)	A Resource’s RTBM Start-Up Offer costs are not eligible for recovery in the following RUC make whole payment eligibility periods:

(i)	Any RUC make whole payment eligibility period that is adjacent to the end of a Day-Ahead Market make whole payment eligibility period;

(ii)	Any RUC make whole payment eligibility period for which a Resource is a Synchronized Resource prior to this commitment period at a time one (1) hour prior to that Resource’s RUC Commit Time less the Resource’s Sync-To-Min Time; and

(iii)	Any RUC make whole payment eligibility period resulting from a RUC Commitment Period that contains an hour for which the Resource was self-committed.

(f)	For each RUC make whole payment eligibility period within an Operating Day, a Resource’s RTBM Start-Up Offer is divided by the lesser of (1) the Resource’s Minimum Run Time multiplied by twelve (12), rounded down to the nearest whole interval, or (2) twenty-four (24) hours multiplied by twelve (12), and that portion of the Start-Up Offer is included as a cost in each interval of the RUC make whole payment eligibility period until the sum of these interval costs are equal to the RTBM Start-Up Offer or until the end of the RUC make whole payment eligibility period, whichever occurs first.

(g)	To the extent that the full amount of the RTBM Start-Up Offer is not accounted for in the last RUC make whole payment eligibility period in the Operating Day, any remaining RTBM Start-Up Offer costs are carried forward for recovery in the first RUC make whole payment eligibility period of the following Operating Day provided that the Resource has not been committed in the Day-Ahead Market in any hour of the first RUC make whole payment eligibility period as described in (h) below.

(h)	If the Resource has been committed in the Day-Ahead Market in a period adjacent to and following a RUC make whole payment eligibility period to the extent that the full amount of the RTBM Start-Up Offer is not accounted for in the RUC make whole payment eligibility period, any remaining RTBM Start-Up Offer costs are carried forward for recovery in the Day-Ahead make whole payment eligibility period.

(i)	If a Resource has operated outside of its Operating Tolerance in any Dispatch Interval, any cost associated with energy output above the Resource’s economic operating point is not eligible for recovery for that Dispatch Interval where such cost is calculated as described under Subsection 4(c) below.

(j)	If a Resource becomes non-dispatchable in any Dispatch Interval, any cost associated with energy output above the Resource’s economic operating point is not eligible for recovery for that Dispatch Interval where such cost is calculated as described under Subsection 4(c) below.

(k)	If a Resource’s minimum operating limit is increased above the Resource’s minimum operating limit that was used to make the commitment decision, the increase is greater than the Resource’s Operating Tolerance and the Resource remains dispatchable in any Dispatch Interval, any cost associated with energy output above the Resource’s economic operating point is not eligible for recovery for that Dispatch Interval where such cost is calculated as described under Subsection 4(c) below.

(4)	The payment to each Asset Owner for each eligible Settlement Location for a given RUC make whole payment eligibility period is calculated as follows:

RUC Make Whole Payment Amount =

Maximum of [Either Zero or (RUC Make Whole Payment Cost Amount in the RUC Make Whole Payment Eligibility Period + RUC Make Whole Payment Revenue Amount in the RUC Make Whole Payment Eligibility Period – Uninstructed Resource Deviation Cost Disallowance – Non-Dispatchable Cost Disallowance – Minimum Limit Cost Disallowance)]

(a)	An Asset Owner’s RUC Make Whole Payment Cost Amount for each eligible Resource is equal the sum for all Dispatch Intervals in the RUC Make Whole Payment Eligibility Period of (i) Start-Up Offer used to make commitment decision, (ii) No-Load Offer used to make commitment decision, (iii) Energy cost at minimum output as calculated from the Energy Offer Curve used to make commitment decision, (iv) Energy cost above minimum output as calculated from the Energy Offer Curve that applied to the current Dispatch Interval, and (v) Operating Reserve cost associated with cleared Real-Time Operating Reserve as calculated from the Operating Reserve Offers except that Operating Reserve costs associated with self-scheduled Operating Reserve where such self-schedules are less than or equal to the amount of Operating Reserve cleared shall be set equal to zero.

(b)	An Asset Owner’s RUC Make Whole Payment Revenue Amount for each eligible Resource is equal the sum for all hours in the RUC Make Whole Payment Eligibility Period of (i) revenue associated with Energy calculated by multiplying the portion of actual Energy above minimum output by Real-Time LMP (ii) the sum of the revenues calculated under Section 8.6.2, 8.6.3 and 8.6.4 of this Attachment AE for that eligible Resource (iii) Energy revenue associated with payments made under Section 8.6.6 of this Attachment AE and (iv) amounts associated with settlement made under Section 8.6.15 of this Attachment AE.

(c)	An Asset Owner’s Uninstructed Resource Deviation Cost Disallowance, Non-Dispatchable Cost Disallowance, or Minimum Limit Cost Disallowance is equal to the positive difference between the Resource’s Energy cost at actual output as calculated from the Resource’s current Dispatch Interval Energy Offer Curve and the Resource’s Energy cost at the Resource’s economic operating point as calculated from the Resource’s current Dispatch Interval Energy Offer Curve.

(d)	A Resource’s economic operating point is the MW output where the cost on the Resource’s current Dispatch Interval Energy Offer Curve is equal to the Real-Time LMP for that Resource.

## 8.6.7	Reliability Unit Commitment Make Whole Payment Distribution Amount [Protocols Section 4.5.9.10]

An RTBM charge will be calculated at each Settlement Location for each Asset Owner for each hour in order to fund the payments made under Section 8.6.5.  The amount will be determined by multiplying An Asset Owners distribution volume by a daily RUC make whole payment rate as follows:

RUC Make Whole Payment Distribution Amount =

[(RUC SPP Make Whole Payment Distribution Rate) * (RUC Make Whole Payment Distribution Volume)]

(1)	The RUC SPP Make Whole Payment Distribution Rate is the sum of all make whole payments for the Operating Day as calculated under Section 8.6.5, divided by the sum of Asset Owners’ RUC Make Whole Payment Distribution Volumes for all Settlement Locations for the entire Operating Day.

(2)	An Asset Owner’s RUC Make Whole Payment Distribution Volume at a Settlement Location for an hour is equal to the sum of following values that are calculated for each Dispatch Interval within the hour:

(a)	The absolute value of the sum of actual Real-Time Settlement Location deviations from Day-Ahead Market cleared amounts for load, virtual transactions and interchange transactions except that, during any Dispatch Interval in which the Transmission Provider has declared an Emergency Condition, Real-Time actual load deviations from Day-Ahead Market cleared amounts shall be limited to deviations associated with actual Real-Time load in excess of amounts cleared in the Day-Ahead Market;

(b)	For Resources cleared in the Day-Ahead Market, the positive difference between RTBM Resource applicable minimum limits and Day-Ahead Market Resource minimum limits, if:

(i)	Such difference is greater than the Resource’s Operating Tolerance; and

(ii)	The applicable RTBM Resource minimum limit is greater than the Day-Ahead Market cleared Energy amount; and

(iii)	The Resource’s economic operating point as calculated from the Resource’s current Dispatch Interval Energy Offer Curve is less than or equal to the RTBM applicable minimum limit for at least one Dispatch Interval in the hour.

(c)	For Resources cleared in the Day-Ahead Market, the positive difference between the Day-Ahead Market Resource applicable maximum limits and the RTBM Resource applicable maximum limits, if:

(i)	Such difference is greater than the Resource’s Operating Tolerance; and

(ii)	The applicable RTBM Resource maximum limit is less than the Day-Ahead Market cleared Energy amount; and

(iii)	The Resource’s economic operating point as calculated from the Resource’s current Dispatch Interval Energy Offer Curve is greater than or equal to the applicable RTBM applicable maximum limit for at least one Dispatch Interval in the hour.

(d)	For Resources cleared in the Day-Ahead Market, the Resource’s Day-Ahead Market cleared amount if that Resource is off-line in the RTBM and if the Resource has not been de-committed by the Transmission Provider;

(e)	For Resources that cleared in the Day-Ahead Market that are not able to follow Dispatch Instructions, the absolute value of the difference between a Resource’s actual output and the Resource’s economic operating point.  The Resource’s economic operating point is calculated as described under Section 8.6.5(4)(d);

(f)	For Resources that were not cleared in the Day-Ahead Market and that self-committed following the close of the Day-Ahead Market, the actual Resource output if the Resource’s economic operating point as calculated from the Resource’s current Dispatch Interval Energy Offer Curve is less than or equal to the applicable RTBM applicable minimum limit for at least one Dispatch Interval in the hour;

(g)	A Resource’s economic operating point, as calculated as described under Section 8.6.5(4)(d), for Resources that were committed following the close of the Day-Ahead Market if that Resource is off-line in the RTBM and that Resource was not de-committed by the Transmission Provider; and

(h)	The absolute value of a Resource’s URD if that Resource operated outside of its Operating Tolerance and the Resource has not been exempted from URD as described under Section 6.4.1.1 of this Attachment AE.

Revised Proposed Criteria Language Revision

N/A

Note that this charge type will almost always produce a charge.  The credit is included here for the rare occasion when a credit may be produced as a result of a data error and/or a resettlement.