EIS Market Integrated Marketplace

Protocols Revision Request

| PRR No. | Markplace-PRR59 | PRR Title | Misc Protocol Typo Errors and Definition Corrections |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision |  | Section No.: See Below Title: - 3.10 Regulation Non-Performance (Appendix F – Settlement Examples) ------------------------------------------------------------------------------------------------- - 4.5.9.15 - Real-Time Regulation Non-Performance Amount ------------------------------------------------------------------------------------------------- - 4.5.12 – Revenue Neutrality Uplift Distribution Amount ------------------------------------------------------------------------------------------------- - 4.5.9.9 – Real-Time Out-Of-Merit Amount Appendix F: 3.7 – Real-Time Out-Of-_merit Energy and Operating Reserve ------------------------------------------------------------------------------------------------- - 4.5.9.10 – RUC Make-Whole-Payment Distribution Amount (RtNetSlDev5minQty) ------------------------------------------------------------------------------------------------- - 4.5.9.8 – RUC Make-Whole-Payment Amount ------------------------------------------------------------------------------------------------- - 4.5.9.10 – RUC Make-Whole-Payment Distribution Amount (ResOpTol5minQty) ------------------------------------------------------------------------------------------------- - 4.2.2.1 – Resource Offer Parameters (Unit Commitment) ------------------------------------------------------------------------------------------------- - 4.5.9.1 – Real-Time Asset Energy Amount ------------------------------------------------------------------------------------------------- - 4.5.9.9 – Real-Time Out-Of-Merit Amount (RtOomor5minAmt) 4.5.9.15 – Real-Time Regulation Non-Performance Amount (RtRegNonPerf5minAmt) ------------------------------------------------------------------------------------------------- - 4.5.9.19 – Real-Time Regulation Deployment Adjustment Amount Protocol Version: 6.0 |  |
| Impact Analysis Required |  | Yes – If yes, estimated cost: No SPP Staff will complete this section. |  |
| Requested Resolution |  | Normal Expedited Urgent Action Provide explanation if Expedited and/or Urgent Action is selected: Expedited has been chosen to properly inform all parties of the software design and business process changes that may impact their products. |  |
| Type of Revision |  | Correction/Clean-Up Clarification Design Enhancement Design Change |  |
| Revision Description |  | Correct various typographical error and definitions found in the protocols. - The URD maximum operating tolerance was changed from 25MW to 20MW. The protocols were updated in 2011, but the example for Regulation Non-Performance in Appendix F was missed. Need to update the example to account for the change. - Missing definitions for URD5minQty and ResOpTol5minQty used in the calculations. - Correction to remove invalid attribute on RtMcc5minPrca,s,i. The 5min price is tied to settlement location and interval, but not Asset Owner. Correct both calculation and definition. - The protocols have an invalid reference to RtOome5minAmt in section 4.5.9.9 and Appendix F. The correct reference should be RtOom5minAmt. DaSuppHrlyQty & DaSuppMcpHrlyPrc definition has Spinning instead of Supplemental. Spelled out acronym for OOMOR - The calculation for RtNetSIDev5minQty in Section 4.5.9.10 RUC Make-Whole-Payment Distribution Amount in the protocols sum Real-Time and Day-Ahead Imports/Exports/Through bill determinants across the transaction attribute separately. It then applies the Reserve Sharing Group Contingency Reserve Deployment Flag, which is summed across the transaction attribute, separately. The issue is by the time the RsgCrdFlg becomes part of the equation, the affected bill determinants are no longer at a transactional level. - Corrections made to Section 4.5.9.8 RUC Make-Whole-Payment Amount. Typo in calculation where RtRegUp5minAmt should be RtRegUpAvail5minAmt to match Then/Else logic. Also mismatched () in some section 4 calculations. - There is an invalid reference in the RtURDDev5minQty definition in Section 4.5.9.10. References ResOpTol and should be ResOpTol5minQty. - Resource Offer Parameters for (10) Sync-To-Min Time and (11) Min-To-Off Time should also have Unit Commitment parameters. - In the RtResMtrHrlyQty calculation, the summation of Real-Time Adjusted Meter Hourly Quantity across Meter Locations needs to also be across Asset Owners but remain within Settlement Area. As it is written, it appears that the summation is across all Meter Locations for an Asset Owner. All other Bill Determinants in the calculation, including the left side of the equal, specifies the Settlement Area attribute. I am proposing we add the Settlement Area attribute to the Real-Time Adjusted Meter Hourly Quantity Bill Determinant and add a summation across Asset Owner. - Add summation for Reserve Zone for RtOomor5minAmt and RtRegNonPerf5minAmt. This is a follow-up for MPRR-38 – OR Attribute Changes. - Correction in section 4.5.9.19 Real-Time Regulation Adjustment Amount to the definition of RtRegUpDepl5minCostRate. The formulae should multiply RtBillMtr5minQty by (-1). |  |
| Reason for Revision |  | Standard clean-up issues. |  |
| Tariff Implications or Changes |  | Yes – Section No.: (Include a summary of impact and/or specific changes) No |  |
| Criteria Implications or Changes |  | Yes - Section No.: (Include a summary of impact and/or specific changes) No |  |
| Credit Implications |  | Yes (Include a summary of impact and/or specific changes) No |  |
| Date |  | 2/9/2012 |  |
| Sponsor |  |  |  |
| Name |  | Tony Alexander |  |
| E-mail Address |  | talexander@spp.org |  |
| Company |  | Southwest Power Pool |  |
| Phone Number |  | 501.614.3303 |  |

Proposed Protocol Language Revision

#1 - The URD maximum operating tolerance was changed from 25MW to 20MW.  The protocols were updated in 2011, but the example for Regulation Non-Performance in Appendix F was missed.  Need to update the example to account for the change based on the calculation for ResOpTol5minQty for both Max and Percentage.

(d.2)	ResOpTol5minQty a, s, i =

Min ( URDMaxTol5minQty a, s, i , Max (URDMinTol5minQty a, s, i ,

URDTol5minPct a, s, i * RtDispMaxEmerCapOL5minQty a, s, i ) )

3.10 	Regulation Non Performance (Appendix F)

Regulation Non-Performance Example: setpoint is 100 MW, the tolerance band is defined as 5% of the Max Emergency Operating Limit ( = 200 MW) bound by a minimum of 5 and maximum of 20 MW)  Billable Metered Output is at -125 MW.  The Resource clears 10 MW of RegUp in the RTBM at a price of $24 in the interval

IF ABS (25 MWi) > 10 MWi AND ( 10 MWi + 0 ) > 0 AND ( 0 = 0 )

THEN

RtRegNonPerf5minAmt  = Max ( 0, ( 0  * 0 + ( 10 MWi - 0 ) * $24 / MWh ) / 12 i/h ) + ( 0 * 0 + ( 0 - 0 ) * 0) / 12 ) = $20.00

Regulation Non-Performance Example: setpoint is 100 MW, the tolerance band is defined as 5% of the Max Emergency Operating Limit ( = 200 MW) bound by a minimum of 5 and maximum of 20 MW)  Billable Metered Output is at -125 MW.  The Resource clears 10 MW of RegUp in the DA Market, but is not cleared in the RTBM in the interval.  Since the Resource did not clear regulation in RTBM in the interval it is not exposed to the penalty.

IF ABS (25 MWi) > 10 MWi AND (0 + 0 ) > 0 AND ( 0 = 0 )

ELSE

RtRegNonPerf5minAmt  = 0

Regulation Non-Performance Example: setpoint is 100 MW, the tolerance band is defined as 5% of the Max Emergency Operating Limit ( = 200 MW) bound by a minimum of 5 and maximum of 20 MW)  Billable Metered Output is at -125 MW.  The Resource clears 10 MW of RegUp in the DA Market at a price of $6/MWh, but only 5 MW in the RTBM at a price of $24 in the interval.  Since the resource is already buying back regulation in RT at a net loss, there is no penalty.

IF ABS (25 MWi) > 10 MWi AND ( 5 MWi + 0 ) > 0 AND ( 0 = 0 )

THEN

RtRegNonPerf5minAmt  = Max ( 0, ( 10 MWh  * $6 / MWh + ( 5 MWi - 10 MWi ) * $24 / MWh ) / 12 i/h ) + ( 0 * 0 + ( 0 - 0 ) * 0) / 12 )

= Max ( 0, $5 + -$10 ) = 0

#2 - Missing definitions for URD and Operating Tolerance Quantity used in the calculations.

4.5.9.15 	Real-Time Regulation Non-Performance Amount

| URD5minQty a, s, i | MW | Dispatch Interval | Uninstructed Resource Deviation per AO per Settlement Location per Dispatch Interval– The value calculated as described under Section 4.5.9.8. |
| --- | --- | --- | --- |
| ResOpTol5minQty a, s, i | MW | Dispatch Interval | Resource Operating Tolerance per AO per Settlement Location per Hour – The value calculated as described under Section 4.5.9.8. |

#3 - Correction to remove invalid attribute on RtMcc5minPrc a,s,i.  The 5min price is tied to settlement location and interval, but not Asset Owner.  Correct both calculation and definition.

(b.4)	#RtCongestionSppAmt spp, d =

( ( ( RtBillMtr5minQty a, s, i – DaClrdHrlyQty a, s, h )

+  (RtImpExp5MinQty a, s, i, t - DaImpExp5MinQty a, s, i, t )

- DaClrdVHrlyQty a, s, h, t ) * RtMcc5minPrc s, i ) / 12

| RtMcc5minPrc s, i | $/MW | Dispatch Interval | Real-Time Marginal Congestion Component of Real-Time LMP – The Marginal Congestion Component of the Real-Time LMP at Settlement Location s for Dispatch Interval i. |
| --- | --- | --- | --- |

#4 - The protocols have an invalid reference to RtOome5minAmt in section 4.5.9.9 and Appendix F.  The correct reference should be RtOom5minAmt.  DaSuppHrlyQty & DaSuppMcpHrlyPrc definition has Spinning instead of Supplemental.  Spelled out acronym for OOMOR.

4.5.9.9 	Real-Time Out-Of-Merit Amount

    - If the RTBM cleared amount of an Operating Reserve product is less than the DA Market cleared amount of the corresponding Operating Reserve product and the RTBM MCP is greater than the DA Market MCP, the Asset Owner will receive a credit for the difference multiplied by the Out-Of-Merit-Operating Reserve (OOMOR) MW.  The OOMOR MW is calculated as Max (0, or the difference between the Resource’s DA Market cleared Operating Reserve MW and the Resource’s RTBM cleared Operating Reserve MW).

| Variable | Unit | Settlement Interval | Definition |
| --- | --- | --- | --- |
| RtOomeIncr5minAmt a, s, i | $ | Dispatch Interval | Real-Time Out-Of-Merit Incremental Energy Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The portion of AO a’s RtOom5minAmt a, s, i amount for eligible Resource Settlement Location s in Dispatch Interval i for Out-Of-Merit Energy resulting from an SPP manual Dispatch Instruction in the up direction. |
| RtOomeDecr5minAmt a, s, i | $ | Dispatch Interval | Real-Time Out-Of-Merit Decremental Energy Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The portion of AO a’s RtOom5minAmt a, s, i amount for eligible Resource Settlement Location s in Dispatch Interval i for Out-Of-Merit Energy resulting from an SPP manual Dispatch Instruction in the down direction. |
| RtOom5minFlg a, s, i | None | Dispatch Interval | Real-Time Out-of-Merit Flag per AO per Settlement Location per Dispatch Interval – A flag that is set equal to 1 when SPP issues a Manual Dispatch Instruction or whenever there is a price correction event as described under Section 7, otherwise, this flag is set equal to zero. |
| RtOomor5minAmt a, s, i | $ | Dispatch Interval | Real-Time Out-Of-Merit Operating Reserve Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The portion of AO a’s RtOom5minAmt a, s, i attributable to buying back a DA Market Operating Reserve position in the RTBM at a RTBM MCP that is greater than the corresponding DA Market MCP. This should not be a normal occurrence but could happen as a result of price corrections as described under Section 7. |

| DaSuppHrlyQty a, z, s, h | MW | Hour | Day-Ahead Supplemental Reserve Quantity per AO per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.7. |
| --- | --- | --- | --- |
| DaSuppMcpHrlyPrc z, h | $/MW | Hour | Day-Ahead Supplemental Reserve Market Clearing Price per Settlement Location per Hour in the DA Market– The value described under Section 4.5.8.7. |

Appendix F

3.7	Real-Time Out-Of-Merit Energy and Operating Reserve

A Resource is having cleared -100 MWh injection of Energy at $20/MWh and 25 MW Supplemental Reserve capacity for the hour in the DA Market, is moved by OOME instruction to -150 MWi actual output and no OR for the interval in RTBM.  The LMP of 30 $ / MWh would put the Resource at 125 MW on its offer curve at an integrated cost of $2000, whereas the integrated cost at its actual output is $3000.  The integrated cost of the DA market cleared injection on the Resource’s RTBM offer curve is $1000.  The MCP of Supplemental Reserve increases from $5/MWh in the DA Market to $10/MWh in the RTBM.

RtOomeIncr5minAmt  = Max ( 0, ($3000i/h - $1000i/h) – ($2000i/h - $1000i/h)  + Min ( 0, -150 MWi  + 125 MWi)  * Max( 0, 30 $/MWh) ) / 12i/h  = Max($1000 - $750) / 12= $20.83

RtOomeDecr5minAmt = Max (0, -150 MWi  - -100 MWi  ) * Max ( 0, 30 $/MWh  - 20 $/MWh  )  / 12i/h = $0

RtOomor5minAmt  = [(Max(0,0)*Max(0,X-Y) + Max(0,0)*Max(0,X-Y) + Max(0,0)*Max(0,X-Y) + Max(0,25 MWi-0) * Max(0, $10/MWh  - $5/MWh)] / 12i/h = $10.42

RtOom5minAmt = 1 * ( $20.83 + 0   + $10.42 ) * (-1) = -$31.25 (credit)

A Resource is having cleared -200 MWh injection of Energy at $20/MWh and no OR in either the DA Market or RTBM, is moved by OOME instruction to -150 MWi actual output.  The LMP of 30 $ / MWh would put the Resource at 250 MW on its offer curve at an integrated cost of $4000, whereas the integrated cost at its actual output is $3000.  The integrated cost of the DA market cleared injection on the Resource’s RTBM offer curve is $2000.

RtOomeIncr5minAmt = Max ( 0, ($3000i/h - $2000i/h) - ($4000i/h - $2000i/h)  + Min ( 0, -150 MWi  + 250 MWi)  * Max( 0, 30 $/MWh) ) / 12i/h  = Max(-$1000 - $0) / 12= $0

RtOomeDecr5minAmt = Max (0, -150 MWi  - -200 MWi  ) * Max ( 0, 30 $/MWh  - 20 $/MWh )  / 12i/h = $41.67

RtOomor5minAmt = [(Max(0,0)*Max(0,X-Y) + Max(0,0)*Max(0,X-Y) + Max(0,0)*Max(0,X-Y) + Max(0,0)*Max(0,X-Y)] / 12i/h = $0

RtOom5minAmt = 1 * ( $0 + $41.67   + $0 ) * (-1) = -$41.67 (credit)

#5 - The calculation for RtNetSIDev5minQty in Section 4.5.9.10 RUC Make-Whole-Payment Distribution Amount in the protocols sum Real-Time and Day-Ahead Imports/Exports/Through bill determinants across the transaction attribute separately. It then applies the Reserve Sharing Group Contingency Reserve Deployment Flag, which is summed across the transaction attribute, separately.  The issue is by the time the RsgCrdFlg becomes part of the equation, the affected bill determinants are no longer at a transactional level. Modify the summation across the transaction attribute simultaneously.

4.5.9.10 	RUC Make-Whole-Payment Distribution Amount

(a.1)	An Asset Owner’s Settlement Location deviation is calculated as the Absolute Value of the sum of (1) (RTBM actual load MWh - DA Market cleared load MWh), (2) (RTBM actual Export Interchange Transactions – DA Market cleared Export Interchange Transactions), (3) (RTBM actual Import Interchange Transactions – DA Market cleared Import Interchange Transactions), (4) (RTBM actual Through Interchange Transactions (sink only) – DA Market cleared Through Interchange Transactions (sink only)), (5) DA Market cleared Virtual Energy Bids * (-1), and (6) DA Market cleared Virtual Energy Offers * (-1).  An Asset Owner’s Settlement Location deviation is calculated as follows.

RtNetSlDevHrlyQty a, s, h = ABS RtNetSlDev5minQty a, s, i

#RtNetSlDev5minQty a, s, i =

{ Max ( 0, RtBillMtr5minQty a, s, i   )  – Max ( 0, DaClrdHrlyQty a, s, h )

+  {[Max ( 0, RtImpExp5minQty a, s, i, t, dir  )

- Max ( 0, DaImpExp5minQty a, s, i, t, dir  )

+ [ IF DIR <> “THROUGH”, THEN

Min ( 0, RtImpExp5minQty a, s, i, t, dir  )

- Min ( 0, DaImpExp5minQty a, s, i, t, dir  ), ELSE 0 ] ]

* (1 – RsgCrdFlgt ) } - DaClrdVHrlyQty a, s, h, t } / 12

#6 - Corrections made to Section 4.5.9.8 RUC Make-Whole-Payment Amount. Typo in calculation where RtRegUp5minAmt should be RtRegUpAvail5minAmt to match Then/Else logic. Also mismatched ()  in some section 4 calculations.

4.5.9.8 	RUC Make-Whole-Payment Amount

(a.3)	If RtRegUp5minQty a, s, i  > RtFixedRegUp5minQty a, s, i

THEN

RtRegUpAvail5minAmt a, s, i, c =

Max ( 0, [ RtRegUp5minQty a, z, s, i   - DaRegUpHrlyQty a, z, s, h] )

* RtRegUpOffer a, s, i, c

ELSE

RtRegUpAvail5minAmt a, s, i  =0

(a.4)	If RtRegDn5minQty a, s, i  > RtFixedRegDn5minQty a, s, i

THEN

RtRegDnAvail5minAmt a, s, i, c =

Max ( 0, [ RtRegDn5minQty a, z, s, i   - DaRegDnHrlyQty a, z, s, h] )

* RtRegDnOffer a, s, i, c

ELSE

RtRegDnAvail5minAmt a, s, i  =0

(a.5)	If RtSpin5minQty a, s, i  > RtFixedSpin5minQty a, s, i

THEN

RtSpinAvail5minAmt a, s, i, c =

Max ( 0, [ RtSpin5minQty a, z, s, i   - DaSpinHrlyQty a, z, s, h] )

* RtSpinOffer a, s, i, c

ELSE

RtSpinAvail5minAmt a, s, i  =0

(a.6)	If RtSupp5minQty a, s, i  > RtFixedSupp5minQty a, s, i

THEN

RtSuppAvail5minAmt a, s, i, c =

Max ( 0, [ RtSupp5minQty a, z, s, i   - DaSuppHrlyQty a, z, s, h] )

* RtSuppOffer a, s, i, c

ELSE

RtSuppAvail5minAmt a, s, i  =0

- The amount to each Asset Owner (AO) for each eligible Resource Settlement Location for a given RUC Make-Whole Payment Eligibility Period is calculated as follows:

#RtMwpCpAmt a, s, c =  ( CncldStartAmt a, s, c

+ Max (0, ( {  IF ( CncldStartRatio a, s, c = 0, THEN 1, ELSE 0) }

*  { RtStartUpElig5minFlg a, s, i, c  * RtStartUp5minAmt a, s, i, c

+ RtRucComStat5minFlg a, s, i, c * [ RtMwpCost5minAmt a, s, i, c

+ RtMwpRev5minAmt a, s, i, c

+ RtOom5minAmt a, s, i + RtRegAdj5minAmt a, s, i

–  RtURDAdj5minAmt a, s, i, c – RtStatusAdj5minAmt a, s, i, c

–  RtLimitAdj5minAmt a, s, i, c  ] } ) ) ) * (-1)

(d) In any Dispatch Interval in which the Resource has operated outside of its Operating Tolerance and that Resource has not been exempted from URD per Section 4.4.4.1, any incremental Energy costs associated with actual Energy output above the Resource’s Desired Dispatch is not eligible for recovery.  The URD adjustment is calculated as follows:

IF ABS (URD5minQty a, s, i ) > ResOpTol5minQty a, s, i AND

( XmptDev5minFlg a, s, i =  0 )

THEN

#RtURDAdj5minAmt a, s, i, c = RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i )) / 12

ELSE

RtURDAdj5minAmt a, s, i, c = 0

(e) In any Dispatch Interval in which a Resource is in “Manual” status, any incremental Energy costs associated with actual Energy output above the Resource’s Desired Dispatch is not eligible for recovery.  The status change adjustment is calculated as follows:

IF ControlStatus5minFlg a, s, i = “Manual”

AND ABS (URD5minQty a, s, i ) <= ResOpTol5minQty a, s, i

THEN

#RtStatusAdj5minAmt a, s, i, c = RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i )) / 12

ELSE

RtStatusAdj5minAmt a, s, i, c = 0

(f)	In any Dispatch Interval in which a Resource has increased its Minimum Economic Capacity Operating Limit (or its Minimum Regulation Capacity Operating Limit if the Resource has cleared for Regulation-Up or Regulation-Down) above the Resource’s minimum limits used by SPP in the commitment decision or the minimum limits used to move from one configuration to another in the case of a Combined Cycle Resource, the Resource is not in “Manual” status and the increase in minimum limit is greater than the Resource’s Operating Tolerance, any incremental Energy costs associated with actual Energy output above the Resource’s Desired Dispatch is not eligible for recovery.  The limit change adjustment is calculated as follows:

IF ControlStatus5minFlg a, s, i < > “Regulating” AND

ControlStatus5minFlg a, s, i  < > “Manual”  AND

( RtDispMinEconCapOL5minQty a, s, i

- RtComMinEconCapOL5minQty a, s, i ) > ResOpTol5minQty a, s, i  AND

ABS (URD5minQty a, s, i ) <= ResOpTol5minQty a, s, i

THEN

#RtLimitAdj5minAmt a, s, i, c =   RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i ) / 12

ELSE IF

ControlStatus5minFlg a, s, i  = “Regulating”  AND

( RtDispMinRegCapOL5minQty a, s, i

- RtComMinRegCapOL5minQty a, s, i ) > ResOpTol5minQty a, s, i  AND

ABS (URD5minQty a, s, i ) < =ResOpTol5minQty a, s, i

THEN

#RtLimitAdj5minAmt a, s, i, c =  RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i )) / 12

ELSE

RtLimitAdj5minAmt a, s, i, c = 0

#7 - There is an invalid reference in the RtURDDev5minQty definition in Section 4.5.9.10.  References ResOpTol and should be ResOpTol5minQty.

4.5.9.10 	RUC Make-Whole-Payment Distribution Amount

| RtURDDev5minQty a, s, i | MWh | Dispatch Interval | Real-Time URD Deviation per AO per Settlement Location per Dispatch Interval – AO a’s portion of RtDevHrlyQty a, s, h associated with Resources that have operated outside of their ResOpTol5minQty a, s, i at Settlement Location s for Dispatch Interval i. |
| --- | --- | --- | --- |

#8 - Resource Offer Parameters for (10) Sync-To-Min Time and (11) Min-To-Off Time should also have Unit Commitment parameters.

4.2.2.1 	Resource Offer Parameters

- Regulation-Down Offer ($/MW);

- Spinning Reserve Offer ($/MW);

- Supplemental Reserve Offer ($/MW);

- Sync-To-Min Time (hours:minutes – Unit Commitment) 1;

- Min-To-Off Time (hours:minutes – Unit Commitment) 1;

- Start-Up Time (hours:minutes, Hot, Intermediate, Cold – Unit Commitment)1;

- Hot to Intermediate Time (hours:minutes– Unit Commitment)1;

For Asset Owners that have registered a JOU under the Combined Resource Option (see Section 6.1.7.2), , this value must be submitted by the specified Asset Owner and represents the value for the entire Physical JOU Resource)

#9 - In the RtResMtrHrlyQty calculation, the summation of Real-Time Adjusted Meter Hourly Quantity across Meter Locations needs to also be across Asset Owners but remain within Settlement Area.  As it is written, it appears that the summation is across all Meter Locations for an Asset Owner.  All other Bill Determinants in the calculation, including the left side of the equal, specifies the Settlement Area attribute.  I am proposing we add the Settlement Area attribute to the Real-Time Adjusted Meter Hourly Quantity Bill Determinant and add a summation across Asset Owner.

4.5.9.1 	Real-Time Asset Energy Amount

- The hourly residual load determinant is the net difference between generation & load, interchange and losses per Settlement Area.  Hourly Net Actual Interchange is derived as the sum of the hourly metering submitted for aggregate ties between interconnected Settlement Areas.  Missing tie values are assumed to be 0.  The hourly residual determinant is expressed in terms of levelized MW.  The formula for the hourly residual load determinant is shown below.

RtResMtrHrlyQty sa, h  = (RtAdjMtrHrlyQty sa ,a, ml, h

+ RtSaNetActIchngHrlyQty sa, h  + RtSELoss5minQty sa, i  / 12) * (-1)

#10 - Add summation for Reserve Zone for RtOomor5minAmt and RtRegNonPerf5minAmt. This is a follow-up for MPRR-38 – OR Attribute Changes.

4.5.9.15 	Real-Time Regulation Non-Performance Amount

IF ABS (URD5minQty a, s, i ) > ResOpTol5minQty a, s, i AND

( RtRegUp5minQty a, z, s, i + RtRegDn5minQty a, z, s, i ) > 0 AND

( XmptDev5minFlg a, s, i =  0 )

THEN

RtRegNonPerf5minAmt a, s, i  =

Max ( 0, (DaRegUpHrlyQty a, z, s, h  * DaRegUpMcpHrlyPrc z, h

+ ( RtRegUp5minQty a, z, s, i - DaRegUpHrlyQty a, z, s, h )

* RtRegUpMcp5minPrc z, i ) / 12

+ ( DaRegDnHrlyQty a, z, s, h  * DaRegDnMcpHrlyPrc z, h

+ ( RtRegDn5minQty a, z, s, i - DaRegDnHrlyQty a, z, s, h )

* RtRegDnMcp5minPrc z, i ) / 12 )

ELSE

RtRegNonPerf5minAmt a, s, i  = 0

Appendix F

3.7	Regulation Non-Performance

Regulation Non-Performance

IF ABS (URD5minQty a, s, i ) > ResOpTol5minQty a, s, i AND ( RtRegUp5minQty a, z, s, i + RtRegDn5minQty a, z, s, i ) > 0 AND ( XmptDev5minFlg a, s, i =  0 )

THEN

RtRegNonPerf5minAmt a, s, i  = Max ( 0, (DaRegUpHrlyQty a, z, s, h  * DaRegUpMcpHrlyPrc z, h + ( RtRegUp5minQty a, z, s, i - DaRegUpHrlyQty a, z, s, h ) * RtRegUpMcp5minPrc z, i ) / 12 )

+ (DaRegDnHrlyQty a, z, s, h  * DaRegDnMcpHrlyPrc z, h + ( RtRegDn5minQty a, z, s, i - DaRegDnHrlyQty a, z, s, h ) * RtRegDnMcp5minPrc z, i ) / 12 )

ELSE

RtRegNonPerf5minAmt a, s, i  = 0

4.5.9.9 	Real-Time Out-Of-Merit Amount

(c)		RtOomor5minAmt a, s, i  =

[ ( Max (0, DaRegUpHrlyQty a, z, s, h  - RtRegUp5minQty a, z, s, i )

* Max ( 0, RtRegUpMcp5minPrc z, i  - DaRegUpMcpHrlyPrc z, h  ) )

+ ( Max (0, DaRegDnHrlyQty a, z, s, h  - RtRegDn5minQty a, z, s, i )

* Max ( 0, RtRegDnMcp5minPrc z, i  - DaRegDnMcpHrlyPrc z, h  ) )

+ ( Max (0, DaSpinHrlyQty a, z, s, h  - RtSpin5minQty a, z, s, i )

* Max ( 0, RtSpinMcp5minPrc z, i  - DaSpinMcpHrlyPrc z, h  ) )

+ ( Max (0, DaSuppHrlyQty a, z, s, h  - RtSupp5minQty a, z, s, i )

* Max ( 0, RtSuppMcp5minPrc z, i  - DaSuppMcpHrlyPrc z, h  ) ) ] / 12

Appendix F

3.7	Real-Time Out-Of-Merit Energy and Operating Reserve

Real-Time Out-Of-Merit Settlement

RtOom5minAmt a, s, i = RtOom5minFlg a, s, i  * ( RtOomeIncr5minAmt a, s, i + RtOomeDecr5minAmt a, s, i   + RtOomor5minAmt ) * (-1)

Where,

RtOomeIncr5minAmt a, s, i  =Max ( 0, RtIncrEn5minAmt a, s, i – RtDesiredEc5minAmt a, s, i

+ Min ( 0, RtBillMtr5minQty a, s, i  + RtDesiredEc5minQty a, s, i )  * Max( 0, RtLmp5minPrc s, i ) ) / 12

RtOomeDecr5minAmt a, s, i  = Max (0, RtBillMtr5minQty a, s, i  - DaClrdHrlyQty a, s, h  ) * Max ( 0, RtLmp5minPrc s, i  - DaLmpHrlyPrc s, h  )  / 12

RtOomor5minAmt a, s, i  =

[ ( Max (0, DaRegUpHrlyQty a, z, s, h  - RtRegUp5minQty a, z, s, i ) * Max ( 0, RtRegUpMcp5minPrc z, i  - DaRegUpMcpHrlyPrc z, h  ) )

+ ( Max (0, DaRegDnHrlyQty a, z, s, h  - RtRegDn5minQty a, z ,s, i ) * Max ( 0, RtRegDnMcp5minPrc z, i  - DaRegDnMcpHrlyPrc z, h  ) )

+ ( Max (0, DaSpinHrlyQty a, z, s, h  - RtSpin5minQty a, z ,s, i ) * Max ( 0, RtSpinMcp5minPrc z, i  - DaSpinMcpHrlyPrc z, h  ) )

+ ( Max (0, DaSuppHrlyQty a, z, s, h  - RtSupp5minQty a, z, s, i ) * Max ( 0, RtSuppMcp5minPrc z, i  - DaSuppMcpHrlyPrc z, h  ) ) ] / 12

Appendix F

3.2.1	Day Ahead Procurement

Regulation Up

DaRegUpHrlyAmt a, s, h =  (DaRegUpMcpHrlyPrc z, h * DaRegUpHrlyQty a, z, s, h ) * (-1)

Regulation Down

DaRegDnHrlyAmt a, s, h =  (DaRegDnMcpHrlyPrc z, h * DaRegDnHrlyQty a, z, s, h ) * (-1)

Spinning Reserve

DaSpinHrlyAmt a, s, h = (DaSpinMcpHrlyPrc z, h * DaSpinHrlyQty a, z, s, h ) *(-1)

Supplemental Reserve

DaSuppHrlyAmt a, s, h =  (DaSuppMcpHrlyPrc z, h * DaSuppHrlyQty a, z, s, h ) * (-1)

#11 - Correction in section 4.5.9.19 Real-Time Regulation Adjustment Amount to the definition of RtRegUpDepl5minCostRate.   The formulae should multiply RtBillMtr5minQty by (-1).

4.5.9.19	Real-Time Regulation Deployment Adjustment Amount

| RtRegUpDepl5minCostRate a, s, i | $/MW | Dispatch Interval | Real-Time Regulation-Up Deployment Cost Rate per AO per Settlement Location per Dispatch Interval – The cost, in $/MW, associated with RtRegUpDepl5minQty a, s, i for AO a’s Resource at Settlement Location s in Dispatch Interval i. The cost is calculated as / (RtRegUpDepl5minQty a, s, i) , where Stop = Min ( RtAvgSetpoint5minQty a, s, i, (-1)*RtBillMtr5minQty a, s, i ) Start = ( Stop - RtRegUpDepl5minQty a, s, i ) |
| --- | --- | --- | --- |

Proposed Tariff Language Revision

N/A

Proposed Criteria Language Revision

N/A