EIS Market Integrated Marketplace

Protocols Revision Request

| PRR No. | Marketplace-PRR84 | PRR Title | Correction to Desired Energy Amount Calculation in RUC Make-Whole Payment |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision |  | Section No.: 4.5.9.8 Title: RUC Make-Whole-Payment Amount Protocol Version: 10.0 |  |
| Impact Analysis Required |  | Yes – If yes, estimated cost: No SPP Staff will complete this section. |  |
| Requested Resolution |  | Normal Expedited Urgent Action Provide explanation if Expedited and/or Urgent Action is selected: |  |
| Type of Revision |  | Correction/Clean-Up Clarification Design Enhancement Design Change |  |
| Revision Description |  | This MPRR changes the calculation for the cost of Real-Time desired energy by establishing the lower limit as a floor for the upper limit for the integration of the offer curve. |  |
| Reason for Revision |  | Real-Time Desired Energy 5-minute Amount is the average incremental energy offer cost for a Resource to operate at its market desired output. It is used in the RUC Make-Whole-Payment Amount to disallow incremental energy costs above desired energy costs as part of the adjustments to a resource’s eligible operating costs. Desired energy amount is calculated as the area under the As-Dispatched offer curve. The lower limit of the integral is defined as the greater of the resource’s Day-Ahead cleared energy and its effective minimum operating limit. The upper limit of the integral is defined as the resources desired output. This MPRR defines the minimum value for the upper limit of the integral as the lower limit of the integral. |  |
| Tariff Implications or Changes |  | Yes – Section No.: (Include a summary of impact and/or specific changes) No |  |
| Criteria Implications or Changes |  | Yes - Section No.: (Include a summary of impact and/or specific changes) No |  |
| Credit Implications |  | Yes (Include a summary of impact and/or specific changes) No |  |
| Date |  | 7/3/2012 |  |
| Sponsor |  |  |  |
| Name |  | Tony Alexander |  |
| E-mail Address |  | talexander@spp.org |  |
| Company |  | Southwest Power Pool |  |
| Phone Number |  | 501.614.3303 |  |

Proposed Protocol Language Revision

#### RUC Make-Whole-Payment Amount

- The RUC Make-Whole-Payment Amount is a credit or charge to a Resource Asset Owner and is calculated for each Resource with a RUC Commitment Period.  A payment is made to the Resource Asset Owner when the sum of the Resource’s eligible RTBM Start-Up Offer costs, No-Load Offer costs, Energy Offer Curve and Operating Reserve Offer costs associated with actual MWh amounts for Energy and cleared RTBM Operating Reserve is greater than the Energy and Operating Reserve RTBM revenues received for that Resource over the Resource’s RUC Make-Whole-Payment Eligibility Period.

- A Resource’s RUC Make-Whole-Payment Eligibility Period is equal to the Resource’s RUC Commitment Period except as described below:

- As shown in Exhibit 4-20, for Resources with a RUC Commitment Period that begins in one Operating Day and ends in the next Operating Day, two RUC Make-Whole-Payment Eligibility Periods are created.  The first period begins in the first Operating Day in the Dispatch Interval associated with the Resource’s RUC Commit Time and ends at the last Dispatch Interval of the first Operating Day.  The second period begins in the first Dispatch Interval of the next Operating Day and ends in the Dispatch Interval associated with the Resource’s RUC De-Commit Time.

Exhibit 4-20: RUC Make-Whole Payment Eligibility Period – Multiple Operating Days

Operating Day 2

Operating Day 1

Real-Time Make-Whole Payment Eligibility Period

Real-Time Make-Whole Payment Eligibility Period

Time

RUC Commitment Period

- The following cost recovery eligible rules apply to each RUC Make-Whole-Payment Eligibility Period.   Offer costs are calculated using the RTBM Offer prices in effect at the time the commitment decision was made.

- If SPP cancels a start-up order prior to the start of the associated RUC Make-Whole-Payment Eligibility Period and the Resource is not a Synchronized Resource, the Asset Owner will receive reimbursement for a time-based pro-rata share of the Resource’s RTBM Start-Up Offer.  Asset Owners may request additional compensation through submittal of actual cost documentation to the SPP.  SPP will review the submitted documentation and confirm that the submitted information is sufficient to document actual costs and that all or a portion of the actual costs are eligible for recovery.

- In order to receive Start-Up Offer recovery within a RUC Make-Whole-Payment Eligibility Period, the Resource must be a Synchronized Resource for at least one Dispatch Interval in the RUC Make-Whole Payment Eligibility Period.

- In order to receive recovery of No-Load Offer costs in any Dispatch Interval in the RUC Make-Whole Payment Eligibility Period, the Resource must be a Synchronized Resource in that Dispatch Interval.

- There may be more than one RUC Make-Whole Payment Eligibility Period for a Resource in a single Operating Day for which a credit or charge is calculated.  A single RUC Make-Whole Payment Eligibility Period is contained within a single Operating Day.

- A Resource’s RTBM Start-Up Offer costs are not eligible for recovery in the following RUC Make-Whole Payment Eligibility Periods:

  - Any RUC Make-Whole Payment Eligibility Period that is adjacent to the end of a DA Market Make-Whole Payment Eligibility Period;

  - Any RUC Make-Whole Payment Eligibility Period for which a Resource is a Synchronized Resource prior to this commitment period at a time one hour prior to that Resource’s RUC Commit Time less the Resource’s Sync-To-Min Time; and

  - Any RUC Make-Whole Payment Eligibility Period resulting from a RUC Commitment Period that contains an hour for which the Resource Commitment Status is Self-Commit.

- For each RUC Make-Whole Payment Eligibility Period within an Operating Day, a Resource’s RTBM Start-Up Offer is divided by the lesser of (1) the Resource’s Minimum Run Time multiplied by 12 rounded down to the nearest whole interval or (2) 24 Hours multiplied by 12, and that portion of the Start-Up Offer is included as a cost in each interval of the RUC Make-Whole Payment Eligibility Period until the sum of these interval costs are equal to the RTBM Start-Up Offer or until the end of the RUC Make-Whole Payment Eligibility Period, whichever occurs first.

- To the extent that the full amount of the RTBM Start-Up Offer is not accounted for in the last RUC Make-Whole Payment Eligibility Period in the Operating Day, any remaining RTBM Start-Up Offer costs are carried forward for recovery in the first RUC Make-Whole Payment Eligibility Period of the following Operating Day provided that the Resource has not been committed in the DA Market in any hour of the first RUC Make-Whole Payment Eligibility Period as described in (h) below.  For example, consider a Resource that is committed starting at 10:00 PM in Operating Day 1 that has a Minimum Run Time of 10 hours and a Start-Up Offer of $12,000.  The RUC Commitment Period is from 10:00 PM in Operating Day 1 through 8:00 AM of Operating Day 2.  For RUC Make-Whole Payment calculation purposes, the RUC Commitment Period is split into two separate RUC Make-Whole Payment Eligibility Periods as described in (2).a above.  The first RUC Make-Whole Payment Eligibility Period will include $100/interval of Start-Up Offer costs ($12,000 / 120 intervals) in hour 23 and 24 intervals.  The second RUC Make-Whole Payment Eligibility Period will include $100/interval of Start-Up Offer costs in hours 1 through 8 intervals.

- If the Resource has been committed in the DA Market in a period adjacent to and following a RUC Make-Whole Payment Eligibility Period to the extent that the full amount of the RTBM Start-Up Offer is not accounted for in the RUC Make-Whole Payment Eligibility Period, any remaining RTBM Start-Up Offer costs are carried forward for recovery in the Day-Ahead Make-Whole Payment Eligibility Period.

- The amount to each Asset Owner (AO) for each eligible Resource Settlement Location for a given RUC Make-Whole Payment Eligibility Period is calculated as follows:

#RtMwpCpAmt a, s, c =  ( CncldStartAmt a, s, c

+ Max (0, ( { IF ( CncldStartRatio a, s, c = 0, THEN 1, ELSE 0) }

* { RtStartUpElig5minFlg a, s, i, c  * RtStartUp5minAmt a, s, i, c

+ RtRucComStat5minFlg a, s, i, c * [ RtMwpCost5minAmt a, s, i, c

+ RtMwpRev5minAmt a, s, i, c

+ RtOom5minAmt a, s, i + RtRegAdj5minAmt a, s, i

–  RtURDAdj5minAmt a, s, i, c – RtStatusAdj5minAmt a, s, i, c

–  RtLimitAdj5minAmt a, s, i, c  ] } ) ) ) * (-1)

Where,

(a)	#RtMwpCost5minAmt a, s, i, c =  RtRucComStat5minFlg a, s, i, c  *

( RtIncrEn5minAmt a, s, i

+  Max ( 0, [ RtNoLoad5minAmt a, s, i, c

- IF (DaClrdHrlyQty a, s, h < 0, THEN DaNoLoadHrlyAmt a, s, h, c , ELSE 0 ) ] )

+ RtMinEn5minAmt a, s, i, c

+  RtRegUpAvail5minAmt a, s, i, c +  RtRegDnAvail5minAmt a, s, i, c

+  RtSpinAvail5minAmt a, s, i, c +  RtSuppAvail5minAmt a, s, i, c ) / 12

(a.1)	IF ABS (DaClrdHrlyQty a, s, h ) > = ABS ( RtBillMtr5minQty a, s, i )

THEN

RtIncrEn5minAmt a, s, i = 0

ELSE

#RtIncrEn5minAmt a, s, i =

Where:

X = Max (ABS (DaClrdHrlyQty a, s, h ), RtEffMin5minQty a, s, i )

AND

IF ControlStatus5minFlg a, s, i  = “Regulating”

THEN

RtEffMin5minQty a, s, i = Min (

RtComMinRegCapOL5minQtya, s, i ,

RtDispMinRegCapOL5minQtya, s, i ,

Max (0, (-1) * RtBillMtr5minQtya, s, i )

ELSE

RtEffMin5minQty a, s, i = Min (

RtComMinEconCapOL5minQtya, s, i ,

RtDispMinEconCapOL5minQtya, s, i ,

Max (0, (-1) * RtBillMtr5minQtya, s, i )

AND

Y = Max ( (-1) * RtBillMtr5minQtya, s, i , 0)

(a.2)	IF ABS (DaClrdHrlyQty a, s, h ) > 0

THEN

RtMinEn5minAmt a, s, i, c = 0

ELSE

# RtMinEn5minAmt a, s, i, c =

(a.3)	If RtRegUp5minQty a, s, i  > RtFixedRegUp5minQty a, s, i

THEN

RtRegUpAvail5minAmt a, s, i, c =

Max ( 0, [ RtRegUp5minQty a, z, s, i   - DaRegUpHrlyQty a, z, s, h] )

* RtRegUpOffer a, s, i, c

ELSE

RtRegUpAvail5minAmt a, s, i  =0

(a.4)	If RtRegDn5minQty a, s, i  > RtFixedRegDn5minQty a, s, i

THEN

RtRegDnAvail5minAmt a, s, i, c =

Max ( 0, [ RtRegDn5minQty a, z, s, i   - DaRegDnHrlyQty a, z, s, h] )

* RtRegDnOffer a, s, i, c

ELSE

RtRegDnAvail5minAmt a, s, i  =0

(a.5)	If RtSpin5minQty a, s, i  > RtFixedSpin5minQty a, s, i

THEN

RtSpinAvail5minAmt a, s, i, c =

Max ( 0, [ RtSpin5minQty a, z, s, i   - DaSpinHrlyQty a, z, s, h] )

* RtSpinOffer a, s, i, c

ELSE

RtSpinAvail5minAmt a, s, i  =0

(a.6)	If RtSupp5minQty a, s, i  > RtFixedSupp5minQty a, s, i

THEN

RtSuppAvail5minAmt a, s, i, c =

Max ( 0, [ RtSupp5minQty a, z, s, i   - DaSuppHrlyQty a, z, s, h] )

* RtSuppOffer a, s, i, c

ELSE

RtSuppAvail5minAmt a, s, i  =0

(b)	#RtMwpRev5minAmt a, s, i, c =

RtRucComStat5minFlg a, s, i, c  * [ ( ( RtLmp5minPrc s, i

* Min (0, [ RtBillMtr5minQty a, s, i - DaClrdHrlyQty a, s, h ] ) ) / 12 )

+ RtRegUpRev5minAmt a, s, i, c + RtRegDnRev5minAmt a, s, i, c

+ RtSpinRev5minAmt a, s, i, c + RtSuppRev5minAmt a, s, i, c ]

(b.1)	RtRegUpRev5minAmt a, s, i, c =

(-1) * RtRucComStat5minFlg a, s, i, c

* ( Max ( 0, [ RtRegUp5minQty a, z, s, i   - DaRegUpHrlyQty a, z, s, h] )

* RtRegUpMcp5minPrc z, i  ) / 12

(b.2)	RtRegDnRev5minAmt a, s, i, c =

(-1) * RtRucComStat5minFlg a, s, i, c

*( Max ( 0, [ RtRegDn5minQty a, z, s, i  - DaRegDnHrlyQty a, z, s, h] )

* RtRegDnMcp5minPrc z, i   ) / 12

(b.3)	RtSpinRev5minAmt a, s, i, c =

(-1) * RtRucComStat5minFlg a, s, i, c

*( Max ( 0, [ RtSpin5minQty a, z, s, i   - DaSpinHrlyQty a, z, s, h ] )

* RtSpinMcp5minPrc z, i ) / 12

(b.4)	RtSuppRev5minAmt a, s, i, c =

(-1) * RtRucComStat5minFlg a, s, i, c

*( Max ( 0, [ RtSupp5minQty a, z, s, i   - DaSuppHrlyQty a, z, s, h ] )

* RtSuppMcp5minPrc z, i   ) / 12

(c)	#CncldStartAmt a, s, c =

( RtStartUp5minAmt a, s, i, c  * RtStartUpElig5minFlg a, s, i, c )

* CncldStartRatio a, s, c

CncldStartRatio a, s, c = (ElapsedTime a, s, c  / StartUpTime a, s, c )

(d) In any Dispatch Interval in which the Resource has operated outside of its Operating Tolerance and that Resource has not been exempted from URD per Section 4.4.4.1, any incremental Energy costs associated with actual Energy output above the Resource’s Desired Dispatch is not eligible for recovery.  The URD adjustment is calculated as follows:

IF ABS (URD5minQty a, s, i ) > ResOpTol5minQty a, s, i AND

( XmptDev5minFlg a, s, i =  0 )

THEN

#RtURDAdj5minAmt a, s, i, c = RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i )) / 12

ELSE

RtURDAdj5minAmt a, s, i, c = 0

(d.1)	URD5minQty a, s, i =

( RtBillMtr5minQty a, s, i * (-1)) - RtAvgSetPoint5minQty a, s, i

(d.2)	ResOpTol5minQty a, s, i =

Min ( URDMaxTol5minQty i , Max (URDMinTol5minQty i ,

URDTol5minPct i * RtDispMaxEmerCapOL5minQty a, s, i ) )

(d.3)	IF RtDesiredEn5minQty a, s, i < ABS (DaClrdHrlyQty a, s, h )

THEN

#RtDesiredEn5minAmt a, s, i = RtIncrEn5minAmt a, s, i

ELSE

#RtDesiredEn5minAmt a, s, i =

Where:

X = Max (ABS (DaClrdHrlyQty a,  s, h ) , RtEffMin5minQty a, s, i )

Y = Max (X, RtDesiredEn5minQtya, s, i )

(e) In any Dispatch Interval in which a Resource is in “Manual” status, any incremental Energy costs associated with actual Energy output above the Resource’s Desired Dispatch is not eligible for recovery.  The status change adjustment is calculated as follows:

IF ControlStatus5minFlg a, s, i = “Manual”

AND ABS (URD5minQty a, s, i ) <= ResOpTol5minQty a, s, i

THEN

#RtStatusAdj5minAmt a, s, i, c = RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i )) / 12

ELSE

RtStatusAdj5minAmt a, s, i, c = 0

(f)	In any Dispatch Interval in which a Resource has increased its Minimum Economic Capacity Operating Limit (or its Minimum Regulation Capacity Operating Limit if the Resource has cleared for Regulation-Up or Regulation-Down) above the Resource’s minimum limits used by SPP in the commitment decision or the minimum limits used to move from one configuration to another in the case of a Combined Cycle Resource, the Resource is not in “Manual” status and the increase in minimum limit is greater than the Resource’s Operating Tolerance, any incremental Energy costs associated with actual Energy output above the Resource’s Desired Dispatch is not eligible for recovery.  The limit change adjustment is calculated as follows:

IF ControlStatus5minFlg a, s, i < > “Regulating” AND

ControlStatus5minFlg a, s, i  < > “Manual”  AND

( RtDispMinEconCapOL5minQty a, s, i

- RtComMinEconCapOL5minQty a, s, i ) > ResOpTol5minQty a, s, i  AND

ABS (URD5minQty a, s, i ) <= ResOpTol5minQty a, s, i

THEN

#RtLimitAdj5minAmt a, s, i, c =   RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i ) / 12

ELSE IF

ControlStatus5minFlg a, s, i  = “Regulating”  AND

( RtDispMinRegCapOL5minQty a, s, i

- RtComMinRegCapOL5minQty a, s, i ) > ResOpTol5minQty a, s, i  AND

ABS (URD5minQty a, s, i ) < =ResOpTol5minQty a, s, i

THEN

#RtLimitAdj5minAmt a, s, i, c =  RtRucComStat5minFlg a, s, i, c

* Max ( 0, ( RtIncrEn5minAmt a, s, i – RtDesiredEn5minAmt a, s, i )) / 12

ELSE

RtLimitAdj5minAmt a, s, i, c = 0

- For each Asset Owner, a daily amount is calculated at each Settlement Location.  The daily amount is calculated as follows:

RtMwpDlyAmt a, s, d =   RtMwpCpAmt a, s, c

- For each Asset Owner associated with Market Participant m, a daily amount is calculated.  The daily amount is calculated as follows:

RtMwpAoAmt a, m, d =  RtMwpDlyAmt a, s, d

- For each Market Participant, a daily amount is calculated representing the sum of Asset Owner amounts associated with that Market Participant.  The daily amount is calculated as follows:

RtMwpMpAmt m, d =  RtMwpAoAmt a, m, d

(8)	For FERC Electric Quarterly Reporting (“EQR”) purposes, SPP calculates RUC Make-Whole Payment $ per RUC Make-Whole-Payment Eligibility Period for each Asset Owner as follows:

(a)	EqrRtMwp5minPrc a, s, c = (-1) * RtMwpCpAmt a, s, c

(b)	IF EqrRtMwp5minPrc a, s, c > 0

THEN

EqrRtMwp5minQty a, s, c = 1

The above variables are defined as follows:

| Variable | Unit | Settlement Interval | Definition |
| --- | --- | --- | --- |
| RtMwpCpAmt a, s, c | $ | Eligibility Period | RUC Make-Whole-Payment Amount per AO per Settlement Location per RUC Make-Whole-Payment Eligibility Period - The amount to AO a for RUC Make-Whole-Payment Eligibility Period c at Resource Settlement Location s.. |
| DaClrdHrlyQty a, s, h | MWh | Hour | Day-Ahead Cleared Energy Quantity per AO per Settlement Location per Hour - The value described under Section 4.5.8.1 for AO a’s combined cycle resource at Settlement Location s for the Hour. |
| RtStartUp5minAmt a s, i, c | $ | Eligibility Period | Real-Time Start-Up Cost Amount per AO per Settlement Location per Dispatch Interval per RUC Make-Whole-Payment Eligibility Period - The RTBM Start-Up Offer associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. This value is calculated by dividing RtStartUpAmt a s, c by the lesser of the Resource’s (RtMinRunTime a, i, s, c *12) or (24 * 12). These interval values are carried forward into the following Operating Day, if needed, to ensure recovery of any remaining RtStartUpAmt a s, c. |
| RtStartUpAmt a s, c (Not Available on Settlement Statement) | $ | Eligibility Period | Real-Time Start-Up Cost Amount per AO per Settlement Location per RUC Make-Whole-Payment Eligibility Period - The RTBM Start-Up Offer used in the commitment decision, associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c. |
| RtStartUpElig5minFlg a, s, i, c | None | Dispatch Interval | RUC Start-Up Recovery Eligibility Flag per AO per Resource Settlement Location per Dispatch Interval per RUC Make-Whole-Payment Eligibility Period – This flag is set equal to 1 in each Dispatch Interval of a RUC Make-Whole-Payment Eligibility Period where the Resource is eligible to recover start-up costs, or 0 where the Resource is not eligible to recover start-up costs. |
| RtRucComStat5minFlg a, s, i, c | None | Dispatch Interval | RUC Commitment Status Flag per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – This flag is set equal to 1 for each Dispatch Interval of a RUC Make-Whole-Payment Eligibility Period in which a Resource’s Commitment Status was “Market” or “Reliability”, or 0 if its Commitment Status was “Self”. |
| CncldStartRatio a, s, c | None |  | Canceled Start Ratio per Resource Settlement Location in RUC Make-Whole-Payment Eligibility Period – The ratio of ElapsedTime a, s, c to StartUpTime a, s, c as calculated for each Dispatch Interval in RUC Make-Whole-Payment Eligibility Period c. |
| RtMinRunTime a, i, s, c | Time | Dispatch Interval | Real-Time Minimum Run Time per AO per Settlement Location Per Dispatch Interval per RUC Make-Whole-Payment Eligibility Period – The Minimum Run Time used in the commitment decision, associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c as submitted as part of the RTBM Market Offer. |
| RtSynchToMinTime a, i, s, c | Time | Dispatch Interval | Real-Time Synch To Minimum Time per AO per Settlement Location Per Dispatch Interval per RUC Make-Whole-Payment Eligibility Period – The Synch To Minimum Time used in determining Start-Up Recovery Eligibility, associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c as submitted as part of the RTBM Market Offer. |
| RtNoLoad5minAmt a, i, s, c | $ | Dispatch Interval | Real-Time No-Load Cost Amount per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period - The No-Load Offer used in the commitment decision, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtMwpCost5minAmt a, s, i, c | $ | Dispatch Interval | RUC Make-Whole-Payment Cost per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period – The total Energy and Operating Reserve cost at actual Resource output, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtMwpRev5minAmt a, s, i, c | $ | Dispatch Interval | RUC Make-Whole-Payment Revenue per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period – The total Energy and Operating Reserve revenue at actual Resource output, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| CncldStartAmt a, s, c | $ | Eligibility Period | Real-Time Cancelled Start Amount per AO per Settlement Location per for the RUC Make-Whole-Payment Eligibility Period – The Start-Up Offer cost reimbursement for an SPP cancelled start-up, in dollars, associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c. |
| ElapsedTime a, s, c | Time | Eligibility Period | Elapsed Time per AO per Settlement Location per for the RUC Make-Whole-Payment Eligibility Period – The elapsed time, in minutes, between the start of a Resource’s StartUpTime a, s, c and the time SPP cancelled the start-up, in dollars, associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c. |
| StartUpTime a, s, c | Time | Eligibility Period | Start-up Time per AO per Settlement Location for the RUC Make-Whole-Payment Eligibility Period – The Start-Up Time, in minutes, used in the commitment decision associated with AO a’s eligible Resource at Settlement Location s for RUC Make-Whole-Payment Eligibility Period c as specified in the RTBM Offer submitted prior to the RUC Make-Whole-Payment Eligibility Period. |
| RtURDAdj5minAmt a, s, i, c | $ | Dispatch Interval | URD Adjustment per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period – The reduction in RUC Make-Whole Payment Amount associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c when the Resource’s URD5minQty a, s, i is outside of the Resource’s ResOpTol5minQty a, s, i. |
| URD5minQty a, s, i | MW | Dispatch Interval | Uninstructed Resource Deviation per AO per Settlement Location per Dispatch Interval – The Uninstructed Resource Deviation associated with AO a’s Resource at Settlement Location s in Dispatch Interval i. |
| ResOpTol5minQty a, s, i | MW | Dispatch Interval | Resource Operating Tolerance per AO per Settlement Location per Dispatch Interval – The Resource Operating Tolerance associated with AO a’s Resource at Settlement Location s in Dispatch Interval i. |
| URDMaxTol5minQty i | MW | Dispatch Interval | Uninstructed Resource Deviation Maximum Tolerance per Dispatch Interval – The maximum value of ResOpTol5minQty a, s, i that is currently set at 20 MW. |
| URDMinTol5minQty i | MW | Dispatch Interval | Uninstructed Resource Deviation Minimum Tolerance per Dispatch Interval – The minimum value of ResOpTol5minQty a, s, i that is currently set at 5 MW. |
| URDTol5minPct i | Percent | Dispatch Interval | Uninstructed Resource Deviation Tolerance Percentage per Dispatch Interval – The percentage used to calculate the value of ResOpTol5minQty a, s, i that is currently set at 5%. |
| RtAvgSetPoint5minQty a, s, i | MW | Dispatch Interval | Real-Time Average Setpoint Instruction MW per AO per Settlement Location per Dispatch Interval – The average Setpoint Instruction over Dispatch Interval i for AO a’s Resource at Settlement Location s. |
| XmptDev5minFlg a, s, i | none | Dispatch Interval | URD Exemption Flag per AO per Resource Settlement Location per Dispatch Interval – A flag associated with AO a’s eligible Resource at Settlement Location s indicating that a Resource that has operated outside of its Operating Tolerance is or is not exempt from any associated penalty charges in Dispatch Interval i. If the flag is equal to zero, the Resource is not exempt. Otherwise, the flag will be set to a positive integer number which will indicate the reason of the exemption as specified under Section 4.4.4.1. |
| RtStatusAdj5minAmt a, s, i, c | $ | Dispatch Interval | Resource Status Change Adjustment per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period – The reduction in RUC Make-Whole Payment Amount associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c when the Resource’s Control Status is set to “Manual”. |
| ControlStatus5minFlg a, s, i | None | Dispatch Interval | Control Status per AO per Settlement Location per Dispatch Interval – A Resource status indicator associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i as set by SPP operators that indicates the current dispatchable status of the Resource. |
| RtDispMaxEmerCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Maximum Emergency Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The Maximum Emergency Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i. |
| RtEffMin5minQty a, s, i | MW | Dispatch Interval | Real-Time Effective Minimum Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The Effective Minimum Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i. |
| RtDispMinEconCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Minimum Economic Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The Minimum Economic Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i. |
| RtDispMinRegCapOL5minQty a, s, i | MW | Dispatch Interval | Real-Time Minimum Regulation Capacity Operating Limit Quantity per AO per Settlement Location per Dispatch Interval – The Minimum Regulation Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i. |
| RtLimitAdj5minAmt a, s, i, c | $ | Dispatch Interval | Resource Limit Change Adjustment per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period – The reduction in RUC Make-Whole Payment Amount associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c for a Real-Time increase in minimum limit. |
| RtComMinEconCapOL5minQty a, s, i | MW | Eligibility Period | Real-Time Minimum Economic Capacity Operating Limit Quantity per AO per Settlement Location – The Minimum Economic Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i as submitted in an RTBM Offer prior to the RUC Make-Whole-Payment Eligibility Period that was used in making the initial Resource commitment decision. |
| RtComMinRegCapOL5minQty a, s, i | MW | Eligibility Period | Real-Time Minimum Regulation Capacity Operating Limit Quantity per AO per Settlement Location– The Minimum Regulation Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i as submitted in an RTBM Offer prior to the RUC Make-Whole-Payment Eligibility Period that was used in making the initial Resource commitment decision. |
| RtIncrEn5minAmt a, s, i | $ | Dispatch Interval | Real-Time Incremental Energy Cost Amount per AO per Settlement Location per Dispatch Interval - The average incremental energy offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i from the Effective Minimum Capacity Operating Limit to RtBillMtr5minQty a, s, i. |
| RtMinEn5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Energy Cost at Minimum Limit per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period - The average incremental energy offer cost at the Effective Minimum Capacity Operating Limit associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c |
| RtDesiredEn5minAmt a, s, i | $ | Dispatch Interval | Real-Time Energy Cost at Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval - The average incremental energy offer cost associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i, in dollars, from the Effective Minimum Capacity Operating Limit to RtDesiredEn5minQty a, s, i. |
| RtDesiredEn5minQty a, s, i | MW | Dispatch Interval | Real-Time Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval – The Desired Dispatch MW for AO a’s eligible Resource for Dispatch Interval i at RtLmp5minPrc s, i as calculated from the Resource’s As Dispatched Energy Offer Curve using the As-Committed Minimum Capacity Limit (Economic or Regulating, as applicable) as an output floor and the As-Committed Maximum Capacity Limit (Economic or Regulating, as applicable) as an output ceiling. |
| RtOom5minAmt a, s, i | $ | Dispatch Interval | Real-Time Out-Of-Merit Make-Whole-Payment Amount per AO per Settlement Location per Dispatch Interval - The value calculated under Section 4.5.9.9. |
| RtRegAdj5minAmt a, s, i | $ | Dispatch Interval | Real-Time Regulation Deployment Adjustment Amount per AO per Resource Settlement Location per Dispatch Interval - The value calculated under Section 4.5.9.19. |
| RtRegUpOffer a, s, i, c (Not Available on Settlement Statement) | $/MW | Dispatch Interval | Real-Time Regulation-Up Offer per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Regulation-Up Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtRegDnOffer a, s, i, c (Not Available on Settlement Statement) | $/MW | Dispatch Interval | Real-Time Regulation-Down Offer per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Regulation-Down Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtSpinOffer a, s, i, c (Not Available on Settlement Statement) | $/MW | Dispatch Interval | Real-Time Spinning Reserve Offer per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Spinning Reserve Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtSuppOffer a, s, i, c (Not Available on Settlement Statement) | $/MW | Dispatch Interval | Real-Time Supplemental Reserve Offer per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Supplemental Reserve Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtFixedRegUp5minQty a, s, i | MW | Dispatch Interval | Real-Time Fixed Regulation-Up Quantity per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Fixed Regulation-Up MW specified in the Regulation-Up Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtFixedRegDn5minQty a, s, i | MW | Dispatch Interval | Real-Time Fixed Regulation-Down Quantity per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Fixed Regulation-Down MW specified in the Regulation-Down Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtFixedSpin5minQty a, s, i | MW | Dispatch Interval | Real-Time Fixed Spinning Reserve Quantity per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Fixed Spinning Reserve MW specified in the Spinning Reserve Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtFixedSupp5minQty a, s, i | MW | Dispatch Interval | Real-Time Fixed Supplemental Reserve Quantity per AO per Resource Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Fixed Supplemental Reserve MW specified in the Supplemental Reserve Offer associated with AO a’s Resource Settlement Location s at the time the commitment decision was made for RUC Make-Whole-Payment Eligibility Period c in Dispatch Interval i. |
| RtRegUpAvail5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Regulation-Up Offer Cost Amount per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period - The Regulation-Up Offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtRegDnAvail5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Regulation-Down Offer Cost Amount per AO per Settlement Location per Dispatch Interval in the RUC Make-Whole-Payment Eligibility Period - The Regulation-Down Offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in DA Market Commitment Period c. |
| RtSpinAvail5minAmt a, s, i, c | c | Dispatch Interval | Real-Time Spin Offer Cost Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period - The Spinning Reserve Offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtSuppAvail5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Supplemental Offer Cost Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period - The Supplemental Reserve Offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtLmp5minPrc s, i | $/MWh | Dispatch Interval | Real-Time LMP - The value defined under Section 4.5.9.1 at Settlement Location s for Dispatch Interval i. |
| RtBillMtr5minQty a, s, i | MW | Dispatch Interval | Real-Time Actual Meter Quantity per AO per Location per Dispatch Interval - The value defined under Section 4.5.9.1 for Dispatch Interval i. |
| RtRegUpRev5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Regulation-Up Revenue Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Real-Time incremental Regulation-Up revenue associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtRegDnRev5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Regulation-Down Revenue Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Real-Time incremental Regulation-Down revenue associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtSpinRev5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Spinning Reserve Revenue Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Real-Time incremental Spinning Reserve associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtSuppRev5minAmt a, s, i, c | $ | Dispatch Interval | Real-Time Supplemental Reserve Revenue Amount per AO per Settlement Location per Dispatch Interval in RUC Make-Whole-Payment Eligibility Period – The Real-Time incremental Supplemental Reserve revenue associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i in RUC Make-Whole-Payment Eligibility Period c. |
| RtMwpDlyAmt a, s, d | $ | Operating Day | RUC Make-Whole-Payment Amount per AO per Settlement Location per Operating Day - The RUC Make-whole amount to AO a for Operating Day d at Resource Settlement Location s. |
| RtMwpAoAmt a, m, d | $ | Operating Day | RUC Make-Whole-Payment Amount per AO per Operating Day - The RUC Make-whole amount to AO a associated with Market Participant m for Operating Day d. |
| RtMwpMpAmt m, d | $ | Operating Day | RUC Make-Whole-Payment Amount per MP per Operating Day - The RUC Make-whole amount to Market Participant m for Operating Day d. |
| EqrRtMwp5minPrc a, s, c | $ | Eligibility Period | RUC Electric Quarterly Reporting Make-Whole-Payment Amount per AO per Settlement Location per RUC Make-Whole-Payment Eligibility Period - The RUC make-whole amount to AO a for RUC Make-Whole-Payment Eligibility Period c at Resource Settlement Location s for use by AO a in reporting such make-whole-payments to FERC in accordance with FERC EQR requirements.. |
| EqrRtMwp5minQty a, s, c | $ | Eligibility Period | RUC Electric Quarterly Reporting Make-Whole-Payment Quantity per AO per Settlement Location per RUC Make-Whole-Payment Eligibility Period – This value is set equal to 1 if EqrRtMwp5minPrc a, s, c > 0 for use by AO a in reporting such make-whole-payments to FERC in accordance with FERC EQR requirements.. |
| a | none | none | An Asset Owner. |
| i | none | none | A Dispatch Interval. |
| h | none | none | An Hour. |
| d |  |  | An Operating Day. |
| s | none | none | A Settlement Location. |
| c | none | none | A RUC Make-Whole-Payment Eligibility Period. |
| m | none | none | A Market Participant. |

Proposed Tariff Language Revision

N/A

Proposed Criteria Language Revision

N/A

Note that this charge type will almost always produce a credit.  The charge is included here for the rare occasion when a charge may be produced as a result of a data error and/or a resettlement.