EIS Market Integrated Marketplace

Protocols Revision Request

| PRR No. | Marketplace-PRR89 | PRR Title | Net Benefits Test (Order No. 745 Compliance) |
| --- | --- | --- | --- |
| Protocol Section(s) Requiring Revision |  | Section No.: 4.1.9 (new) Title: Calculation of Net Benefits Test for Compensation of Controllable Load Protocol Version: 11.0 |  |
| Impact Analysis Required |  | Yes – If yes, estimated cost: No SPP Staff will complete this section. |  |
| Requested Resolution |  | Normal Expedited Urgent Action Provide explanation if Expedited and/or Urgent Action is selected: |  |
| Type of Revision |  | Correction/Clean-Up Clarification Design Enhancement Design Change |  |
| Revision Description |  | This MPRR adds language consistent with language proposed in the Tariff for the EIS Market. The language outlines the steps undertaken by SPP in calculating the net benefits test for compensating controllable load consistent with FERC Order No. 745. |  |
| Reason for Revision |  | In order to comply with FERC Order 745, SPP filed language in the EIS Market on May 2, 2012 outlining the steps that SPP will take to calculate the net benefits test monthly. The net benefits test will also be required for the Integrated Marketplace and while there has not been final acceptance of that language for the EIS Market, provisions must be made for inclusion in the Integrated Marketplace Protocols and Tariff. |  |
| Tariff Implications or Changes |  | Yes – Section No.: (Include a summary of impact and/or specific changes) Attachment AE, section 3.8 (new) No |  |
| Criteria Implications or Changes |  | Yes - Section No.: (Include a summary of impact and/or specific changes) No |  |
| Credit Implications |  | Yes (Include a summary of impact and/or specific changes) No |  |
| Date |  | 8/10/2012 |  |
| Sponsor |  |  |  |
| Name |  | Patti Kelly |  |
| E-mail Address |  | pkelly@spp.org |  |
| Company |  | Southwest Power Pool |  |
| Phone Number |  | 501.614.3381 |  |

Proposed Protocol Language Revision

4.1.9	Calculation of Net Benefits Test for Compensation of Controllable Load

The Transmission Provider shall identify each month the price on a supply curve, representative of economic conditions expected for that month, at which the benefits of dispatching Controllable Load exceed the costs of the load reductions to other loads. In formulaic terms, the net benefit is deemed to be realized at the price point on the supply curve where (Delta LIP x MWh consumed) > (LIP NEW x CL) where LIP NEW is the market clearing price after the Controllable Load is dispatched and Delta LIP is the price before Controllable Load is dispatched minus the LIP NEW.

The Transmission Provider shall update and post the Net Benefits Test results and analysis for a calendar month no later than the 15th day of the preceding calendar month. The price level from the Net Benefits Test shall be calculated using the following steps:

Step 1: Retrieve the Energy Offer Curves for the on-peak hours of the same calendar month (of the prior calendar year) for which the calculation is being performed. In the case that an Energy Offer Curve is not available for a Resource, the Mitigated Energy Offer Curve will be used.

Step 2: Adjust a portion of each prior-year offer representing the typical share of fuel costs in energy offers in the SPP Region for changes in fuel prices based on the ratio of the reference month spot price to the study month forward price.

Step 3: Combine the offers to create an hourly supply curve for each on-peak hour in the period and compute an average supply curve for the period.

Step 4: Smooth the average supply curve by fitting the following function to the raw data using a non-linear, least-squares regression:

P(MW) = A + B * MW + C * MW2 + D * MW3 +  e(E*MW+F)

where P(MW) is the EIS Market offer price in $/MWh, and MW is cumulative capacity. A through F are the parameters to be estimated.

Step 5: Compute the price elasticity of the smoothed average supply curve at each MW point

Step 6: Find the threshold price for the smoothed average supply curve at which elasticity falls below one for the duration of the curve. This is the Net Benefits Test threshold price for the month.

Proposed Tariff Language Revision

In Section 3 of Attachment AE:

(new section – all language is new)

3.8	   Calculation of Net Benefits Test for Compensation of Controllable Load

The Transmission Provider shall identify each month the price on a supply curve, representative of economic conditions expected for that month, at which the benefits of dispatching Controllable Load exceed the costs of the load reductions to other loads. In formulaic terms, the net benefit is deemed to be realized at the price point on the supply curve where (Delta LIP x MWh consumed) > (LIP NEW x CL) where LIP NEW is the market clearing price after the Controllable Load is dispatched and Delta LIP is the price before Controllable Load is dispatched minus the LIP NEW.

The Transmission Provider shall update and post the Net Benefits Test results and analysis for a calendar month no later than the 15th day of the preceding calendar month. The price level from the Net Benefits Test shall be calculated using the following steps:

Step 1: Retrieve the Energy Offer Curves for the on-peak hours of the same calendar month (of the prior calendar year) for which the calculation is being performed.  In the case that an Energy Offer Curve is not available for a Resource, the Mitigated Energy Offer Curve will be used.

Step 2: Adjust a portion of each prior-year offer representing the typical share of fuel costs in energy offers in the SPP Region for changes in fuel prices based on the ratio of the reference month spot price to the study month forward price.

Step 3: Combine the offers to create an hourly supply curve for each on-peak hour in the period and compute an average supply curve for the period.

Step 4: Smooth the average supply curve by fitting the following function to the raw data using a non-linear, least-squares regression:

P(MW) = A + B * MW + C * MW2 + D * MW3 +  e(E*MW+F)

where P(MW) is the EIS Market offer price in $/MWh, and MW is cumulative capacity. A through F are the parameters to be estimated.

Step 5: Compute the price elasticity of the smoothed average supply curve at each MW point

Step 6: Find the threshold price for smoothed average supply curve at which elasticity falls below one for the duration of the curve. This is the Net Benefits Test threshold price for the month.

Proposed Criteria Language Revision

N/A