EIS Market Integrated Marketplace

PRR Recommendation Report

| PRR No. | Marketplace-PRR89 | PRR Title | Net Benefits Test (Order No. 745 Compliance) |
| --- | --- | --- | --- |
| Timeline |  | Normal Expedited Urgent Action Provide explanation if Expedited and/or Urgent Action is selected: |  |
| Recommendation Action |  | Approve Reject Require additional information Defer Refer |  |
| Impact Analysis Required |  | Yes – If yes, estimated cost: No SPP Staff will complete this section. |  |
| Protocol Section(s) Requiring Revision |  | Section No.: 4.1.9 (new) Title: Calculation of Net Benefits Test for Compensation of Controllable Load Protocol Version: 11.0 |  |
| Type of Revision |  | Correction/Clean-Up Clarification Design Enhancement Design Change |  |
| Revision Description |  | This MPRR adds language consistent with language proposed in the Tariff for the EIS Market. The language outlines the steps undertaken by SPP in calculating the net benefits test for compensating controllable load consistent with FERC Order No. 745. |  |
| Tariff Implications or Changes |  | Yes – Section No: (Include a summary of impact and/or specific changes) Attachment AE, section 3.8 (new) No |  |
| MWG Review PRR Recommendation |  | Date of Vote: 8/14/2012—Unanimously approved with modifications All Segments present for the vote: Yes No Segment of Parties that voted No or Abstained: N/A |  |
| RTWG Review |  | 10/25/2012—Approved with modifications |  |
| ORWG Review |  | 9/12/2012—Approved with no Reliability Impact |  |
| MOPC Recommendation |  | 10/16/2012—Approved |  |
| Board Review |  | 10/30/2012—Approved |  |

| Date |  |  |  | 8/10/2012 |  |
| --- | --- | --- | --- | --- | --- |
| Sponsor |  |  |  |  |  |
| Name |  |  |  | Patti Kelly |  |
| E-mail Address |  |  |  | pkelly@spp.org |  |
| Company |  |  |  | Southwest Power Pool |  |
| Phone Number |  |  |  | 501.614.3381 |  |
| Comments Received |  |  |  |  |  |
| Comment Author |  |  | MWG |  |  |
| Date |  |  | 8/14/2012 |  |  |
| Comment Description |  |  | The steps in the Tariff were removed since they were already in the Protocols. Other Typos were corrected. |  |  |
| Comment Status |  |  | Comments were taken into consideration. The approved language is reflected in this recommendation report. |  |  |
| Comments Received |  |  |  |  |  |
| Comment Author |  |  | Brenda Fricano on behalf of RTWG |  |  |
| Date |  |  | 10/10/2012 |  |  |
| Comment Description |  |  | I think the members of the RTWG on the call would state that the Tariff language, as revised by the RTWG, implements the proposed Protocol change, however, the RTWG also believes that the MPRR is incomplete and does not fully meet the FERC Order since it does not specifically state how the test is going to be used. No Protocol language was changed. |  |  |
| Comment Status |  |  | Comments were taken into consideration. The approved language is reflected in this recommendation report. |  |  |

| Comments Received |  |
| --- | --- |
| Comment Author | Brenda Fricano on behalf of RTWG |
| Date | 10/25/2012 |
| Comment Description | Tariff language was altered to make it less detailed since the Protocols have the detail. A sentence was added to clarify how the Net Benefits Test will be used. |
| Comment Status |  |

Proposed Protocol Language Revision

4.1.9	Calculation of Net Benefits Test for Compensation of Controllable Load

The Transmission Provider shall identify each month the price on a supply curve, representative of economic conditions expected for that month, at which the benefits of dispatching Controllable Load exceed the costs of the load reductions to other loads. In formulaic terms, the net benefit is deemed to be realized at the price point on the supply curve where (Delta LMP x MWh consumed) > (LMP NEW x CL) where LMP NEW is the market clearing price after the Controllable Load is dispatched and Delta LMP is the price before Controllable Load is dispatched minus the LMP NEW.

The Transmission Provider shall update and post the Net Benefits Test results and analysis for a calendar month no later than the 15th day of the preceding calendar month. The price level from the Net Benefits Test shall be calculated using the following steps:

Step 1: Retrieve the Energy Offer Curves for the on-peak hours of the same calendar month (of the prior calendar year) for which the calculation is being performed. In the case that an Energy Offer Curve is not available for a Resource, the Mitigated Energy Offer Curve will be used.

Step 2: Adjust a portion of each prior-year offer representing the typical share of fuel costs in energy offers in the SPP Region for changes in fuel prices based on the ratio of the reference month spot price to the study month forward price.

Step 3: Combine the offers to create an hourly supply curve for each on-peak hour in the period and compute an average supply curve for the period.

Step 4: Smooth the average supply curve by fitting the following function to the raw data using a non-linear, least-squares regression:

P(MW) = A + B * MW + C * MW2 + D * MW3 +  e(E*MW+F)

where P(MW) is the  Market offer price in $/MWh, and MW is cumulative capacity. A through F are the parameters to be estimated.

Step 5: Compute the price elasticity of the smoothed average supply curve at each MW point

Step 6: Find the threshold price for the smoothed average supply curve at which elasticity falls below one for the duration of the curve. This is the Net Benefits Test threshold price for the month.

Proposed Tariff Language Revision

In Section 3 of Attachment AE:  (new section – all language is new)

3.8	   Calculation of Net Benefits Test for Compensation of Demand Response Resources

The Transmission Provider shall calculate the price on a supply curve, representative of economic conditions expected for that month, at which the benefits will exceed the costs of dispatching Demand Response Resources..

The Transmission Provider shall update and post the Net Benefits Test results and analysis for a calendar month no later than the 15th day of the preceding calendar month.  In any period in which the Demand Response Resource is economically dispatched and meets or exceeds the pricing point determined by the Net Benefits Test, the Demand Response Resource will be paid the LMP at its Settlement Location.

Proposed Criteria Language Revision

N/A