PRR Recommendation Report

| MPRR No. | 200 | PRR Title | Clean-up and Corrections |
| --- | --- | --- | --- |
| Timeline |  | Normal Expedited Urgent Action Provide explanation if Expedited and/or Urgent Action is selected: |  |
| Recommendation Action |  | Approve Reject Require additional information Defer Refer |  |
| Ranking |  | N/A |  |
| Impact Analysis Required |  | Yes, Estimated Cost: Duration: No SPP Staff will complete this section. |  |
| Protocol Section(s) Requiring Revision |  | Section No.: , 4.4.3.3; 4.5.8.8; 4.5.8.22; 4.5.8.16; 4.5.9.3; 4.5.9.9; 4.5.9.10; 4.5.9.25 Title: Regulation Deployment; Day-Ahead Regulation-Up Service Distribution Amount; Day-Ahead Demand Reduction Distribution Amount; Transmission Congestion Rights Monthly Payback Amount; Real-Time Virtual Energy Amount; Real-Time Out-Of-Merit Amount; RUC Make-Whole-Payment Distribution Amount; Real-Time Demand Reduction Distribution Amount Protocol Version: 20.b |  |
| Type of Revision |  | Correction/Clean-Up Clarification Design Enhancement Design Change |  |
| Revision Description |  | 1) 4.4.3.3 The language currently says that the Real-Time Balancing Market (RTBM) assigns the priorities, but it is actually the Intra-Day Reliability Unit Commitment (ID RUC) process that assigns regulation priority. If any Resources do not get assigned a priority group by ID RUC, RTBM assigns those Resources to the lowest priority group. 2) 4.4.3.3 Currently, the Protocols are not clear how regulating Resources are assigned to a priority group when neither reserve zone nor ramp rate are considered. It is also not clear how long that assignment lasts. Clarification is made that Resources are randomly assigned for an ID RUC interval (one hour) in this situation. 3) 4.4.3.3 Clarification that SPP will “attain approval” for changes to Regulation Deployment from MWG, ORWG and MOPC. 4) 4.5.8.8 Summation signs were added to show the summation across MP, AO, and SL for the Real-Time SPP Load. 5) 4.5.8.22 Day-Ahead Demand Reduction Distribution Amount variable was missing from the definition table 6) 4.5.8.16 # added to Excess Congestion Fund Amount to defined rounding 7) 4.5.9.3 The Settlement Interval for Real-Time Virtual Energy Amount and Real-Time LMP are updated to reflect “Dispatch Interval” and not “Hour” 8) 4.5.9.9 Correction made to the billing determinate name Real-Time OOME Energy Cost at desired Dispatch Quantity in the definition table 9) 4.5.9.10 MPRR80 broke the reference to the definition for the billing determinate Real-Time Desired Dispatch Quantity. The added language is the definition for the determinate. 10) 4.5.9.25 Real-Time Demand Reduction Distribution Amount variable was missing from the definition table 11) 9.2.7 The MOPC section of the PRR process currently has incorrect statements regarding gray-boxing and estimating implementation dates. The process currently used gives more beneficial information such as the FERC filing progress and docket number |  |
| Tariff Implications or Changes |  | Yes – Section No: (Include a summary of impact and/or specific changes) No |  |
| Criteria Impact or Changes |  | Yes – Section No: (Include a summary of impact and/or specific changes) No |  |
| Working Group |  | Voting Record |  |
| MWG |  | Date of Vote: 8/22/2014 Vote: Unanimously Approved Opposed: N/A Abstained: N/A |  |
| RTWG |  | Date of Vote: 9/24/2014 Vote: Unanimously Approved with no Tariff Impact |  |
| ORWG |  | Date of Vote: 9/4/2014 Vote: Unanimously Approved with no Reliability Impact |  |
| MOPC |  | Date of Vote: 10/14/2014 Vote: Unanimously Approved |  |
| Board/Members Committee |  | Date of Vote: N/A Vote: N/A |  |

| Date | 7/31/2014 |
| --- | --- |

| Sponsor |  |
| --- | --- |
| Name | Micha Bailey |
| E-mail Address | mcbailey@spp.org |
| Company | Southwest Power Pool |
| Phone Number | 501.688.2522 |

| Comments Received |  |
| --- | --- |
| Comment Author | Micha Bailey on behalf of MWG |
| Date | 8/22/2014 |
| Comment Description | Language in Section 9.2.7 was reverted back to the original Protocol Language. A new MPRR will be written to clarify this section. |
| Comment Status | The MPRR was approved as modified in these comments. The approved language is reflected in this recommendation report. |

Proposed Protocol Language Revision

      - 4.4.3.3	Regulation Deployment

Regulation Deployment is limited to Resources that have cleared Regulation-Up and/or Regulation-Down with a Control Status of “Regulating”.  Regulation-Up and/or Regulation-Down is deployed on specific Resources through Setpoint Instructions via the AGC system.  The deployment is on a pro-rata basis, based upon Regulation-Up and/or Regulation-Down cleared MW and assigned priority group order.  As the Intra-Day RUC is run, it will populate the priority groups for each interval.  Each of the Regulation-Up and Regulation-Down cleared Resources will be assigned to a Regulation-Up and/or Regulation-Down priority group.  If any Resources remain without a priority group assignment after the Intra-Day RUC is run, then RTBM will assign these Resources to the lowest priority group.  The AGC system deploys starting with lowest priority group to highest group in the respective direction when regulation is needed in that direction.  The number of priority groups is a configurable parameter that may be adjusted from time to time.  SPP will analyze the market operations and will recommend to the MWG, ORWG and MOPC the number of priority groups it believes will produce the best operational results.

Resources are assigned to priority groups based on the three criteria below which are independent of each other:

(1)	Reserve Zone in which the resource is located

(2)	The effective Ramp Rate of the resource

(3)	The number of priority groups

The first two criteria are made configurable to be switched ON or OFF independently or in conjunction with each other as deemed necessary for operational purposes.  In the event that more than one priority group is designated for Regulation-Up or Regulation-Down, the third criterion is always in effect.  When the first two criteria are both set to ‘off’ and the third criterion is greater than 1, Resources are randomly assigned to a priority group for an Intra-Day RUC interval.  The current configuration is (1) off, (2) off, (3) = 6.  If SPP's analysis shows that a change to the current configuration needs to occur, SPP will attain approval for such change from the MWG, ORWG and MOPC, including the appropriate compensation and the effective date of the change.  The active configuration will be posted on SPP OASIS.  No Regulation Deployment will occur on Resources that have not cleared Regulation-Up and/or Regulation-Down even if their Control Statuses are set to “Regulating”.

#### 4.5.8.8	Day-Ahead Regulation-Up Service Distribution Amount

- A DA Market charge or credit will be calculated for each Asset Owner for each hour for each Reserve Zone.  The Asset Owner amount within each Reserve Zone will be equal to the net Reserve Zone procurement rate for Regulation-Up Service multiplied by the Asset Owners Regulation-Up obligation within the Reserve Zone.  For the purpose of allocating DA Market Regulation-Up Service procurement costs, all Non-Binding Reserve Zones will be combined into a single Non-Binding Reserve Zone.  The amount to each Asset Owner is calculated as follows:

#DaRegUpDistHrlyAmt a, z, h =

DaRegUpDistHrlyRate z, h * DaRegUpAoObligHrlyQty a, z, h

Where,

(a)	IF DaRegUpObligRznHrlyQty z, h > 0

THEN

#DaRegUpDistHrlyRate z, h =

DaRegUpRznHrlyCost z, h / DaRegUpObligRznHrlyQty z, h

ELSE

DaRegUpDistHrlyRate z, h = 0

(a.1)	DaRegUpObligRznHrlyQty z, h =  DaRegUpAoObligHrlyQty a, z, h

(a.2)	#DaRegUpRznHrlyCost z, h =

Min (DaRegUpRznHrlyQty z, h  , DaRegUpObligRznHrlyQty z, h)

* DaRegUpMcpHrlyPrc z, h

+ Max ( 0, (DaRegUpObligRznHrlyQty z, h - DaRegUpRznHrlyQty z, h  ) )

* DaRegUpSpxHrlyRate  h

(a.2.1)	IF   Max (0, DaRegUpRznHrlyQty z, h  - DaRegUpObligRznHrlyQty z, h) > 0

THEN

#DaRegUpSpxHrlyRate  h =

( Max (0, DaRegUpRznHrlyQty z, h  - DaRegUpObligRznHrlyQty z, h)

* DaRegUpMcpHrlyPrc z, h )

/   Max (0, DaRegUpRznHrlyQty z, h  - DaRegUpObligRznHrlyQty z, h )

ELSE

DaRegUpSpxHrlyRate  h = 0

(a.2.2)	DaRegUpRznHrlyQty z, h  =  DaRegUpHrlyQty a, z, s, h

(b) 	#DaRegUpAoObligHrlyQty a, z, h = DaRegUpSppHrlyQty h

* ( RtRegUpRznLoadHrlyQty a, z, s, h / RtLoadSppHrlyQtyh ) - RegUpFinHrlyQty a, z, h, t

(b.1)	RtLoadSppHrlyQty h =    [ Max ( 0,   RtBillMtr5minQty a, s, i  )

+ Max ( 0,  RtImpExp5minQty a, s, i, t  * (1 – RsgCrdFlgt ) ) ]  / 12

...

#### 4.5.8.22	Day-Ahead Demand Reduction Distribution Amount

…

The above variables are defined as follows:

| Variable | Unit | Settlement Interval | Definition |
| --- | --- | --- | --- |
| DaDRDistHrlyAmt a, s, h | $ | Hour | Day-Ahead Demand Reduction Distribution Amount per AO per Hour - The amount to AO a for AO a’s share of DA Market Demand Reduction costs per Settlement Location per Hour. |
| DaDRLoadHrlyQty a, s, h | MWh | Hour | Day-Ahead Demand Reduction Load per AO per Settlement Location for Hour h – Asset Owner a’s load, virtual withdrawal and Export Interchange Transactions cleared in the DA Market at Settlement Location s for Hour h for use in Demand Reduction cost allocation. |
| DaDRDistHrlyRate h | $/MWh | Hour | Day-Ahead Demand Reduction Distribution Rate per Hour – The rate applied to AO a’s Demand Reduction load in Hour h. |
| DaDRDistHrlyCost h | $ | Hour | Day-Ahead Demand Reduction Distribution Cost per Hour – The cost of Demand Reduction in Hour h. |
| DaDRDistHrlyQty h | MWh | Hour | Day-Ahead Demand Reduction Distribution Quantity per Hour – The total cost allocation quantity for Demand Reduction in Hour h. |
| DaDRHrlyAmt a, s, h | $ | Hour | Day-Ahead Demand Reduction Amount per AO per Settlement Location per Hour - The value described under Section 4.5.8.21. |
| DaClrdHrlyQty a, s, h | MWh | Dispatch Interval | Day-Ahead Cleared Energy Quantity per AO per Settlement Location per Hour in the DA Market – The value described under Section 4.5.8.1 for AO a at Settlement Location s for Hour h. |
| DaClrdVHrlyQty a, s, h, t | MWh | Hour | Day-Ahead Cleared Virtual Energy Quantity per AO per Transaction per Settlement Location per Hour in the DA Market – The value described under Section 4.5.8.3. |
| DaImpExp5minQty a, s, i, t | MWh | Dispatch Interval | Day-Ahead Interchange Transaction Quantity per AO per Transaction per Settlement Location per Dispatch Interval – The value described under Section 4.5.8.2. |
| DaDRDistAoAmt a, m, d | $ | Operating Day | Day-Ahead Demand Reduction Distribution Amount per AO per Operating Day - The DA Market amount to AO a associated with Market Participant m for Demand Reduction for the Operating Day. |
| DaDRDistMpAmt m, d | $ | Operating Day | Day-Ahead Demand Reduction Distribution Amount per Market Participant per Operating Day - The DA Market amount to Market Participant m for Demand Reduction for the Operating Day. |
| DaDRDistDlyAmt a, s, d | $ | Operating Day | Day-Ahead Demand Reduction Distribution Amount per Settlement Location per Operating Day - The DA Market amount to Settlement Location s associated with AO a for Demand Reduction for the Operating Day. |
| a | none | none | An Asset Owner. |
| s | none | none | A Settlement Location. |
| h | none | none | An Hour. |
| d | none | none | An Operating Day. |
| M | none | none | A Market Participant. |

#### 4.5.8.16	Transmission Congestion Rights Monthly Payback Amount

- A DA Market monthly credit or charge will be calculated for each Asset Owner receiving a charge under Section 4.5.8.15 in any Operating Day of the month in order to ensure full funding of TCRs to the extent possible.  The amount is calculated as follows:

#TcrPaybackMnthlyAmt a, mn =

(-1) * Min {TcrUpliftAoMnthlyAmt a, mn, [ ECFMnthlyAmt mn

* TcrUpliftAoMnthlyAmt a, mn / TcrUpliftSppMnthlyAmt mn ] }

Where,

(a)	TcrUpliftAoMnthlyAmt a, mn = TcrUpliftDlyAmt a, d

(b)	TcrUpliftSppMnthlyAmt mn =  TcrUpliftDlyAmt a, d

(c)            # ECFMnthlyAmt mn  =  ECFDlyAmt d

(c.1)	# ECFDlyAmt d  =

Max { 0, [ DaMccHrlyPrc s, h * ( DaClrdHrlyQty a, s, h

+ ( DaImpExp5minQty a, s, i, t  / 12 ) + DaClrdVHrlyQty a, s, h, t ) ]

+   TcrFundHrlyAmt a, h  }

- For each Market Participant, a monthly amount is calculated representing the sum of Asset Owner amounts associated with that Market Participant.  The monthly amount is calculated as follows:

TcrPaybackMnthlyMpAmt m, mn =  TcrPaybackMnthlyAmt a, mn

…

#### 4.5.9.3	Real-Time Virtual Energy Amount

…

The above variables are defined as follows:

| Variable | Unit | Settlement Interval | Definition |
| --- | --- | --- | --- |
| RtVEnergy5minAmt a, s, i | $ | Dispatch Interval | Real-Time Virtual Energy Amount per AO per Settlement Location per Dispatch Interval - The amount to AO a for DA Market cleared Virtual Energy Offers and Virtual Energy Bids at Settlement Location s for each transaction for the Dispatch Interval. |
| RtLmp5minPrc s, i | $/MWh | Dispatch Interval | Real-Time LMP - The value defined under Section 4.5.9.1 at Settlement Location s for Dispatch Interval i. |
| DaClrdVHrlyQty a, s, h, t | MWh | Hour | Day-Ahead Cleared Virtual Energy Quantity per AO per Settlement Location per Hour in the DA Market – The value described under Section 4.5.8.3. |
| RtVEnergyHrlyAmt a, s, h | $ | Hour | Real-Time Virtual Energy Amount per AO per Settlement Location per Hour - The amount to AO a for DA Market cleared Virtual Energy Offers and Virtual Energy Bids at Settlement Location s for the Hour. |
| RtVEnergyDlyAmt a, s, d | $ | Operating Day | Real-Time Virtual Energy Amount per AO per Settlement Location per Operating Day - The amount to AO a for DA Market cleared Virtual Energy Offers and Virtual Energy Bids at Settlement Location s for the Operating Day. |
| RtVEnergyAoAmt a, m, d | $ | Operating Day | Real-Time Virtual Energy Amount per AO per Operating Day - The amount to AO a associated with Market Participant m for DA Market cleared Virtual Energy Offers and Virtual Energy Bids for the Operating Day. |
| RtVEnergyMpAmt m, d | $ | Operating Day | Real-Time Virtual Energy Amount per MP per Operating Day - The amount to MP m for DA Market cleared Virtual Energy Offers and Virtual Energy Bids for the Operating Day. |
| a | none | none | An Asset Owner. |
| s | none | none | A Settlement Location. |
| h | none | none | An Hour. |
| i | none | none | A Dispatch Interval. |
| t | none | none | A single tagged Interchange Transaction, a single virtual energy transaction, a single Bilateral Settlement Schedule, a single contracted Operating Reserve transaction, a single TCR instrument, a single ARR award or a single Reserve Sharing Event transaction. |
| d | none | none | An Operating Day. |
| m | none | none | A Market Participant. |

#### 4.5.9.9	Real-Time Out-Of-Merit Amount

…

| RtOomeDesiredEn5minAmt a, s, i | $ | Dispatch Interval | Real-Time OOME Energy Cost at Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval - The average incremental energy offer cost, in dollars, associated with AO a’s eligible Resource at Settlement Location s for Dispatch Interval i as calculated from the Resource’s As Dispatched Energy Offer Curve from 0 MW to an output level equal to RtOomeDesiredEn5minQty a, s, i. |
| --- | --- | --- | --- |

…

#### 4.5.9.10	RUC Make-Whole-Payment Distribution Amount

…

| RtDispDesiredEn5minQty a, s, i | MW | Dispatch Interval | Real-Time Desired Dispatch Quantity per AO per Settlement Location per Dispatch Interval – The Desired Dispatch MW for AO a’s eligible Resource for Dispatch Interval i at RtLmp5minPrc s, i as calculated from the Resource’s As Dispatched Energy Offer Curve using the As-Dispatched Minimum Capacity Limit (Economic or Regulating, as applicable) as an output floor and the As-Dispatched Maximum Capacity Limit (Economic or Regulating, as applicable) as an output ceiling. |
| --- | --- | --- | --- |

…

#### 4.5.9.25	Real-Time Demand Reduction Distribution Amount

    - …

The above variables are defined as follows:

| Variable | Unit | Settlement Interval | Definition |
| --- | --- | --- | --- |
| RtDRDistHrlyAmt a, s, h | $ | Hour | Real-Time Demand Reduction Distribution Amount per Hour - The amount to AO a for AO a’s share of RTBM Demand Reduction costs per Settlement Location per Hour. |
| RtDRLoadHrlyQty a, s, h | MWh | Hour | Real-Time Demand Reduction Load per AO per Settlement Location for Hour h – Asset Owner a’s load and Export Interchange Transactions in the RTBM at Settlement Location s for Hour h for use in Demand Reduction cost allocation. |
| RtDRDistHrlyRate h | $/MWh | Hour | Real-Time Demand Reduction Distribution Rate per Hour – The rate applied to AO a’s Demand Reduction load in Hour h. |
| RtDRDistHrlyCost h | $ | Hour | Real-Time Demand Reduction Distribution Cost per Hour – The cost of Demand Reduction in Hour h. |
| RtDRDistHrlyQty h | MWh | Hour | Real-Time Demand Reduction Distribution Quantity per Hour – The total zonal cost allocation quantity for Demand Reduction in Hour h. |
| RtDRHrlyAmt a, s, h | $ | Hour | Real-Time Demand Reduction Amount per AO per Settlement Location per Hour - The value described under Section 4.5.9.24. |
| RtBillMtr5minQty a, s, i | MW | Dispatch Interval | Real-Time Billing Meter Quantity per AO per Settlement Location per Dispatch Interval - The value described under Section 4.5.9.1. |
| RtImpExp5minQty a, s, i, t | MW | Dispatch Interval | Real-Time Interchange Transaction Quantity per AO per Settlement Location per Dispatch Interval per Transaction – The value described under Section 4.5.9.2. |
| RsgCrdFlgt (Not Available on Settlement Statement) | none | none | Reserve Sharing Group Contingency Reserve Deployment Flag per Event – The value described under Section 4.5.8.8. |
| RtDRDistAoAmt a, m, d | $ | Operating Day | Real-Time Demand Reduction Distribution Amount per AO per Operating Day - The DA Market amount to AO a associated with Market Participant m for Demand Reduction for the Operating Day. |
| RtDRDistMpAmt m, d | $ | Operating Day | Real-Time Demand Reduction Distribution Amount per Market Participant per Operating Day - The DA Market amount to Market Participant m for Demand Reduction for the Operating Day. |
| RtDRDistDlyAmt a, s, d | $ | Operating Day | Real-Time Demand Reduction Distribution Amount per Settlement Location per Operating Day - The DA Market amount to Settlement Location a associated with AO a for Demand Reduction for the Operating Day. |
| a | none | none | An Asset Owner. |
| s | none | none | A Settlement Location. |
| h | none | none | An Hour. |
| i | none | none | A Dispatch Interval. |
| d | none | none | An Operating Day. |
| m | none | none | A Market Participant. |

Proposed Tariff Language Revision

N/A

Proposed Criteria Language Revision

N/A

Note that this charge type will almost always produce a charge.  The credit is included here for the rare occasion when a credit may be produced as a result of a data error and/or a resettlement.

Note that this charge type will almost always produce a credit.  The charge is included here for the rare occasion when a charge may be produced as a result of a data error and/or a resettlement.