PRR Recommendation Report

| MPRR No. | 210 | PRR Title | Sync to Min Costs for Mitigated Start-Up Offers |
| --- | --- | --- | --- |
| Timeline |  | Normal Expedited Urgent Action Provide explanation if Expedited and/or Urgent Action is selected: |  |
| Recommendation Action |  | Approve Reject Require additional information Defer Refer |  |
| Ranking |  | N/A |  |
| Impact Analysis Required |  | Yes, Estimated Cost: Duration: No SPP Staff will complete this section. |  |
| Protocol Section(s) Requiring Revision |  | Section No.: 8.2.2.4; Appendix G; 2.4.1; 3.4.1; 3.4.2; 3.4.3; 4.4; 4.4.1; 4.4.2; 5.4 Title: Mitigation Measures for Start-Up and No-Load Offers; Mitigated Offer Development Guidelines; Start- Up Offer Definitions; Hot Start Cost; Intermediate Start Cost; Cold Start Cost; Hot Start Cost, Intermediate Start Cost; Cold Start cost; Hot Start Cost; Intermediate Start Cost; Cold Start Cost; Mitigated Start-Up Offer Protocol Version: 20 |  |
| Type of Revision |  | Correction/Clean-Up Clarification Design Enhancement Design Change |  |
| Revision Description |  | When calculating startup costs for mitigated offers, Market Participants are directed to include fuel usage from first fire of start process to breaker closing. This is different from the normal startup offer which includes fuel usage from first fire of start process to the Resource reaching its Minimum Economic Capacity Operating Limit. The difference is due to the mitigated offer removing a double payment that the normal offer allows; a market committed resource is guaranteed its startup costs (through make whole payment) plus it is paid the LMP for energy generated during its Sync-to-Min time. This MPRR proposes to include the fuel costs from synchronization to minimum output and to offset those costs with an estimate of the corresponding market revenues, decreasing the risk we currently operate under in which the LMP may or may not cover all of the startup fuel costs. The following calculations are added to Appendix G : 1. Market Participants calculate the cost of the fuel used between breaker close and the unit reaching it Minimum Economic Capacity Operating Limit 2. Estimate potential market revenues per unit per hour based on the following assumptions: a. A linear increase in the Resource’s megawatt hour output between breaker close and Minimum Economic Capacity Operating Limit b. An LMP price equal to the Resource’s Mitigated Offer at its Minimum Economic Capacity Operating Limit for the entire Sync-to-Min period 3. Fuel Cost (item 1 above) minus LMP Revenue (item 2 above) equals Unrecovered Fuel Cost 4. Unrecovered Fuel Cost may be added to the Resource’s Mitigated Startup Costs |  |
| Tariff Implications or Changes |  | Yes – Section No: (Include a summary of impact and/or specific changes) Attachment AF, Section 3 No |  |
| Criteria Impact or Changes |  | Yes – Section No: (Include a summary of impact and/or specific changes) No |  |
| Working Group |  | Voting Record |  |
| MWG |  | Date of Vote: 11/18/2014 Vote: Rejected Opposed: KCPL Abstained: TNSK, WR, Xcel, OPPD |  |
| RTWG |  | Date of Vote: Vote: |  |
| ORWG |  | Date of Vote: Vote: |  |
| MOPC |  | Date of Vote: Vote: |  |
| Board/Members Committee |  | Date of Vote: Vote: |  |

| Date | 9/5/2014 |
| --- | --- |

| Sponsor |  |
| --- | --- |
| Name | Jim Flucke |
| E-mail Address | jim.flucke@kcpl.com |
| Company | Kansas City Power & Light |
| Phone Number | 816-701-7836 |

| Comments Received |  |
| --- | --- |
| Comment Author | Catherine Tyler Mooney |
| Date | 11/13/2014 |
| Comment Description | These comments propose to change the estimate of LMPs used in the “sync to min” cost for the mitigated start up offer to an average historic LMP, as opposed to the resource energy cost at the economic minimum output level. Average LMPs present a more defendable estimate of LMP than the resource cost. |
| Comment Status | These comments were taken into consideration by the MWG. MWG did not make any language changes based on these comments at this time. |

Proposed Protocol Language Revision

#### 8.2.2.4  Mitigation Measures for Start-Up and No-Load Offers

- A Mitigated Start-up Offer and a Mitigated No-load Offer shall be submitted daily by the Market Participant in accordance with the Mitigated Offer Development Guidelines. The Mitigated Start-up and No-load Offers may be updated up to 1100 hours on the day before the Operating Day for use in the DA Market. In the case a Resource in not committed by the DA Market, the Mitigated Start-up and No-load Offers may be updated until the Day-Ahead RUC process begins. The Mitigated Start-up and No-load Offers submitted at the time the Day-Ahead RUC process begins will apply to the Day-Ahead RUC process on the day before the Operating Day and the Intra-Day RUC processes on the Operating Day.

- The Start-Up and No-Load Offer conduct thresholds are as follows:

  - For Resources with local market power as described in Section 8.2.2.6(3), the threshold is a 10% increase above the mitigated offer for the applicable offer;

  - For all other Resources the threshold is a 25% increase above the mitigated offer for the applicable offer.

- The Transmission Provider shall apply mitigation measures by replacing the Start-Up or No-Load Offer with the applicable Mitigated Start-up or mitigated No-load Offer if:

  - The Resource’s Start-Up or No-Load Offer exceeds the mitigated offer by the applicable threshold; and

  - The Resource has local market power as determined in Section 8.2.2.6; and

  - The Resource fails the Market Impact Test as described in Section 8.2.2.8, or the Resource has local market power as described in Section 8.2.2.6(3).

# The mitigated Start-Up Offer shall represent the cost per start as determined from start fuel usage to synchronization and the costs related to that fuel usage, fuel usage from synchronization to achievement of the Minimum Economic Capacity Operating Limit and the costs related to that fuel usage, an estimate of energy market revenue from synchronization to achievement of the Minimum Economic Capacity Operating Limit, electrical costs (station service), maintenance costs attributed to starts, and additional labor costs, if required above normal station manning levels.  The formula for mitigated Start-Up Offers can be found in Appendix G Section 2.6:

# Appendix G - Mitigated Offer Development Guidelines

## Mitigated Start- Up Offer

### Start- Up Offer Definitions

The Mitigated Start-Up Offer is the dollars per start as determined from start fuel, total fuel-related cost, performance factor, electrical costs, start VOM adder, and additional labor cost, if required above normal station manning levels.

Mitigated Start-Up Offer$Start=

Start Fuel mmBtuStart*TFRC$mmBtu*Performance Factor+[Sync to Min Fuel(mmBtu/Start)*TFRC($/mmBtu)*Performance Factor-Sync to Min Energy Revenue($/Start)]+Station ServiceMWh/Start*StationService Rate$MWh+Start VOM Adder$Start+Start Additional Labor Cost$Start

Station Service Rate is either a $/MWh value equal to the 12-month rolling average LMP at the station, updated at least quarterly, or the retail tariff rate applicable to the resource not included in the VOM calculated under Section 2.4.

Start Fuel  is the fuel consumed from first fire of start process (initial reactor criticality for nuclear resources) to breaker closing (including auxiliary boiler fuel) plus fuel expended from breaker opening of the previous shutdown to initialization of the resource start-up, excluding normal plant heating/auxiliary equipment fuel requirements.

Sync to Min Fuel is the cost of fuel used between breaker closing and the Resource reaching its Minimum Economic Capacity Operating Limit.

Sync to Min Energy Revenue is an estimate of the energy revenue earned after the breaker closes but before the Resource reaches it Minimum Economic Capacity Operating Limit.  The estimate assumes a linear increase in output over this time frame and an LMP equal to the resource cost of energy at its Minimum Economic Capacity Operating Limit.

Sync to Min Energy Revenue($/Start)=HeatRate at Econ. Min(mmBtu/MWh)*TFRC($/mmBtu)*Performance Factor +EOC VOM($/MWh)*Econ.Min(MW)/2*Sync to Min Time (hours)

Start VOM – The Start VOM is calculated using an allocated portion of the total VOM calculated under Section 2.4.3.  Note that the sum of total allocated $ used to calculate Start VOM included here, the allocated VOM $ used to calculate TFRC VOM under Section 2.3, the allocated VOM $ used to calculate No-Load VOM under 2.7, the allocated VOM $ used to calculate EOC VOM under 2.5 and the allocated VOM $ used to calculate Regulation VOM under 2.10 must not exceed the total VOM $ calculated under Section 2.4.

Start Additional Labor Cost – Additional labor costs for start-up required above normal station manning levels not included in the VOM calculated under Section 2.4.  A different value may be submitted for on-peak versus off-peak periods.

## Mitigated Start-Up Offer

Note: The information in Section 2.6 contains basic Mitigated Start-Up Offer information relevant for all unit types.  The following information only pertains to nuclear units.

Start Cost – The dollars per start as determined from start fuel, total fuel-related cost, performance factor, electrical costs, start VOM adder, and additional labor cost, if required above normal station manning levels.

Start Fuel – Fuel consumed from first fire of start process (initial reactor criticality for nuclear units) to breaker closing and fuel expended from breaker opening of the previous shutdown to initialization of the unit start-up, excluding normal plant heating/auxiliary equipment fuel requirements.

### Hot Start Cost

Hot start cost is the expected cost to start a steam unit, which is in the "hot" condition.  Hot conditions vary unit by unit, but in general, a unit is hot after an overnight shutdown.  Components of hot start cost include:

- Total fuel-related cost from first fire of start process (initial reactor criticality for nuclear units) to breaker closing priced at the cost of fuel currently in effect

- Total fuel-related cost from breaker closing to the minimum economic capacity operating limit, priced at the cost of fuel currently in effect, less the estimate of energy market revenues during the time from synchronization to minimum output

- And shutdown fuel cost defined as the cost of fuel expended from breaker opening of the previous shutdown to initialization of the unit start-up, excluding normal plant heating/auxiliary equipment fuel requirements.

### Intermediate Start Cost

Intermediate start cost is the expected cost to start a steam unit during a period where neither hot or cold conditions apply.  Use of intermediate start cost is optional based on Market Participant’s policy and physical machine characteristics.  The only restriction is that once an intermediate start cost is defined for a unit, the cost must be used consistently in scheduling and accounting.  Components of intermediate start cost include:

- Total fuel-related cost from first fire (initial reactor criticality for nuclear units) to breaker closing priced at the cost of fuel currently in effect

- Total fuel-related cost from breaker closing to the minimum economic capacity operating limit, priced at the cost of fuel currently in effect, less the estimate of energy market revenues during the time from synchronization to minimum output

- And shutdown fuel cost defined as the cost of fuel expended from breaker opening of the previous shutdown to initialization of the unit start-up, excluding normal plant heating/auxiliary equipment fuel requirements.

### Cold Start Cost

Cold start cost is the expected cost to start a steam unit that is in the “cold” condition.  Cold conditions vary unit by unit, but in general, a unit is cold after a two or three-day shutdown.  Components of cold start cost include:

- Total fuel-related cost from first fire (initial reactor criticality for nuclear units) to breaker closing priced at the cost of fuel currently in effect

- Total fuel-related cost from breaker closing to the minimum economic capacity operating limit, priced at the cost of fuel currently in effect, less the estimate of energy market revenues during the time from synchronization to minimum output

- And shutdown fuel cost defined as the cost of fuel expended from breaker opening of the previous shutdown to shutdown of equipment needed for normal cool down of plant components, excluding normal plant heating/auxiliary equipment fuel requirements.

## Hot Start Cost, Intermediate Start Cost, and Cold Start cost

Note: The information in Section 2.4 contains basic Start Cost information relevant for all unit types.  The following information only pertains to fossil steam units.

Mitigated Start-Up Offer$Start=

Start Fuel ConsumedmmBtuStart*TFRC$mmBtu*Performance Factor+[Sync to Min Fuel Consumed (mmBtu/Start)*TFRC($/mmBtu)*Performance Factor-Sync to Min Energy Revenue ($/start)]+Station ServiceMWh/Start*Station Service Rate$MWh+Start VOM Adder$Start+Start Additional Labor Cost$Start+Shut-Down Fuel ConsumedmmBtuStart*TFRC$mmBtu*Performance Factor

### Hot Start Cost

Hot start cost is the expected cost to start a steam unit, which is in the “hot” condition.  Hot conditions vary unit by unit, but in general, a unit is hot after an overnight shutdown.

Components of hot start cost include:

Start Fuel Consumed is the amount of fuel consumed from first fire of start process to breaker closing (including auxiliary boiler fuel).

Sync to Min Fuel Consumed is the amount of fuel consumed from breaker closing to the achievement of the Minimum Economic Capacity Operating Output.

Sync to Min Energy Revenue is an estimate of the energy market revenues received from breaker closing to the achievement of the Minimum Economic Capacity Operating Output.  This offsets the fuel consumed in the start-up cost calculation.

Shutdown Fuel Consumed is the amount of fuel consumed from breaker opening of the previous shutdown to initialization of the unit start-up, excluding normal plant heating/auxiliary equipment fuel requirements.

Station Service from initiation of start sequence to breaker closing (total station use minus normal base station use) priced at the Station Service rate and station service after breaker opening during shutdown (station service during shutdown should be that associated with the normal unit auxiliary equipment operated during shutdown in excess of base unit use, this station service is not to include VOM or non-normal use) priced at the Station Service rate.

Additional labor costs in excess of normal station manning requirements that are incurred when starting the unit.

Start VOM Adder - Section 3.6 contains information regarding calculation of VOM Adder.

### Intermediate Start Cost

Intermediate start cost is the expected cost to start a steam unit during a period where neither hot nor cold conditions apply.  Use of intermediate start cost is optional based on company policy and physical machine characteristics.  The only restriction is that once an intermediate start cost is defined for a unit, the cost must be used consistently in scheduling and accounting.

Components of intermediate start cost include:

Start Fuel Consumed is the amount of fuel consumed from first fire of start process to breaker closing (including auxiliary boiler fuel).

Sync to Min Fuel Consumed is the amount of fuel consumed from breaker closing to the achievement of the Minimum Economic Capacity Operating Output.

Sync to Min Energy Revenue is an estimate of the energy market revenues received from breaker closing to the achievement of the Minimum Economic Capacity Operating Output.  This offsets the fuel consumed in the start-up cost calculation.

Shutdown Fuel Consumed is the amount of fuel consumed from breaker opening of the previous shutdown to initialization of the unit start-up, excluding normal plant heating/auxiliary equipment fuel requirements.

Station Service from initiation of start sequence to breaker closing (total station use minus normal base station use) priced at the Station Service rate and station service after breaker opening during shutdown (station service during shutdown should be that associated with the normal unit auxiliary equipment operated during shutdown in excess of base unit use, this station service is not to include VOM or non-normal use) priced at the Station Service rate.

Additional labor costs in excess of normal station manning requirements that are incurred when starting the unit.

Start VOM Adder - Section 2.6 contains information for calculation of the Start VOM Adder.

### Cold Start Cost

Cold start cost is the expected cost to start a steam unit that is in the “cold” condition.  Cold conditions vary unit by unit, but in general, a unit is cold after a two or three-day shutdown.  Components of cold start cost include:

Start Fuel Consumed is the amount of fuel consumed from first fire of start process to breaker closing (including auxiliary boiler fuel).

Sync to Min Fuel Consumed is the amount of fuel consumed from breaker closing to the achievement of the Minimum Economic Capacity Operating Output.

Sync to Min Energy Revenue is an estimate of the energy market revenues received from breaker closing to the achievement of the Minimum Economic Capacity Operating Output.  This offsets the fuel consumed in the start-up cost calculation.

Shutdown Fuel Consumed is the amount of fuel consumed from breaker opening of the previous shutdown to initialization of the unit start-up, excluding normal plant heating/auxiliary equipment fuel requirements.

Station Service from initiation of start sequence to breaker closing (total station use minus normal base station use) priced at the Station Service rate and station service after breaker opening during shutdown (station service during shutdown should be that associated with the normal unit auxiliary equipment operated during shutdown in excess of base unit use, this station service is not to include VOM or non-normal uses) priced at the Station Service rate.

Additional labor costs in excess of normal station manning requirements that are incurred when starting the unit.

Start VOM Adder - Section 2.6 contains information for calculation of the Start VOM Adder.

## Mitigated Start-Up Offer

Note: The information in Section 2.4 contains basic Mitigated Start-Up Offer information relevant for all unit types.  The following additional information only pertains to combined cycle units.

Start costs for Combined Cycle (CC) plants shall include only the following components and shall never be less than zero:

Start Cost$Start=

Start Fuel ConsumedmmBtuStart*TFRC$mmBtu*Performance Factor+(Sync to Min Fuel Consumed(mmBtu/Start)*TFRC($/mmBtu)*Performance Factor-Sync to Min Energy Revenue($/Start))+Station ServiceMWh*Station Service Rate$MWh+Start VOM Adder$Start+Incremental Labor Cost$Start

Start cost can be calculated and offered for Hot, Intermediate and Cold Start conditions.

Start Fuel Cost is the cost of fuel consumed from first CT fire to the achievement of the Minimum Economic Capacity Output limit, as measured during a normal start sequence, and the amount of fuel consumed from breaker opening for the steam turbine resource to fuel valve closure.  This includes the amount of fuel consumed from CT first fire to the point where heat recovery steam resource (HRSG) steam pressure matches steam turbine inlet pressure, for any CT unit/HRSG combinations started after synchronization of the steam turbine resource.  This amount is offset by an estimate of the energy revenue from breaker closing to the achievement of the Minimum Economic Capacity Output limit.

Station service is included from initiation of start sequence of initial combustion turbine to breaker closing of the steam turbine resource (total station use minus normal base station use) priced at the Station Service Rate.

Add to this (+) station service after breaker opening of the last component when finished operating as a combined cycle unit, priced at the Station Service rate.  (Station service during shutdown should be that associated with the normal unit auxiliary equipment operated during shutdown in excess of base unit use.  This station service is not to include VOM or non-normal uses.  )

Minus (-) the integration of net generation from CT synchronization to steam turbine resource synchronization or to HRSG steam output at line pressure, priced at the actual cost of the unit.

Minus (-) the integration of net generation during the shutdown period, priced at the actual cost of the unit.

Incremental labor costs in excess of normal station manning requirements (only when necessary to start the CC unit).

Start VOM Adder - this quantity includes  Start VOM $ for CT Starting from CT breaker closing to steam turbine resource breaker closing and from steam turbine resource breaker opening at the start of unit shutdown to CT breaker opening.

Proposed Tariff Language Revision

3.3	Mitigation Measures for Start-Up Offers and No-Load Offers

A mitigated Start-Up Offer and a mitigated No-Load Offer shall be submitted daily by the Market Participant in accordance with the mitigated offer development guidelines in the Market Protocols.  The mitigated Start-Up and No-Load Offers may be updated up to 1100 hours on the day before the Operating Day for use in the Day-Ahead Market.  In the case a Resource is not committed by the Day-Ahead Market, the Start-Up and No-Load Offers may be updated until the Day-Ahead RUC begins.  The mitigated Start-Up and No-Load Offers submitted at the time the Day-Ahead RUC begins will apply to the Day-Ahead RUC on the day before the Operating Day and the Intra-Day RUC on the Operating Day.

A.	The Start-Up and No-Load Offer conduct thresholds are as follows:

(1)	For Resources with local market power as described in Section 3.1(4), the conduct threshold is a 10% increase above the mitigated Start-Up or mitigated No-Load Offer, as applicable;

(2)	For all other Resources the conduct threshold is a 25% increase above the mitigated Start-Up or mitigated No-Load Offer, as applicable.

B.	The Transmission Provider shall apply mitigation measures by replacing the Start-Up or No-Load Offer with the applicable mitigated Start-Up or No-Load Offer if:

(1)	The Resource’s Start-Up or No-Load Offer exceeds the mitigated Start-Up or mitigated No-Load Offer, as applicable, by the applicable conduct threshold; and

(2)	The Resource has local market power as determined in Section 3.1; and

(3)	The Resource either:

(a)	Fails the Market Impact Test as described in Section 3.7, or

(b)	Has local market power as described in Section 3.1(4).

C.	The mitigated Start-Up Offer shall represent the cost per start as determined from start fuel usage to synchronization and the costs related to that fuel usage, fuel usage from synchronization to achievement of the Minimum Economic Capacity Operating Limit (“Sync to Min”) and the costs related to that fuel usage, an estimate of energy market revenue from synchronization to achievement of the Minimum Economic Capacity Operating Limit, Performance Factor, cost of electricity for station use to start (“Station Service”), maintenance costs attributed to starts, and additional labor costs, if required above normal station staffing levels.  The following formula shall apply to all mitigated Start-Up Offers:

Mitigated Start-Up Offer ($/Start) = [Start Fuel (mmBtu/Start) *

Total Fuel Related Costs ($/mmBtu) * Performance Factor] + [Sync to Min Fuel (mmBtu/Start) * Total Fuel Related Costs ($/mmBtu) * Performance Factor – Sync to Min Energy Revenue ($/Start)] + [Station Service (MWh/Start) *

Station Service Rate ($/MWh)] + Start VOM ($/Start) + Start Additional Labor Cost ($/Start)

The mitigated Start-Up Offer for Demand Response resources shall be the cost to shut down or curtail load for a given period, which varies with the number of deployments rather than the amount of response, and/or the start cost of Behind-The-Meter Generation utilizing the mitigated Start-Up Offer calculation applicable to other generation Resources as defined above.

The mitigated Start-Up Offer for Variable Energy Resources shall be zero.

Proposed Criteria Language Revision

N/A